# PART IV: Defense Platform Analysis, Surge Demand, and Policy

## Chapter 15

## Israeli Defense Platform Permanent Magnet Requirements

Open sources provide strong evidence that Israeli missile-defense and precision-weapon ecosystems rely on subsystems where permanent magnets are commonly required (actuators, control surfaces, guidance-related motors, and radar/RF hardware), but they rarely disclose magnet grades, quantities, or suppliers at platform level. Public reporting also shows that the highest-volume driver is the Iron Dome/Tamir interceptor enterprise, which has been fielded since 2011 and is supported by accelerating serial production and a growing U.S.–Israeli co-production base.

At the strategic-materials level, multiple sources describe significant exposure to China’s dominance in rare-earth mining, refining/processing, and magnet manufacturing, creating a supply-chain vulnerability that affects Israel as well as the U.S. and other allies. This exposure is particularly important for high-temperature and high-coercivity permanent magnets (SmCo and Dy/Tb-enhanced NdFeB), which are widely referenced as necessary in defense actuators, radar, and missile guidance contexts, even when platform-specific details remain classified or proprietary.

### Iron Dome

Iron Dome is described as a Rafael-developed air defense system designed to intercept short-range threats (including rockets and artillery), with the Tamir interceptor as the missile effector and a radar and command-and-control architecture supporting engagement decisions and midcourse updates. Several sources specify that a typical Iron Dome battery comprises an EL/M-2084 multi-mission radar (Elta/IAI), a battle management/weapon control element (mPrest is frequently cited for the control function), and three to four launchers, with each launcher carrying up to 20 Tamir interceptors (i.e., ~60–80 ready missiles per battery).

Tamir’s publicly described functional components include a solid rocket motor, datalink for midcourse updates, an onboard terminal seeker described as active radar in multiple open sources, and a proximity-fuzed warhead; several sources also describe electro-optical sensors and steering fins for maneuvering. These descriptions do not enumerate actuators, servo architectures, or magnet materials; however, multiple defense supply-chain and technical references explicitly tie permanent magnets to guided-missile actuators and flight-control surface manipulation (i.e., the class of hardware that would be required to drive steering fins).

Operational tempo and replenishment dynamics are visible in open reporting, including statements that Iron Dome has intercepted thousands of rockets since October 2023 and that the system has intercepted over 9,000 rockets since activation with an approximately 95% success rate (Rafael corporate reporting). This high usage rate underpins the strong linkage between Iron Dome interceptor procurement and magnet demand in the guided-munition supply chain (even if magnet quantities per interceptor are not publicly disclosed).

### Tamir specifications that matter for magnets

Tamir’s published physical envelope is consistently reported at approximately 3 m length, ~160 mm diameter, and ~90 kg weight, with an engagement range up to ~70 km, and performance references such as Mach 2.2 appearing in some open sources. These constraints suggest that any embedded electromechanical actuation for steering fins (or seeker gimbal mechanisms, if present) must be compact and power-dense, which is the regime where rare-earth permanent magnets are generally used in military actuation and guidance contexts.

Some sources go further and claim or imply “permanent magnet” use in missile guidance for Tamir, but without providing technical substantiation such as magnet composition, grade, coercivity, or operating temperature.

### David’s Sling

David’s Sling is widely described as the middle layer of Israel’s multi-layered air defense architecture, designed to intercept rockets, missiles (including ballistic and cruise missiles), aircraft, and drones, and jointly developed by Rafael and Raytheon. Open sources describe the Stunner interceptor as a two-stage solid-propellant missile with an operational range cited around 70–300 km and maximum speed around Mach 7.5, which implies high-g maneuver demands and tight mass/volume constraints for guidance and control hardware.

Guidance is described as integrating the EL/M-2084 AESA radar with a dual electro-optical seeker suite (imaging infrared plus CCD), plus datalink and counter-countermeasure capabilities in some summaries. One open source explicitly states that David’s Sling actuator motors “utilize permanent magnet components,” but it does not identify whether these are SmCo versus high-temperature NdFeB grades, nor provide quantities or temperature requirements.

Cost signals also indicate high per-interceptor value density: open sources cite Stunner unit costs around $700,000 and note U.S. financial support for the program (e.g., $1.39B in one recent report; $1.99B FY2006–2020 in another). These economics typically motivate conservative material selections, qualification rigor, and second-source strategies for components like rare-earth magnets, even when procurement specifics are not public.

### Arrow 2 and Arrow 3

Open reporting describes Arrow 2 as an upper-atmosphere interceptor (around ~50 km altitude) and Arrow 3 as an exo-atmospheric interceptor designed to engage long-range ballistic missiles at the top of their trajectory, with Arrow 3 credited to Israel Aerospace Industries (state-owned) with U.S. support (e.g., Boeing). These operational regimes strongly imply extreme thermal cycling (cold-soak at altitude/space-like conditions and potential heating during atmospheric flight), which is the regime where defense systems often prefer SmCo or specially engineered high-temperature NdFeB, but open sources reviewed here do not disclose Arrow magnet types, quantities, or suppliers.

### Trophy and other active protection systems

Trophy (Windbreaker) is described by Rafael as a combat-proven active protection system intended to enhance survivability of armored vehicles (tanks and APCs). System-level descriptions emphasize 360-degree detection and autonomous neutralization, but do not describe internal motor/actuator architectures or magnet materials for turreted sensors, countermeasure launcher drives, or stabilization motors.

Open sources on another Israeli APS (Elbit’s Iron Fist) emphasize compactness (low volume, weight, and power) and multi-sensor cueing to turreted countermeasure launchers, which are application spaces where permanent-magnet motors are common, but the cited materials remain non-specific on magnet selection and sourcing.

### Spike and other Rafael precision weapons

Rafael describes the Spike family (multiple variants) as electro-optically guided missiles and provides an aggregate volume indicator: as of 2023, more than 38,000 missiles have been delivered or are under order, with over 7,000 deployed in combat and training. While the guidance suite is described at a high level (electro-optical guidance), the reviewed sources do not disclose magnet types, actuator designs, or magnet counts per missile for Spike variants.

In adjacent Israeli precision munitions, Elbit’s “Iron Sting” guidance descriptions (GPS/laser guidance, deployable fins, multi-mode fuze concepts) indicate the presence of guidance fins and seekers, but open sources do not provide magnet-level specifications for the fin deployment or any actuation mechanisms.

### Radar and electronic warfare hardware

Multiple defense supply-chain sources explicitly cite traveling-wave tubes (TWTs) as components that use rare earth permanent magnets to amplify radio-frequency signals, and they also cite radar systems as major magnet-using assets (including samarium-cobalt magnet components in long-lived radar systems such as Aegis SPY-1). Separate technical descriptions of microwave tubes reinforce that magnetrons can incorporate “an extremely powerful permanent magnet” and that electron beams in some tube types are guided by magnets, which is relevant to high-power radar/EW transmitter architectures even when specific Israeli radar bill-of-materials details are not public.

For Israeli radar specifically, open sources identify the EL/M-2084 as an AESA radar used in Iron Dome and David’s Sling, and provide performance descriptors (e.g., tracking ~1,200 rockets at once; surveillance tracking to 474 km in one description), but none of the reviewed sources breaks down whether the radar uses solid-state T/R modules only, or whether any vacuum electronics (and thus permanent-magnet focusing assemblies) are used in associated transmit/receive or ancillary subsystems.

Elbit publicly describes an EW suite deployed on “hundreds” of aircraft and enumerates functional blocks (EW controller, radar warning receiver, IR missile warning, countermeasures, jammer), but it does not discuss permanent magnets, nor does it disclose rare-earth magnet suppliers or grades used in any actuators or cooling/power subsystems within those products.

### Magnet materials and performance requirements

The open-source record in this dataset supports a materials-level conclusion: SmCo magnets are preferred where high-temperature demagnetization resistance is required, while NdFeB magnets are used where higher energy density is needed but temperature limits must be managed, sometimes through heavy rare earth (Dy/Tb) additions for coercivity at elevated temperature. One DOE reference states NdFeB magnets are used up to about 180°C and that operation at higher temperatures can require replacing some Nd/Pr with heavy rare earths (Dy or Tb). Separately, a U.S. government-linked supply chain assessment notes sintered NdFeB magnet alloys may include ~0.5% to 11% Dy or Tb by weight to improve high-temperature resistance to demagnetization (a range that covers common “high-temp” grades and heavy-REE-enhanced defense grades).

For SmCo, multiple sources emphasize high-temperature robustness and suitability above 300°C for some weapon-system contexts, while also noting higher cost and processing complexity compared to NdFeB in general market descriptions. Industrial supplier datasheets reinforce the high-temperature envelope: VACOMAX SmCo is described as suitable up to 350°C depending on alloy, with Br and (BH)max ranges given, and Arnold describes RECOMA 35E as engineered for high temperature applications around 150–250°C in high-speed/high-corrosion environments.

Because missile-defense interceptors and seekers can encounter elevated internal temperatures from power electronics and proximity to propulsion sections (as well as external aerodynamic heating), these materials considerations strongly influence the “design space” for guidance and actuation magnets; however, platform-to-material mapping (e.g., “Tamir seeker uses SmCo”) is not supported by the reviewed open sources and should be treated as an open question pending program-specific disclosures or supplier documentation.

### Supply chain and co-production implications

Multiple sources document that Iron Dome/Tamir has shifted from Israel-only production to a joint U.S.–Israeli industrial base, including references to a U.S. Tamir production facility and a Raytheon–Rafael joint venture producing Tamir rounds and the U.S. SkyHunter variant. Several sources identify a $1.25B production contract for Tamir interceptors at a Camden/East Camden, Arkansas facility and describe investments reported at $33M (and in some reporting increased to $63M) to establish and expand production capacity.

From a magnet supply standpoint, this co-production structure matters because one RTX/Raytheon source states that “the majority of Tamir missile components are procured through the Raytheon supply chain in the United States,” while also describing SkyHunter as producible in the U.S. for the U.S. and its allies. At the same time, none of the cited sources identifies which magnet suppliers (or which countries of origin for magnet raw materials) are used for Tamir or SkyHunter, which is a critical data gap given upcoming provenance and disclosure requirements for rare earth permanent magnets in U.S. defense procurement.

For David’s Sling, open sources state that roughly 50% of components are built in the United States and that some parts of the interceptor and launcher are manufactured in the U.S., reinforcing that magnet sourcing decisions may be influenced by both Israeli and U.S. compliance/traceability frameworks.

### Rare earth dependency and risk to Israel

Several sources explicitly describe Israel’s vulnerability stemming from dependency on Chinese rare earth supplies for defense technologies including magnets, with Chinese export restrictions (including restrictions applying to medium/heavy rare earths such as samarium, terbium, and dysprosium) cited as a significant disruption factor. More broadly, U.S. government sources and analyses document that most rare earth materials are mined and processed abroad, with China identified as a dominant actor across mining, processing, and magnet manufacturing, which implies that Israeli defense platforms embedded in global supply chains inherit this dependency unless mitigated by allied sourcing and domestic capacity initiatives.

On the mitigation side, U.S. initiatives to establish a “mine-to-magnet” industrial base explicitly include permanent magnets used in weapon systems “including the Iron Dome missile interceptor,” and note substantial investment scale since 2020. The same U.S. effort cites domestic manufacturing nodes such as a rare earth magnet facility in San Marcos, Texas (Noveon Magnetics) funded via a $28.8M award, which may become relevant to U.S.-based co-production supply chains for interceptors and for allied procurement ecosystems over time.

### Suppliers and procurement signals in open sources

Open sources in this dataset provide partial but meaningful signals about potential magnet supplier categories relevant to Israeli systems. Permag (a group comprising Dexter Magnetic Technologies, Electron Energy Corporation, and Magnetic Component Engineering) positions itself as a DFARS-capable supplier for defense magnetics, states that “through EEC” it is “the only U.S. producer of samarium cobalt (SmCo) magnets,” and lists certifications including ITAR registration and AS9100D, which are prerequisites for many U.S. defense magnet supply chains. These statements do not establish actual supply into Iron Dome/David’s Sling, but they highlight a limited U.S. SmCo industrial base and the scarcity of qualified sources for high-temperature magnets.

Within Europe, VAC positions VACOMAX (SmCo) and VACODYM (NdFeB) as vertically integrated product families, including in-house processing “from the melting of the alloy” through coating, magnetization, assembly, and final inspection, which aligns with the traceability and reliability requirements typical of defense procurement, even though Israeli program participation is not stated in the sources provided.

Within Israel, one open catalog confirms that Magma Magnetic Technologies is based in Israel and offers SmCo and NdFeB magnets with stated heat resistance ranges (SmCo 250–350°C; NdFeB 80–240°C), which suggests a local channel for some magnet forms, although defense certifications and actual program tie-ins are not provided in the cited material.

### Production scale indicators and implied magnet demand

While most sources avoid publishing annual interceptor production rates, procurement and inventory numbers provide a basis to understand potential scale. A U.S. Marine Corps plan to acquire three Iron Dome batteries and 1,840 Tamir/SkyHunter missiles is explicitly cited in multiple open sources, and a U.S. Army acquisition for test and evaluation is cited as 480 missiles (with associated battery hardware). Independent reporting also emphasizes that the Arkansas facility and related contracts are designed to “accelerate serial output” and bolster production capacity for both Iron Dome and SkyHunter variants, implying increased throughput and sustained demand for components such as permanent magnets used in actuators and guidance-relevant motors.

At the same time, the publicly available sources do not provide magnets-per-interceptor counts or kilograms of magnet material per interceptor, which prevents a fully grounded bottom-up magnet-demand calculation in this report.

### Implications for Algamol

The most direct implication for a wastewater treatment and recovery technology provider is that Israel’s defense industrial activity spans high-volume interceptors (Tamir), high-value interceptors (Stunner), and very high-value sensor and EW ecosystems, all of which are associated with permanent-magnet-containing subsystems (actuators, control surfaces, and RF vacuum electronics in some architectures). The open-source record also shows a tightening regulatory environment on magnet provenance and disclosure in U.S. defense procurement (implementation time windows cited between 2025 and 2027; expanded prohibitions effective January 1, 2027), which could increase the value of domestic and traceable recycling/recovery pathways for magnet-bearing waste streams in U.S. co-production facilities supporting Israeli programs.

Because U.S. co-production nodes exist for Tamir/SkyHunter (Camden/East Camden, Arkansas) and because a U.S. rare earth magnet facility exists in San Marcos, Texas, a plausible commercialization pathway for wastewater and waste-stream treatment solutions is to target U.S.-based machining, finishing, and subassembly operations tied to these supply chains (where traceability constraints and disposal economics can be aligned).

### Unknowns

This review finds that several critical, decision-relevant data elements are either absent from open sources or explicitly deferred to direct engagement with OEMs. Specifically, sources generally do not disclose which magnet families (SmCo versus Dy/Tb-enhanced NdFeB) are used in each Israeli platform’s actuators, seekers, and radar subsystems, nor do they provide magnet counts per interceptor or magnet mass per unit. In addition, detailed procurement channels and supplier lists for magnets into Rafael, IAI/Elta, and Elbit are not provided in the cited materials, and at least one Elbit product page explicitly indicates that inquiries about rare earth magnets or sensors require direct follow-up through Elbit or other technical resources.

Finally, none of the reviewed platform sources quantify manufacturing or machining scrap rates, coolant volumes, or wastewater compositions associated with magnet production or finishing in Israel or in U.S. co-production facilities, which is a prerequisite for sizing and valuing an Algamol-type treatment deployment. As a result, primary research (supplier interviews, site visits, or analysis of publicly permitted industrial discharge data where accessible) is required to translate the clear strategic importance of magnets in these systems into a site-specific waste and wastewater opportunity model.

## Chapter 16

## US Iron Dome Co-Production and DFARS Magnet Sourcing

### Executive Summary

The establishment of a U.S. production line for Israel’s Iron Dome missile defense system and its American variant, SkyHunter, creates a significant new demand signal for domestically sourced, high-performance permanent magnets compliant with U.S. defense regulations. Through the Raytheon-Rafael Area Protection Systems (R2S) joint venture, a new "all-up-round" interceptor production facility in East Camden, Arkansas, is now operational, supported by over a billion dollars in contracts and U.S. aid designated for replenishing Israeli stockpiles and equipping U.S. forces. This domestic manufacturing effort directly implicates the Defense Federal Acquisition Regulation Supplement (DFARS), which mandates that critical materials, including rare earth magnets, be sourced from non-adversarial nations.

With an expanded DFARS prohibition taking full effect on January 1, 2027—covering the entire supply chain from mining to finished magnet—prime contractors like Raytheon (RTX) must secure compliant sources for the magnets essential to interceptor guidance and control systems. These systems rely on high-temperature samarium-cobalt (SmCo) magnets for seeker head gimbals and high-coercivity neodymium-iron-boron (NdFeB) magnets for fin actuators. While DFARS provides an exemption for NdFeB magnets made from recycled material milled and sintered in the U.S., no such exemption exists for SmCo magnets, creating a significant sourcing challenge. This report analyzes the emerging U.S. Iron Dome production ecosystem, maps the DFARS compliance requirements onto the magnet supply chain, and identifies the key domestic magnet suppliers positioned to support this critical defense industrial base initiative.

### Iron Dome Co-Production Program Overview

The U.S.-Israeli collaboration on the Iron Dome system has evolved from financial support to direct industrial base integration. Over the last decade, the U.S. has provided billions in funding, including $1.6 billion between 2011 and 2021 and an additional $1 billion approved by Congress in 2022. This partnership deepened with the formation of the Raytheon-Rafael Area Protection Systems (R2S) joint venture in 2020, created explicitly to establish an Iron Dome production facility in the United States.

This initiative is driven by several factors:

- Replenishment Needs: Increased operational use of the system, particularly since October 2023, created an urgent need to replenish Israel's stockpile of Tamir interceptors, prompting a "multi-billion dollar contract" to accelerate production. A U.S. aid package approved in April 2024 allocated $5.2 billion (out of an $8.7 billion total) specifically for Israeli air and missile defense systems.

- U.S. Military Adoption: The U.S. military is procuring its own Iron Dome derivatives. The U.S. Army acquired two batteries for evaluation, and the U.S. Marine Corps is integrating the system as its Medium-Range Intercept Capability (MRIC) program, using the U.S. variant of the Tamir interceptor, known as SkyHunter. The Marine Corps' FY25 budget requested $111 million for 12 launchers and 242 missiles, and a separate planned order may include 1,840 interceptors.

- Industrial Base Security: The establishment of a U.S. production line is a strategic move to create a "robust American production base" and a shared U.S.-Israeli supply chain, enhancing resilience and ensuring interceptors can be delivered "efficiently and at scale".

The culmination of this effort is the new R2S facility in Arkansas, which received a $1.25 billion contract in late 2025 to begin supplying Tamir interceptors to Israel.

### US Production Facilities and Capabilities

The domestic production of the Tamir and SkyHunter interceptors is centered around a growing network of Raytheon/RTX facilities, with the new R2S plant in Arkansas as the centerpiece.

- R2S Facility (East Camden, Arkansas): Opened in November 2025, this is the first "all-up-round" production facility for Iron Dome interceptors in the United States. The joint venture's investment in the site grew from an initial $33 million to $63 million. The plant, which broke ground in February 2024, is slated to begin missile production in 2025 and will create up to 60 jobs. It is located at the Highland Industrial Park, building on RTX's existing presence in the area.

- Raytheon Facility (Tucson, Arizona): Tucson is a major hub for Raytheon's missile defense activities. Sources from 2014 and later confirm that Raytheon was producing components for the Tamir interceptor at its Tucson facility, which entered full production by July 2015. In 2020, the joint venture also agreed to a separate plant investment in Tucson.

- Raytheon Facility (Andover, Massachusetts): This facility is mentioned as being involved in the production of the seeker and guidance sections for missile systems, making it a likely contributor of critical components for the U.S.-built interceptors.

- Huntsville, Alabama: This location was noted as one of the sites that would benefit from the co-production arrangement, suggesting a role in engineering, prototyping, or component manufacturing.

This distributed manufacturing footprint indicates that while final assembly and integration of all-up-rounds will occur in Camden, critical subsystems and components are being produced across multiple specialized Raytheon sites.

### US vs Israeli-Built Components

The co-production model involves a deep integration of U.S. and Israeli supply chains. While the new Camden facility is capable of all-up-round production, the existing supply chain already features significant U.S. content.

Raytheon states that the "majority of Tamir missile components are procured through the Raytheon supply chain in the United States". Another source notes that the Israeli production line historically receives approximately half of its components from Raytheon. The U.S. version of the interceptor, SkyHunter, shares 95-99% commonality with the Israeli Tamir, meaning the designs are nearly identical.

The establishment of a U.S. production line for systems like the seeker, guidance, and control sections is the critical factor for magnet sourcing. The production of these magnet-intensive subsystems in U.S. facilities (like Tucson for missile integration and Andover for seekers) means the components used within them are subject to U.S. procurement law, specifically DFARS.

### Contract Values and Production Quantities

The scale of U.S. and Israeli demand is driving significant investment and large-scale production contracts.

- R2S Production Contract: In November 2025, the R2S joint venture was awarded a $1.25 billion contract to produce Tamir interceptors at the new Camden facility to supply Israel. This is the first production contract for the R2S partnership.

- U.S. Aid Package: The production surge is substantially funded by an $8.7 billion U.S. aid package approved in April 2024, which allocated $5.2 billion specifically for Israeli air and missile defense systems. One report suggests this funding could cover the production of "about 100 Iron Dome batteries and at least 14,000 Tamir interceptor missiles".

- U.S. Military Procurements: The U.S. Army previously acquired two Iron Dome batteries with 480 missiles in a 2019 deal valued at $373 million. The U.S. Marine Corps has a planned order for three batteries, 44 launchers, and 1,840 interceptors for its MRIC program. The FY25 budget request for MRIC includes $111 million for 242 missiles.

### Permanent Magnet Requirements for US-Built Interceptors

The Tamir and SkyHunter interceptors rely on high-performance permanent magnets for their agility and precision. These magnets are critical components within the guidance and control subsystems.

- Seeker Head: The interceptor uses an active radar and/or electro-optical seeker for terminal guidance. This seeker is mounted on a gimbal system that requires small, powerful motors to rapidly articulate the sensor. These gimbal motors use permanent magnets. Due to the high temperatures generated by aerodynamic heating on the missile's nose cone, these are likely high-temperature SmCo magnets.

- Fin Actuators: The missile is equipped with several steering fins for high-g maneuvering. Each fin is controlled by an actuator, typically a brushless DC motor that relies on a permanent magnet rotor. These actuators must perform under extreme thermal and vibratory stress, requiring high-temperature NdFeB or SmCo magnets.

- Control Section Electronics: Other sensors and electronic components within the guidance and control section may also contain permanent magnets.

Given the high-volume production plans, with thousands of interceptors required annually, the demand for these specialized, defense-grade magnets will be substantial.

### DFARS Compliance for US-Built Interceptor Magnets

Any interceptor or component produced in the U.S. under a Department of Defense contract must comply with DFARS. The final DFARS rule published on May 30, 2024, implements and expands upon these restrictions.

- The Regulation: DFARS clause 252.225-7052 explicitly lists SmCo and NdFeB magnets as "covered materials".

- The Deadline: Effective January 1, 2027, the rule prohibits the acquisition of any covered material that was "mined, refined, separated, melted, or produced" in a covered country (China, Russia, North Korea, Iran). This covers the entire upstream supply chain.

- Application: Because the US-built Tamir/SkyHunter interceptors are part of a U.S. defense procurement (for the USMC and funded by U.S. aid for Israel), the magnets within their U.S.-made components must be DFARS-compliant. Prime contractors like Raytheon are required to flow this clause down to their subcontractors.

There is a critical distinction in compliance pathways for the two key magnet types:

- NdFeB Magnets: The rule provides an exception for NdFeB magnets manufactured from recycled material, but only if the milling of the recycled material and the sintering of the final magnet occur in the United States.

- SmCo Magnets: There is no recycling exemption for SmCo magnets. As of January 1, 2027, the entire supply chain—from the mining of samarium and cobalt ore—must be free of covered countries. This presents a much higher compliance hurdle.

### Magnet Supply Chain for US Iron Dome

Given the stringent DFARS requirements, Raytheon and its subcontractors must source magnets for U.S. Iron Dome production from a very narrow list of qualified domestic suppliers. The likely Tier-1 suppliers for these magnet assemblies include:

- Electron Energy Corporation (EEC) / Permag: EEC, part of the Permag group, is the only vertically integrated U.S. producer of SmCo magnets, controlling the process from alloy melt to finished magnet. This makes them a critical supplier for the SmCo magnets likely used in the interceptor's seeker head.

- Arnold Magnetic Technologies: Arnold is a key U.S. defense supplier with a long history of producing high-performance SmCo (RECOMA series) and other magnets. They have publicly stated their commitment to establishing a fully DFARS-compliant supply chain for both NdFeB and SmCo magnets ahead of the 2027 deadline.

- Vacuumschmelze (VAC): While traditionally a German company, VAC has received a $94.1 million DoD award to establish a rare earth magnet manufacturing facility in Sumter, South Carolina, specifically to support defense needs and DFARS compliance. This plant will become a new domestic source once fully operational.

The extreme stress on the supply chain from surge demand for Iron Dome, combined with demand from other platforms like the F-35 (900 lbs of REEs) and DDG-51 destroyers (5,200 lbs of REEs), creates a significant risk of bottlenecks. Qualifying a second source for a critical missile component can take 12-24 months, making the existing capacity of these few suppliers a key strategic concern.

### Production Surge and Supply Chain Stress

The surge in demand for Iron Dome interceptors following October 2023 places immense pressure on an already constrained global supply chain for high-performance magnets. Israel's need to replenish its stocks, combined with new U.S. production, will test the limits of domestic magnet producers.

The SmCo supply chain is particularly vulnerable. Samarium is almost exclusively sourced from China, and cobalt is also a critical material with a concentrated supply chain. The DFARS rule's lack of a recycling exemption for SmCo means that, post-2027, SmCo magnets for U.S. defense must originate from fully compliant ore-to-magnet chains, which are still nascent outside of China. This regulatory pressure, combined with high demand, creates a significant risk for programs like the US-built Iron Dome that rely on these materials for high-temperature applications.

### Implications for Algamol

The ramp-up of U.S. Iron Dome co-production creates a concentrated and valuable opportunity for wastewater treatment and resource recovery technologies. The precision machining of SmCo and NdFeB magnets into final parts for seeker gimbals and fin actuators is a waste-intensive process. This waste stream, generated at Raytheon subcontractor facilities, is a key target.

- Concentrated Waste Streams: Instead of being diffuse, magnet machining waste for the Iron Dome program will be concentrated at a few Tier-2 supplier locations that specialize in grinding and finishing rare earth magnets.

- High-Value Waste: This machining sludge contains high concentrations of valuable and strategic materials, including samarium, cobalt, neodymium, and dysprosium.

- DFARS Compliance Driver: For NdFeB magnets, on-site recovery of machining waste provides a direct pathway to creating a DFARS-compliant feedstock. Under DFARS 252.225-7052, NdFeB magnets manufactured from recycled material are exempt, provided the milling and sintering occur in the U.S. An on-site recovery system would enable a Tier-2 supplier to create a secure, domestic source of recycled powder.

- Strategic Value for SmCo: While SmCo lacks a recycling exemption, the strategic importance of cobalt and samarium, coupled with the supply chain risks, makes on-site recovery and purification highly valuable for securing a resilient internal supply.

- Ideal Customer Profile: A large defense prime contractor like Raytheon has established Environmental, Health, and Safety (EHS) infrastructure and would be receptive to a pilot program that offers both cost savings and a tangible solution to a pressing supply chain and regulatory challenge. The priority target is the Tier-2 machining house supplying these finished magnet components to Raytheon's integration facilities in Tucson or Andover.

### Key Unknowns and Intelligence Gaps

While the strategic direction is clear, several critical pieces of information remain proprietary or classified:

- The specific Tier-1 and Tier-2 magnet suppliers and machining subcontractors to Raytheon for the U.S.-built Tamir/SkyHunter program.

- The exact bill of materials for the interceptor, including the precise quantity, grade, and weight of SmCo and NdFeB magnets used in each seeker and actuator assembly.

- The division of labor for component manufacturing between U.S. and Israeli facilities under the new co-production agreements.

- Whether the technical specifications for magnets in the U.S.-built SkyHunter are identical to those in the Israeli-built Tamir, or if they have been "Americanized" with different materials.

# US Iron Dome Co-Production and DFARS Magnet Sourcing: The Permanent Magnet Supply Chain for US-Built Interceptors

## Executive Summary

The establishment of a U.S. production line for Israel’s Iron Dome missile defense system and its American variant, SkyHunter, creates a significant new demand signal for domestically sourced, high-performance permanent magnets compliant with U.S. defense regulations. Through the Raytheon-Rafael Area Protection Systems (R2S) joint venture, a new "all-up-round" interceptor production facility in East Camden, Arkansas, is now operational, supported by over a billion dollars in contracts and U.S. aid designated for replenishing Israeli stockpiles and equipping U.S. forces. This domestic manufacturing effort directly implicates the Defense Federal Acquisition Regulation Supplement (DFARS), which mandates that critical materials, including rare earth magnets, be sourced from non-adversarial nations.

With an expanded DFARS prohibition taking full effect on January 1, 2027—covering the entire supply chain from mining to finished magnet—prime contractors like Raytheon (RTX) must secure compliant sources for the magnets essential to interceptor guidance and control systems. These systems rely on high-temperature samarium-cobalt (SmCo) magnets for seeker head gimbals and high-coercivity neodymium-iron-boron (NdFeB) magnets for fin actuators. While DFARS provides an exemption for NdFeB magnets made from recycled material milled and sintered in the U.S., no such exemption exists for SmCo magnets, creating a significant sourcing challenge. This report analyzes the emerging U.S. Iron Dome production ecosystem, maps the DFARS compliance requirements onto the magnet supply chain, and identifies the key domestic magnet suppliers positioned to support this critical defense industrial base initiative.

## Iron Dome Co-Production Program Overview

The U.S.-Israeli collaboration on the Iron Dome system has evolved from financial support to direct industrial base integration. Over the last decade, the U.S. has provided billions in funding, including $1.6 billion between 2011 and 2021 and an additional $1 billion approved by Congress in 2022. This partnership deepened with the formation of the Raytheon-Rafael Area Protection Systems (R2S) joint venture in 2020, created explicitly to establish an Iron Dome production facility in the United States.

This initiative is driven by several factors:

- Replenishment Needs: Increased operational use of the system, particularly since October 2023, created an urgent need to replenish Israel's stockpile of Tamir interceptors, prompting a "multi-billion dollar contract" to accelerate production. A U.S. aid package approved in April 2024 allocated $5.2 billion (out of an $8.7 billion total) specifically for Israeli air and missile defense systems.

- U.S. Military Adoption: The U.S. military is procuring its own Iron Dome derivatives. The U.S. Army acquired two batteries for evaluation, and the U.S. Marine Corps is integrating the system as its Medium-Range Intercept Capability (MRIC) program, using the U.S. variant of the Tamir interceptor, known as SkyHunter. The Marine Corps' FY25 budget requested $111 million for 12 launchers and 242 missiles, and a separate planned order may include 1,840 interceptors.

- Industrial Base Security: The establishment of a U.S. production line is a strategic move to create a "robust American production base" and a shared U.S.-Israeli supply chain, enhancing resilience and ensuring interceptors can be delivered "efficiently and at scale".

The culmination of this effort is the new R2S facility in Arkansas, which received a $1.25 billion contract in late 2025 to begin supplying Tamir interceptors to Israel.

## US Production Facilities and Capabilities

The domestic production of the Tamir and SkyHunter interceptors is centered around a growing network of Raytheon/RTX facilities, with the new R2S plant in Arkansas as the centerpiece.

- R2S Facility (East Camden, Arkansas): Opened in November 2025, this is the first "all-up-round" production facility for Iron Dome interceptors in the United States. The joint venture's investment in the site grew from an initial $33 million to $63 million. The plant, which broke ground in February 2024, is slated to begin missile production in 2025 and will create up to 60 jobs. It is located at the Highland Industrial Park, building on RTX's existing presence in the area.

- Raytheon Facility (Tucson, Arizona): Tucson is a major hub for Raytheon's missile defense activities. Sources from 2014 and later confirm that Raytheon was producing components for the Tamir interceptor at its Tucson facility, which entered full production by July 2015. In 2020, the joint venture also agreed to a separate plant investment in Tucson.

- Raytheon Facility (Andover, Massachusetts): This facility is mentioned as being involved in the production of the seeker and guidance sections for missile systems, making it a likely contributor of critical components for the U.S.-built interceptors.

- Huntsville, Alabama: This location was noted as one of the sites that would benefit from the co-production arrangement, suggesting a role in engineering, prototyping, or component manufacturing.

This distributed manufacturing footprint indicates that while final assembly and integration of all-up-rounds will occur in Camden, critical subsystems and components are being produced across multiple specialized Raytheon sites.

## US vs Israeli-Built Components

The co-production model involves a deep integration of U.S. and Israeli supply chains. While the new Camden facility is capable of all-up-round production, the existing supply chain already features significant U.S. content.

Raytheon states that the "majority of Tamir missile components are procured through the Raytheon supply chain in the United States". Another source notes that the Israeli production line historically receives approximately half of its components from Raytheon. The U.S. version of the interceptor, SkyHunter, shares 95-99% commonality with the Israeli Tamir, meaning the designs are nearly identical.

The establishment of a U.S. production line for systems like the seeker, guidance, and control sections is the critical factor for magnet sourcing. The production of these magnet-intensive subsystems in U.S. facilities (like Tucson for missile integration and Andover for seekers) means the components used within them are subject to U.S. procurement law, specifically DFARS.

## Contract Values and Production Quantities

The scale of U.S. and Israeli demand is driving significant investment and large-scale production contracts.

- R2S Production Contract: In November 2025, the R2S joint venture was awarded a $1.25 billion contract to produce Tamir interceptors at the new Camden facility to supply Israel. This is the first production contract for the R2S partnership.

- U.S. Aid Package: The production surge is substantially funded by an $8.7 billion U.S. aid package approved in April 2024, which allocated $5.2 billion specifically for Israeli air and missile defense systems. One report suggests this funding could cover the production of "about 100 Iron Dome batteries and at least 14,000 Tamir interceptor missiles".

- U.S. Military Procurements: The U.S. Army previously acquired two Iron Dome batteries with 480 missiles in a 2019 deal valued at $373 million. The U.S. Marine Corps has a planned order for three batteries, 44 launchers, and 1,840 interceptors for its MRIC program. The FY25 budget request for MRIC includes $111 million for 242 missiles.

## Permanent Magnet Requirements for US-Built Interceptors

The Tamir and SkyHunter interceptors rely on high-performance permanent magnets for their agility and precision. These magnets are critical components within the guidance and control subsystems.

- Seeker Head: The interceptor uses an active radar and/or electro-optical seeker for terminal guidance. This seeker is mounted on a gimbal system that requires small, powerful motors to rapidly articulate the sensor. These gimbal motors use permanent magnets. Due to the high temperatures generated by aerodynamic heating on the missile's nose cone, these are likely high-temperature SmCo magnets.

- Fin Actuators: The missile is equipped with several steering fins for high-g maneuvering. Each fin is controlled by an actuator, typically a brushless DC motor that relies on a permanent magnet rotor. These actuators must perform under extreme thermal and vibratory stress, requiring high-temperature NdFeB or SmCo magnets.

- Control Section Electronics: Other sensors and electronic components within the guidance and control section may also contain permanent magnets.

Given the high-volume production plans, with thousands of interceptors required annually, the demand for these specialized, defense-grade magnets will be substantial.

## DFARS Compliance for US-Built Interceptor Magnets

Any interceptor or component produced in the U.S. under a Department of Defense contract must comply with DFARS. The final DFARS rule published on May 30, 2024, implements and expands upon these restrictions.

- The Regulation: DFARS clause 252.225-7052 explicitly lists SmCo and NdFeB magnets as "covered materials".

- The Deadline: Effective January 1, 2027, the rule prohibits the acquisition of any covered material that was "mined, refined, separated, melted, or produced" in a covered country (China, Russia, North Korea, Iran). This covers the entire upstream supply chain.

- Application: Because the US-built Tamir/SkyHunter interceptors are part of a U.S. defense procurement (for the USMC and funded by U.S. aid for Israel), the magnets within their U.S.-made components must be DFARS-compliant. Prime contractors like Raytheon are required to flow this clause down to their subcontractors.

There is a critical distinction in compliance pathways for the two key magnet types:

- NdFeB Magnets: The rule provides an exception for NdFeB magnets manufactured from recycled material, but only if the milling of the recycled material and the sintering of the final magnet occur in the United States.

- SmCo Magnets: There is no recycling exemption for SmCo magnets. As of January 1, 2027, the entire supply chain—from the mining of samarium and cobalt ore—must be free of covered countries. This presents a much higher compliance hurdle.

## Magnet Supply Chain for US Iron Dome

Given the stringent DFARS requirements, Raytheon and its subcontractors must source magnets for U.S. Iron Dome production from a very narrow list of qualified domestic suppliers. The likely Tier-1 suppliers for these magnet assemblies include:

- Electron Energy Corporation (EEC) / Permag: EEC, part of the Permag group, is the only vertically integrated U.S. producer of SmCo magnets, controlling the process from alloy melt to finished magnet. This makes them a critical supplier for the SmCo magnets likely used in the interceptor's seeker head.

- Arnold Magnetic Technologies: Arnold is a key U.S. defense supplier with a long history of producing high-performance SmCo (RECOMA series) and other magnets. They have publicly stated their commitment to establishing a fully DFARS-compliant supply chain for both NdFeB and SmCo magnets ahead of the 2027 deadline.

- Vacuumschmelze (VAC): While traditionally a German company, VAC has received a $94.1 million DoD award to establish a rare earth magnet manufacturing facility in Sumter, South Carolina, specifically to support defense needs and DFARS compliance. This plant will become a new domestic source once fully operational.

The extreme stress on the supply chain from surge demand for Iron Dome, combined with demand from other platforms like the F-35 (900 lbs of REEs) and DDG-51 destroyers (5,200 lbs of REEs), creates a significant risk of bottlenecks. Qualifying a second source for a critical missile component can take 12-24 months, making the existing capacity of these few suppliers a key strategic concern.

## Production Surge and Supply Chain Stress

The surge in demand for Iron Dome interceptors following October 2023 places immense pressure on an already constrained global supply chain for high-performance magnets. Israel's need to replenish its stocks, combined with new U.S. production, will test the limits of domestic magnet producers.

The SmCo supply chain is particularly vulnerable. Samarium is almost exclusively sourced from China, and cobalt is also a critical material with a concentrated supply chain. The DFARS rule's lack of a recycling exemption for SmCo means that, post-2027, SmCo magnets for U.S. defense must originate from fully compliant ore-to-magnet chains, which are still nascent outside of China. This regulatory pressure, combined with high demand, creates a significant risk for programs like the US-built Iron Dome that rely on these materials for high-temperature applications.

## Implications for Algamol

The ramp-up of U.S. Iron Dome co-production creates a concentrated and valuable opportunity for wastewater treatment and resource recovery technologies. The precision machining of SmCo and NdFeB magnets into final parts for seeker gimbals and fin actuators is a waste-intensive process. This waste stream, generated at Raytheon subcontractor facilities, is a key target.

- Concentrated Waste Streams: Instead of being diffuse, magnet machining waste for the Iron Dome program will be concentrated at a few Tier-2 supplier locations that specialize in grinding and finishing rare earth magnets.

- High-Value Waste: This machining sludge contains high concentrations of valuable and strategic materials, including samarium, cobalt, neodymium, and dysprosium.

- DFARS Compliance Driver: For NdFeB magnets, on-site recovery of machining waste provides a direct pathway to creating a DFARS-compliant feedstock. Under DFARS 252.225-7052, NdFeB magnets manufactured from recycled material are exempt, provided the milling and sintering occur in the U.S. An on-site recovery system would enable a Tier-2 supplier to create a secure, domestic source of recycled powder.

- Strategic Value for SmCo: While SmCo lacks a recycling exemption, the strategic importance of cobalt and samarium, coupled with the supply chain risks, makes on-site recovery and purification highly valuable for securing a resilient internal supply.

- Ideal Customer Profile: A large defense prime contractor like Raytheon has established Environmental, Health, and Safety (EHS) infrastructure and would be receptive to a pilot program that offers both cost savings and a tangible solution to a pressing supply chain and regulatory challenge. The priority target is the Tier-2 machining house supplying these finished magnet components to Raytheon's integration facilities in Tucson or Andover.

## Key Unknowns and Intelligence Gaps

While the strategic direction is clear, several critical pieces of information remain proprietary or classified:

- The specific Tier-1 and Tier-2 magnet suppliers and machining subcontractors to Raytheon for the U.S.-built Tamir/SkyHunter program.

- The exact bill of materials for the interceptor, including the precise quantity, grade, and weight of SmCo and NdFeB magnets used in each seeker and actuator assembly.

- The division of labor for component manufacturing between U.S. and Israeli facilities under the new co-production agreements.

- Whether the technical specifications for magnets in the U.S.-built SkyHunter are identical to those in the Israeli-built Tamir, or if they have been "Americanized" with different materials.

## Chapter 17

## Combined US-Israeli Interceptor Magnet Demand Under Surge

### Executive Summary

Post–October 2023 operational demand and subsequent procurement actions have driven expansion of Iron Dome Tamir interceptor production in both Israel and the United States, with a new U.S. “all-up-round” Tamir/SkyHunter facility in East Camden/Camden, Arkansas established explicitly to accelerate serial output and serve both Israeli Iron Dome and the U.S. Marine Corps’ MRIC program. Multiple sources tie this expansion to a U.S. aid package approved in April 2024 totaling $8.7B and including $5.2B dedicated to air and missile defense (Iron Dome, David’s Sling, and laser defense). The practical implication for permanent magnets is that surge missile output increases demand for high-temperature and high-reliability permanent magnets used in guidance and control actuators, and that these magnets are subject to tightened sourcing rules effective January 1, 2027 under DFARS-related restrictions and provenance expectations.

The binding constraint is likely not magnet design knowledge (which is well understood in broad terms) but supply-chain capacity and compliance for high-temperature SmCo magnets, because U.S. industrial statements and government reporting emphasize dependence on foreign rare-earth processing and limited domestic specialty magnet production, with samarium supply and SmCo market share heavily concentrated in China and only a very small U.S. producer base for SmCo magnets. In contrast, NdFeB magnets are also strategically constrained but have more pathways for domestic capacity expansion (e.g., Mine-to-Magnet investments) and recycling-related compliance strategies than SmCo, as reflected by DoD investment programs and the attention to NdFeB processing-chain completeness in China.

### Baseline Production

Public sources used in this dataset provide only limited, sometimes secondary estimates for pre-surge annual Tamir production, with one internal “input document” estimate citing approximately 1,000–2,000 Tamirs per year pre–October 2023. Several public sources instead anchor baseline demand through stockpile and procurement indicators (e.g., U.S. Army acquisition of 480 missiles in the 2019–2021 timeframe, and U.S. Marine Corps plans to acquire 1,840 Tamir/SkyHunter missiles for MRIC). These indicators establish that U.S. demand existed prior to the current surge and that Israel–U.S. co-production channels were already in place before the Arkansas plant opened, including U.S. manufacture of a majority share of components in some periods.

Baseline unit-cost anchors (useful for inferring procurement volumes when totals are known) put Tamir interceptor costs in the $40,000–$50,000 range in multiple references, though broader ranges also appear depending on accounting conventions and urgency premiums.

### Surge Scenarios

The surge scenarios in open reporting are most defensible when framed as (i) a production-capacity step change (new U.S. all-up-round plant plus accelerated Israeli serial lines) and (ii) a range of annual output targets and procurement quantities inferred from budgets and public contract statements, because many official announcements do not disclose annual unit production numbers. One estimate in the dataset explicitly frames post–October 2023 surge demand as “8,000–12,000+ interceptors per year” for Israel, reflecting the intensity and duration of high-tempo engagements and stockpile replenishment pressures. A separate internal estimate in the dataset states Tamir production could “surge to potentially 10,000+/year” post–October 2023, reinforcing the possibility of five-to-tenfold increases over a ~1,000–2,000/year baseline estimate.

For U.S. demand, MRIC procurement provides a bounded near-term (FY25) quantity signal, with a Marine Corps FY-25 budget request that includes $111M to support purchase of 12 launchers and 242 missiles, and a separate contract action covering 80 missiles in 16 all-up-round magazines with delivery scheduled for April 2027 (work in Tucson, AZ). Separately, Israeli–U.S. industrial reporting cites a U.S. Marine Corps order for 2,000 SkyHunter missiles as the intended load for the U.S. production line, indicating an upper-bound programmatic target even where annual profiles are not disclosed.

Given these constraints, this report defines three surge scenarios as planning constructs rather than confirmed production commitments:

- Scenario A (Moderate Surge): Israel replenishment plus early U.S. MRIC fielding, bounded by FY25 MRIC quantities and the ramp-up implied by “accelerating serial output.”

- Scenario B (High Surge): Israel demand toward the 8,000–12,000+/year range plus continued U.S. MRIC procurement growth during 2024–2026 ramp period.

- Scenario C (Maximum Surge plus U.S. adoption): High Israeli demand plus U.S. MRIC full fielding across three battalions during 2026–2028, using published fielding windows and contract growth to approximate sustained demand.

### Procurement Signals

Multiple U.S. and Israeli public statements connect accelerated interceptor production to large funding packages and contracts, even when unit quantities are not disclosed. U.S. Congress-approved April 2024 aid is repeatedly cited as $8.7B total, with $5.2B earmarked for air and missile defense programs including Iron Dome and David’s Sling, establishing a multi-program funding pool that can drive magnet demand across multiple interceptor families beyond Tamir alone. On the industrial side, the Raytheon–Rafael joint venture (R2S) received a $1.25B contract to supply Tamir interceptors to Israel, and the contract disclosure coincided with inauguration/opening of the U.S. Tamir production facility. Several sources also record a $33M capital investment for the new U.S. facility, while one state economic development release describes the project as an approximately $63M investment, implying possible scope growth or additional related investments beyond the originally cited figure.

MRIC-specific procurement signals include contract values reaching $412.5M in total (including a potential $380M order), which indicates an expanding U.S. demand curve for Tamir-derived interceptors even if annual unit numbers beyond FY25 are not stated in those sources.

### Magnet-Intensive Subsystems

Open sources in this dataset generally do not disclose per-interceptor permanent magnet mass, magnet grade, or exact magnet counts, and at least one technical source explicitly notes it does not address “the weight of permanent magnets or the quantity of actuator motors in guided munitions.” Nonetheless, multiple sources support the qualitative mapping that guided interceptors use permanent magnets in actuator motors and fin actuation, and that missile subsystems can use SmCo magnets (often chosen for thermal stability and resistance to demagnetization) as well as NdFeB magnets (common in fin actuators).

Within the dataset, one internal synthesis asserts that Tamir includes permanent-magnet-containing components in the guidance section (seeker head motor and gimbal actuators) and control section (fin actuators), with a possibility of additional motor-related magnets closer to propulsion-related structures. A separate internal table-level statement ties SmCo magnets to missile propulsion-system magnets and control-actuator magnets, supporting the inference that at least some interceptor subsystems in the relevant performance envelope may require SmCo rather than standard NdFeB grades.

### Parametric Magnet Demand Model

Because the dataset lacks publicly supported values for magnet mass per interceptor or magnet mass per actuator, this report models magnet demand parametrically and ties the model to confirmed interceptor quantities and credible surge ranges. Let:

- (N_T) = annual Tamir-equivalent interceptor output (Israel + U.S. co-production).

- (m_{SmCo}) = average SmCo magnet mass per interceptor (kg/interceptor), covering seeker/gimbal and any high-temperature actuator use where SmCo is selected.

- (m_{NdFeB}) = average NdFeB magnet mass per interceptor (kg/interceptor), covering fin actuators and other permanent-magnet motor functions where NdFeB is used.

Then annual magnet demand in mass terms is:

- SmCo demand (kg/year) = (N_T \times m_{SmCo}).

- NdFeB demand (kg/year) = (N_T \times m_{NdFeB}).

The table below summarizes the interceptor quantity inputs available from sources and how they map into the three scenarios; it intentionally keeps magnet mass outputs symbolic due to absent public per-interceptor magnet mass data.

| Scenario | Israel Tamir demand anchor | U.S. MRIC SkyHunter demand anchor | Combined annual output basis | Magnet mass output form |
| --- | --- | --- | --- | --- |
| A | No official annual rate disclosed; constrained by stated intent to accelerate serial output and replenish stockpiles. | FY25 request: 242 missiles (plus 12 launchers) and early fielding timeline. | (N_T) treated as “Israel replenishment + 242 U.S. missiles” for minimum planning case. | SmCo = (N_T m_{SmCo}); NdFeB = (N_T m_{NdFeB}). |
| B | Estimate of 8,000–12,000+ interceptors/year under surge conditions is explicitly stated in one source set. | MRIC contract growth to $412.5M suggests expanding procurement beyond initial buys, but annual quantities not stated in those contract summaries. | (N_T) approximated by Israel surge range plus incremental U.S. production capacity ramp during 2024–2026. | SmCo = (N_T m_{SmCo}); NdFeB = (N_T m_{NdFeB}). |
| C | Israel surge range as in Scenario B (upper bound), recognizing that official quantities are often undisclosed. | Full fielding across three battalions scheduled 2026–2028 and delivery window signals sustained procurement, with “full-rate production begins this year” in one MRIC description. | (N_T) treated as Israel surge range plus sustained U.S. MRIC missile deliveries through 2026–2028. | SmCo = (N_T m_{SmCo}); NdFeB = (N_T m_{NdFeB}). |

### SmCo Supply Chain Capacity

Multiple sources describe SmCo supply as unusually concentrated and therefore vulnerable under surge conditions, including a claim that China accounts for “more than 90 percent of the global samarium–cobalt magnet market.” A market estimate in the dataset puts global SmCo magnet production capacity at approximately 2,400 tonnes per year and indicates aerospace and defense account for over 30% of SmCo consumption, which implies that a defense surge can materially compete for a constrained global capacity pool even if defense is not the dominant end market in every segment. Separately, U.S. industrial statements emphasize that through EEC, Permag is “the only U.S. producer” of SmCo magnets, reinforcing the thinness of domestic SmCo capacity and the likely need for allied/foreign upstream sourcing even when final manufacturing is domestic.

From a surge-production perspective, two additional supply-side constraints are salient in the dataset: (1) samarium supply dependence (with one source stating “Samarium supply: 95%+ from China”), and (2) cobalt supply concentration and disruption risk (with one internal summary stating DRC accounted for 76% of mined output in 2024 and referencing a February 2025 DRC export ban).

### NdFeB Supply Chain Considerations

NdFeB magnets are described in government reporting as strategically important and “irreplaceable” in key defense applications including missile guidance and fin actuators, and the U.S. is shown as having imported 75% of sintered NdFeB magnet supply from China in 2021 in a congressional report. A GAO report also notes that China is the only country with processing capabilities for each stage of the NdFeB permanent magnet supply chain, which creates systemic risk when defense missile production surges. However, the dataset also documents significant U.S. industrial-policy investments aimed at expanding domestic rare-earth processing and magnet manufacturing, including DoD awards totaling more than $439M since 2020 and specific funding to MP Materials (with projected full-scale milestones through 2025), Lynas USA (oxide capability by 2026), Noveon Magnetics (San Marcos, TX magnet facility), and E‑VAC Magnetics (commercial-scale magnet capability by 2025).

From a manufacturing yield perspective (relevant to effective supply), one U.S. government report states that “final magnet machining has a recovery rate of 60 to 90 percent” depending on magnet size and complexity, which implies significant swarf/sludge and scrap generation that can tighten effective supply during surge periods.

### Bottlenecks

Several sources directly connect defense missile production surges (2024–2025) to increased demand for rare-earth permanent magnets, and also emphasize that concentrated overseas supply chains for rare-earth magnets are a limiting factor for precision-guided munitions output under surge. Separately, one source identifies “specific bottlenecks like rocket motors” as constraints across missile lines, underscoring that magnets compete with other constrained subsystems for industrial attention and qualification bandwidth.

For magnets specifically, the dataset supports five bottleneck categories that align with the user’s framing, though not all are quantified in the public record:

- Raw materials bottleneck: samarium supply dependence and cobalt supply concentration create risk for SmCo production under surge conditions.

- SmCo manufacturing bottleneck: a narrow supplier base and limited domestic production footprint for SmCo magnets raises the likelihood that SmCo becomes the binding constraint for high-temperature actuator and seeker assemblies.

- NdFeB processing bottleneck: China’s unique completeness across NdFeB processing stages and high import dependence for sintered NdFeB creates vulnerability during periods of heightened demand and/or export restrictions.

- Qualification bottleneck: DFARS-related sourcing and provenance requirements are tightening into 2025–2027 with an expanded prohibition effective January 1, 2027, raising the time and cost to qualify alternative sources for magnets and their upstream materials.

- Compliance asymmetry for SmCo: one dataset source explicitly states “SmCo has NO recycling exemption under DFARS,” which implies that recycling-driven compliance pathways may be more constrained for SmCo than for NdFeB under defense procurement rules as interpreted in this dataset.

### Machining Waste Implications

Even though per-interceptor magnet mass is not publicly supported in the provided sources, machining yield data implies that a substantial fraction of magnet material can become machining swarf/sludge, which scales with interceptor production rates and precision requirements. One dataset source provides SmCo swarf composition (approximately 23% Sm and 50% Co by weight, with Fe, Cu, and Zr constituents), implying that SmCo machining waste is not only an environmental liability but also a concentrated strategic-metal stream whose value and compliance relevance increase during surge production.

This matters for surge conditions because a higher missile output implies higher magnet machining throughput at Tier-2/Tier-3 suppliers, which in turn implies higher volumes of magnet-bearing waste streams and greater incentives to treat and recover valuable metals domestically—especially when procurement rules and supply chain security concerns are tightening in parallel with surge demand.

### Assumptions and Uncertainties

Most platform-specific magnet composition, grade selection, and per-interceptor magnet mass values are not available in the cited public sources, and at least one technical source explicitly notes it does not provide permanent magnet weights or actuator motor quantities in guided munitions, limiting the defensibility of any fixed “kg of magnets per interceptor” estimate from this dataset alone. Many production announcements (including large contracts and aid-funded procurements) do not disclose interceptor quantities, and multiple sources explicitly state that quantities were not provided for security or procurement reasons, which forces scenario modeling to rely on bounded procurement signals (e.g., FY25 MRIC missiles) and a small number of surge-demand estimates (e.g., 8,000–12,000+ interceptors/year). Finally, supply-side capacity for SmCo is described as highly concentrated and vulnerable to disruption, but the dataset provides limited, potentially inconsistent global capacity figures across market reports, so capacity analysis should be treated as indicative rather than definitive unless validated against primary industry production statistics.

## Chapter 18

## The SmCo DFARS Compliance Gap

### Executive Summary

Samarium-cobalt (SmCo) permanent magnets are irreplaceable in high-temperature defense and aerospace applications, including missile guidance systems, radar traveling-wave tubes (TWTs), and satellite actuators, where performance above 300°C is required. However, the U.S. defense industrial base faces a critical compliance gap concerning these magnets under the Defense Federal Acquisition Regulation Supplement (DFARS). The final rule, published May 30, 2024, expands sourcing prohibitions to the entire supply chain—from mine to magnet—effective January 1, 2027, for "covered materials" sourced from China, Russia, Iran, or North Korea. While SmCo magnets are explicitly defined as a covered material, they lack a critical compliance pathway available to their lower-temperature counterparts, neodymium-iron-boron (NdFeB) magnets.

The central issue is a significant asymmetry in the regulation: DFARS provides an exemption for NdFeB magnets manufactured from recycled feedstock that is milled and sintered in the United States. SmCo magnets have no such recycling exemption. This gap is compounded by a precarious supply chain; approximately 95% of the world's samarium is processed in China, and while non-China processing is emerging in Europe, domestic U.S. capacity is nascent. The Department of Defense (DoD) has invested over $439 million in its "Mine-to-Magnet" initiative, but these efforts have predominantly focused on the NdFeB supply chain.

Without a policy remedy, defense programs reliant on SmCo magnets will face an untenable choice between sourcing non-compliant materials or facing significant production disruptions after the 2027 deadline. This report analyzes the specifics of the SmCo compliance gap, evaluates the state of the global supply chain, and proposes actionable policy remedies, including extending the recycling exemption to SmCo, issuing a class nonavailability determination (NAD), and strategically investing in domestic SmCo recycling and stockpiling.

### DFARS Treatment of Samarium-Cobalt Alloys

Samarium-cobalt (SmCo) magnets are explicitly identified as a "covered material" under DFARS 225.7018 and the corresponding contract clause 252.225-7052. These regulations prohibit the DoD from procuring certain sensitive materials that are mined, refined, separated, melted, or manufactured in "covered countries," defined as China, Russia, Iran, and North Korea.

The prohibition is being implemented in phases. According to the final rule published on May 30, 2024, the restriction through December 31, 2026, covers the later stages of production, such as "Melting samarium with cobalt to produce the samarium-cobalt alloy" and all subsequent manufacturing steps like powder formation, pressing, sintering, and magnetization.

Effective January 1, 2027, this restriction expands dramatically to cover the entire supply chain. From that date, it will be prohibited to acquire any SmCo magnet if any part of its production—from the mining of cobalt and samarium ore or feedstock, through all stages of processing and manufacturing—occurred in a covered country. This expanded prohibition also explicitly includes recycled material in its scope, a critical distinction from the treatment of NdFeB magnets. This restriction is in addition to any applicable restrictions on the melting of specialty metals under DFARS 252.225-7009.

### The Recycling Exemption Asymmetry

A central challenge for SmCo compliance is a critical asymmetry in the DFARS rules compared to NdFeB magnets. The statute (10 U.S.C. 4872) and the implementing DFARS clause 252.225-7052 provide a specific exemption for NdFeB magnets manufactured from recycled material. This exemption allows for the procurement of an end item containing an NdFeB magnet made from recycled stock, provided that the "milling of the recycled material and sintering of the final magnet takes place in the United States".

This pathway does not exist for SmCo magnets. The Federal Register explicitly notes that the legislative amendments creating the NdFeB recycling exemption "did not add exceptions for the remaining covered materials," which includes SmCo. This means that even if SmCo scrap is recovered and re-processed entirely within the United States, the resulting magnet would be non-compliant if the original samarium or cobalt ore was mined or initially processed in a covered country. The lack of a recycling exemption for SmCo is a significant policy gap, likely stemming from the fact that the SmCo recycling market is less commercially established than that for NdFeB. This asymmetry effectively closes off a vital potential compliance pathway for SmCo, forcing complete reliance on a compliant primary supply chain that is not yet mature.

### Samarium Supply Chain Analysis

The viability of a DFARS-compliant SmCo supply chain is fundamentally constrained by the global geography of samarium processing. Samarium, a middle-heavy rare earth element, is almost entirely processed in China. Various sources estimate that China controls between 70% and 99% of global rare earth refining and processing, with a near-monopoly on heavy rare earths. This dominance has been identified as a significant national security risk, as demonstrated by China's use of export controls on rare earths, including samarium. A USGS analysis concluded that a trade disruption of samarium from China would have the largest probability-weighted negative impact on U.S. GDP of all rare earths, estimated at nearly $4.5 billion.

While the situation is critical, a non-Chinese supply chain is beginning to emerge. Key players include:

- Solvay at its facility in La Rochelle, France, which is now the largest separation facility outside of China and has begun producing samarium oxide.

- Neo Performance Materials (Silmet) in Estonia, which is the only other commercial rare earth separation plant in Europe.

- Lynas Rare Earths, which operates in Australia and Malaysia and is developing heavy rare earth separation capacity in Texas with DoD funding.

However, these efforts will take years to reach a scale sufficient to displace Chinese production, with DoD assessments suggesting it could take a decade or more to establish fully independent domestic sources.

### Cobalt Supply Chain Analysis

The cobalt supply chain presents a different but related set of challenges. While cobalt is not sourced from DFARS-prohibited countries, its supply chain is highly concentrated. The Democratic Republic of the Congo (DRC) accounts for approximately 70-76% of the world's mined cobalt. This concentration creates significant supply risk, which is compounded by the fact that an estimated 80-90% of DRC-origin cobalt is further refined in China.

Furthermore, the DRC cobalt sector is plagued by ethical concerns, including documented cases of forced labor and human rights abuses, which present reputational and legal risks for defense contractors. The U.S. government has recognized this vulnerability and is taking steps to reshape cobalt supply chains, including supporting new refining capacity and considering additions to the National Defense Stockpile.

### US SmCo Manufacturer Sourcing Practices

The United States has a very limited domestic industrial base for SmCo magnet production. Electron Energy Corporation (EEC), now part of the unified Permag brand, is the only U.S.-based, vertically integrated domestic supplier of SmCo magnets, meaning it produces the alloy in-house in Landisville, PA. Arnold Magnetic Technologies also produces its RECOMA® SmCo grades in the U.S. (Rochester, NY) and describes itself as an integrated supplier, though its level of vertical integration for SmCo alloy is less clear.

Both companies are actively pursuing DFARS compliance. Arnold has announced a target of mid-2026 for its supply chain to meet the 2027 requirements and has entered strategic agreements with non-China sources like Solvay and Less Common Metals (LCM) to secure samarium oxide. Similarly, Permag (EEC) is working with compliant global suppliers and is part of a tri-national partnership with Solvay and LCM to establish a secure samarium supply chain.

However, a critical gap remains: while these companies can melt alloy in the U.S. to meet current DFARS requirements, the ultimate origin of the samarium metal they use is a significant vulnerability. Publicly available information does not confirm whether their samarium feedstock can be traced to a non-covered country mine and separation facility, which will be required after the January 1, 2027 deadline.

### Nonavailability Determination for SmCo

DFARS provides a mechanism to address supply gaps through nonavailability determinations (NADs). Under DFARS 225.7018-4, DoD can issue both individual NADs for a single contract and class NADs that apply more broadly. An NAD can be granted if compliant materials "of satisfactory quality and quantity, in the required form, cannot be procured as and when needed at a reasonable price".

- Individual NADs can be approved by the head of a contracting activity.

- Class NADs require approval from the Under Secretary of Defense for Acquisition and Sustainment (USD(A&S)) and must undergo a public notice period of at least 30 days to solicit information from interested parties.

Historically, DoD has issued national security waivers for non-compliant, high-performance SmCo magnets, acknowledging the persistent lack of compliant sources. Given the looming 2027 deadline and the immature state of the domestic SmCo supply chain, many industry stakeholders have recommended that DoD proactively issue a class NAD for SmCo magnets to prevent widespread procurement disruptions. Without a class NAD, every contract requiring SmCo magnets would need an individual determination, creating an unsustainable administrative burden for high-volume defense programs.

### DoD Mine-to-Magnet Investments

The Department of Defense has committed over $439 million since 2020 to re-establish a domestic "Mine-to-Magnet" supply chain for rare earth materials under authorities like the Defense Production Act. This initiative is a direct policy response to the supply chain vulnerabilities highlighted by China's market dominance.

However, a review of the funded projects reveals a strong emphasis on the NdFeB magnet supply chain. Major awards have gone to:

- MP Materials for light and heavy rare earth separation and NdFeB magnet production.

- Lynas USA to establish light and heavy rare earth separation facilities in Texas.

- Noveon Magnetics and E‑VAC Magnetics (a subsidiary of VAC) for domestic NdFeB magnet manufacturing and recycling.

While these investments are critical, there is a comparative lack of large-scale, publicly announced DoD funding specifically targeting the SmCo supply chain. Although President Trump issued a DPA Title III Determination that included "samarium-cobalt magnet manufacturing," subsequent large-scale investments have largely prioritized NdFeB. This funding gap overlooks the fact that for many high-temperature defense applications, SmCo is more critical and has fewer substitutes than NdFeB.

### SmCo Recycling

Technical literature demonstrates that recycling SmCo magnets is feasible, with multiple processes capable of achieving high recovery rates for both samarium and cobalt. Promising methods include:

- Hydrometallurgy: Acid leaching followed by solvent extraction or precipitation can recover over 95% of Sm and Co in laboratory settings.

- Pyrometallurgy: Processes like selective chlorination have shown samarium recovery yields as high as 99.7% at the bench scale.

- Ionic Liquids: Oxidative dissolution using ionic liquids has been demonstrated to recover metals effectively, with the solvent being reusable over multiple cycles.

Despite this technical feasibility, commercial-scale SmCo recycling is not well-established. This is primarily due to economic factors. The market for SmCo magnets is significantly smaller than for NdFeB, meaning the volume of manufacturing scrap and end-of-life magnets is lower. This makes it difficult to achieve the economies of scale necessary for a dedicated SmCo recycling facility to be profitable, especially when competing with primary raw materials. While companies like Cyclic Materials are building capacity to process recycled magnets, their initial focus is on the more abundant NdFeB-containing feedstock.

### Policy Remedies

The SmCo DFARS compliance gap requires proactive policy intervention to avoid significant disruption to critical defense programs. Based on the analysis, several remedies should be considered:

- Extend the DFARS Recycling Exemption to SmCo: Congress and the DoD should amend 10 U.S.C. 4872 and the corresponding DFARS clause to create a recycling exemption for SmCo magnets, analogous to the one for NdFeB. This would enable U.S. companies like EEC/Permag to use recycled SmCo feedstock (from any origin) as long as the final milling and sintering occurs domestically, immediately creating a viable compliance pathway.

- Issue a Class Nonavailability Determination (NAD) for SmCo: As an interim measure, the USD(A&S) should issue a class NAD for SmCo magnets and alloys. This would provide temporary relief for all defense contractors, acknowledging that a fully compliant primary supply chain does not currently exist and will take years to build.

- Fund Domestic SmCo-Specific Infrastructure: DoD's Mine-to-Magnet and DPA Title III investments should specifically target gaps in the SmCo supply chain. This includes funding for domestic samarium separation from U.S. or allied rare earth concentrates and co-funding commercial-scale SmCo recycling facilities, potentially co-located with existing magnet producers.

- Expand the National Defense Stockpile (NDS): Congress should authorize and fund an expansion of the NDS to acquire and hold not just samarium oxide, but also DFARS-compliant SmCo alloy and finished magnet blocks. This would create a strategic buffer to mitigate supply shocks and provide a ready source of material for defense surges.

- Qualify Allied Sources: The U.S. should formalize agreements that recognize SmCo materials produced and processed in allied nations with secure supply chains (e.g., samarium oxide from Solvay in France, alloy from VAC in Germany or LCM in the UK) as compliant sources under DFARS, promoting "friend-shoring" of critical capabilities.

### Compliance Timeline Analysis

The timeline for establishing a fully DFARS-compliant SmCo supply chain extends well beyond the January 1, 2027, deadline. While individual companies like Arnold and Permag are targeting compliance by mid-2026, this relies on a nascent and fragile non-Chinese primary supply chain.

Key timeline constraints include:

- Domestic Separation: DoD assessments indicate it can take a decade or longer to establish new domestic sources for rare earths. Even with investment, facilities for separating samarium from concentrate are unlikely to be operating at scale in the U.S. before the late 2020s or early 2030s.

- Regulatory Action: Extending the recycling exemption would require a federal rulemaking process, which typically takes 12-18 months at a minimum. Issuing a class NAD requires a 30-day public notice period but could be accomplished more quickly if prioritized by DoD leadership.

- Facility Construction: New processing plants and magnet factories, such as those being built by MP Materials and Lynas, have construction and commissioning timelines of 2-4 years.

Given these realities, it is highly probable that the defense industry will not have access to sufficient volumes of fully compliant SmCo magnets by 2027 without interim policy relief, such as a class nonavailability determination.

### Implications for Algamol

The SmCo DFARS compliance gap creates a significant strategic opportunity for wastewater treatment and resource recovery technology providers like Algamol. The challenges outlined in this analysis directly enhance the value proposition for on-site recovery of samarium and cobalt from magnet manufacturing waste streams.

- Direct Compliance Enabler (with Policy Change): If DFARS is amended to include a recycling exemption for SmCo (Remedy 1), Algamol's technology becomes a direct compliance enabler. A magnet manufacturer like EEC/Permag could use an on-site Algamol system to recover samarium and cobalt from its own machining swarf, re-introduce it into its alloy production, and certify the final magnet as DFARS-compliant under the recycling provision.

- Strategic Value Creation (Current Policy): Even without a recycling exemption, on-site recovery of SmCo waste has immense strategic value. The recovered cobalt (~50 wt% of SmCo scrap) and samarium have significant economic value, estimated at $12,000-$17,000 per tonne of swarf, primarily from cobalt. By recovering these materials, a defense contractor can hedge against price volatility, reduce reliance on external suppliers, and improve supply chain resilience.

- Policy-Relevant Capability: Algamol can position itself as a provider of a key enabling technology that supports DoD's industrial base resilience goals. By demonstrating a commercially viable method for SmCo recycling at the point of waste generation, Algamol can provide data to support policy changes, such as the extension of the recycling exemption. Engaging with DoD's Industrial Base Policy office and advocating for such changes, with EEC as a potential demonstration partner, could be a powerful go-to-market strategy.

### Key Unknowns and Intelligence Gaps

Despite extensive analysis of public records, several critical questions remain that are likely answered only in proprietary or classified domains:

- Is the Department of Defense actively considering or drafting a class nonavailability determination specifically for samarium-cobalt magnets ahead of the 2027 deadline?

- What are the precise primary sourcing chains for samarium and cobalt metal for the key U.S. SmCo producers (EEC/Permag and Arnold)? Can they currently trace their feedstock to a non-covered country mine?

- What is the true volume of SmCo manufacturing scrap and swarf generated annually within the U.S. defense industrial base, which would define the total addressable market for a recycling technology?

- Will the January 1, 2027, deadline be strictly enforced for SmCo-dependent programs, or is it likely to be extended or modified as the date approaches and supply gaps remain?

- What are the current holdings, if any, of samarium metal, samarium oxide, or SmCo alloy within the National Defense Stockpile?

Answering these questions would require direct engagement with DoD program offices, defense prime contractors, and the magnet manufacturers themselves.

## Chapter 19

## Bench-Scale Validation Protocol

### Objective and scope

This protocol is designed to validate an algae-based biosorption approach for removing and recovering rare earth elements and cobalt from defense magnet machining coolant streams, using a staged program that starts with synthetic solutions, transitions to real coolant, and then proceeds to continuous column testing.

The experimental scope explicitly includes batch sorption testing (pH effects, dose-response, kinetics, and equilibrium/isotherms) run in triplicate with controls, followed by column tests for breakthrough and scale-up assessment.

### Test matrix design

The work should be executed as three phases to separate fundamental biosorption behavior from real-matrix effects (oils/emulsions, suspended solids, and competing ions) and then to confirm continuous-flow performance.

Phase 1 uses synthetic wastewaters to map pH, contact time, temperature, and biosorbent dosage over the specified screening windows (pH 3–9, 5–360 min, 0.5–10 g/L, and 20–40 °C).

Phase 2 repeats the key Phase 1 conditions using real machining coolant (or coolant-derived aqueous phase) as the feed and includes pre-treatment for oil/emulsion removal prior to biosorption tests where needed.

Phase 3 uses fixed-bed column tests to generate breakthrough curves and dynamic capacities, because column (fixed-bed) testing is the standard pathway for continuous flow and breakthrough studies after batch equilibrium/kinetics work.

### Recommended experimental matrix

The table below provides a practical minimum matrix aligned to the required screening windows and commonly reported biosorption behaviors (rapid uptake over 20–60 min with equilibrium typically within 2–6 h).

| Sub-test | Purpose | Conditions to run | Primary response variables |
|---|---|---|---|
| pH edge (batch) | Identify pH window where uptake is maximized without confounding hydroxide precipitation | pH = 3, 4, 5, 6, 7, 8, 9 at fixed dose and time, 20–40 °C | % removal, q (mg/g), evidence of precipitation (turbidity/solids) |
| Kinetics (batch) | Determine time to reach near-equilibrium and fit kinetic models | t = 5, 10, 15, 30, 60, 120, 240, 360 min (or equivalent) within the allowed 5–360 min window | PFO/PSO rate constants; time to 90% of equilibrium uptake |
| Isotherms (batch) | Determine qmax and affinity constants | Multiple C0 points per metal system; fit Langmuir/Freundlich models | Langmuir qmax, KL; Freundlich parameters |
| Dose-response (batch) | Determine minimum dose needed to meet removal targets | 0.5–10 g/L (screen) at fixed pH and time | % removal vs g/L, q (mg/g) vs g/L |
| Regeneration cycling | Determine reusability and metal recovery | ≥5 adsorption/desorption cycles with acid stripping 0.01–1.0 M HCl/HNO3 | % desorption, capacity retention, eluate quality |

### Synthetic wastewater preparation

Synthetic matrices should be prepared from analytical-grade parent salts and made fresh as working solutions for each experimental run to reduce variability and contamination risk.

Two synthetic wastewater families are required to represent the two defense-relevant magnet waste streams targeted in this protocol: SmCo-dominant streams (cobalt and samarium) and NdFeB-dominant streams (neodymium/praseodymium with dysprosium as the high-temperature performance driver).

### SmCo-type synthetic matrix

Prepare an SmCo-type solution using SmCl3, CoCl2, FeCl3, and CuSO4, targeting Sm = 50–500 mg/L and Co = 200–2000 mg/L as the primary concentration ranges for the bench challenge tests.

If the test plan includes multicomponent interference studies (recommended for real coolants), Fe and Cu should be retained as co-solutes because multicomponent systems can show reduced uptake due to competition for limited binding sites.

### NdFeB-type synthetic matrix

Prepare an NdFeB-type solution using NdCl3, PrCl3, DyCl3, and FeCl3, targeting Nd = 100–1000 mg/L and Dy = 10–100 mg/L for bench challenge tests.

Because many biosorption studies on REEs report optimal performance near pH ≈5 for dried biomass systems, the initial pH screening should include a dense set of points around pH 4–6 even though the protocol-wide screening window extends from pH 3–9.

### Algae biomass preparation

Biomass should be produced and handled in ways that support both (i) bench screening with dried or otherwise stabilized biosorbent and (ii) eventual transition to immobilized/biofilm formats suitable for columns, since suspended microbial biomass can create pressure drops and channeling in packed beds.

If the Algamol concept uses acid-tolerant extremophile algae, one demonstrated approach is to grow Galdieria sulphuraria to OD 4, concentrate by centrifugation, and resuspend to OD 10 for sorption testing against multi-metal solutions.

Because multiple sources report that inactive (dead) biomass can be effective and often has faster kinetics than living biomass, bench tests should include at least one “inactive biomass” condition (e.g., freeze-dried) to quantify achievable capacities without growth-dependent confounders.

### Pre-treatment module

Real machining coolants can contain oils and stable emulsions, so the protocol should include a pre-treatment step to remove oil/emulsion before biosorption (especially before column tests) using an ultrafiltration module as a front-end unit operation.

If pre-treatment is used, evaluate biosorption on both (i) raw coolant aqueous phase after coarse settling/filtration and (ii) UF-treated permeate to quantify how emulsion removal changes apparent uptake and analytical stability.

### Batch sorption tests

Batch testing should be conducted in triplicate and include blanks and controls, because triplicate batch formats are explicitly used in multiple biosorption studies and are required by this protocol’s design criteria.

#### Batch setup

A representative REE batch format is to contact dried biosorbent dosages in the 0.5–3 g/L range with fixed-volume solutions under agitation and temperature control; one detailed example used 0.2 L solution, 90 min contact, 300 rpm agitation, and temperatures of 21, 35, and 50 °C with pH screening between 3 and 5.

A representative smaller-volume batch format (suitable for high-throughput screening) incubates 10–20 mg dry biomass in 2 mL metal solutions for ~3 h under constant shaking at room temperature, with centrifugation prior to analysis.

#### pH effects

pH should be treated as a primary control variable because uptake is strongly pH-dependent across biosorption systems, and higher uptake of cationic metals is often observed as pH increases into the ~4–6 range (until precipitation becomes significant).

For Nd(III) on Chlorella vulgaris using magnet-derived leachate, one study found an optimum pH of 5 at 21–35°C, while at 50°C the optimum pH shifted to ~4 due to Nd precipitation risk at higher pH combined with higher temperature.

Independent work on REE biosorption also identified an optimum equilibrium pH of ~4.9 ± 0.2 for favorable REE uptake on algal biomass, reinforcing the need to resolve performance around pH ~5.

#### Kinetics and equilibrium

Kinetic tests should capture early-time uptake because many systems exhibit rapid initial adsorption (often within the first 20–60 minutes) followed by a slower approach to equilibrium, and equilibrium for cationic metals is commonly reached within 2–6 h in biosorption studies.

For Nd(III) biosorption on C. vulgaris, equilibrium was reported to be reached within 30 minutes across tested temperatures, and the kinetic data fit a pseudo-second-order model with R2 > 0.99.

For some microbial/algal systems, much faster equilibration is also plausible; for example, Ce(III) adsorption to tested cyanobacterial biomasses reached equilibrium within ~5 minutes under the reported conditions.

#### Equilibrium and isotherm modeling

Equilibrium isotherms should be fit using standard models such as Langmuir and Freundlich, which are commonly applied across biosorption studies and explicitly used in REE biosorption work.

For Nd(III) on C. vulgaris, a Langmuir fit with R2 = 0.98 and qmax = 188.68 mg/g (KL = 0.029 L/mg) was reported at 35°C and pH 5, providing a benchmark for what “high-performing” REE uptake can look like in a controlled system.

For REE mixtures (La, Ce, Eu, Yb), maximum biosorption uptakes from a Langmuir model were reported as 154.7 mg/g (La), 152.8 mg/g (Ce), 138.2 mg/g (Eu), and 121.2 mg/g (Yb), which can be used as additional benchmarking targets for capacity in REE systems.

### Real coolant testing

Real coolant tests are required because the literature emphasizes that many studies focus on idealized synthetic solutions, and semi-realistic matrices can exhibit reduced sorption efficiency due to competing ions and matrix effects.

Real coolant samples should be pre-characterized for major water-quality parameters and matrix load because high total dissolved solids and suspended solids influence analytical method choice and can require dilution or pre-treatment prior to ICP-MS measurement.

For real-matrix REE testing, one demonstrator approach used a complex multi-element mixture intended to mimic industrial effluents, illustrating the value of including multiple competing metals when validating performance outside idealized solutions.

### Desorption and regeneration

Regeneration should be tested by stripping metals from loaded biomass using acid solutions and evaluating performance over multiple adsorption–desorption cycles, because regeneration is necessary for reuse and enables production of a metal-bearing eluate for downstream recovery.

This protocol’s required regeneration screen is acid stripping over 0.01–1.0 M HCl or HNO3 with a minimum of five cycles, with regeneration efficiency targeted at >80%.

Where rapid desorption kinetics are desired, one reported approach achieved desorption equilibrium in approximately 15–20 minutes using either 0.03 M HCl (pH 1.5) or 0.1 M Na2EDTA (pH 5), enabling relatively short elution steps in cyclic operation.

Where an HNO3-based stripping approach is preferred, another reported desorption procedure treated 0.5 g of loaded biosorbent with 10 mL of 10−1 mol/L HNO3 for 60 minutes at 150 rpm and tested five biosorption/desorption cycles.

### Analytical methods

Analytical method selection should be driven by matrix strength (TDS/suspended solids) and required detection limits, because ICP-MS generally provides much lower detection limits than ICP-OES but has lower tolerance for high-TDS matrices, often requiring dilution or matrix separation.

#### Sample collection and preservation

For dissolved analyte measurements, aqueous samples should be filtered through a 0.45 µm membrane and acidified immediately following filtration to pH < 2 using nitric acid, consistent with EPA dissolved-metals methods.

For ICP-MS workflows, acid-preservation after collection is also standard practice; one application note specifies acidifying all water samples with 2% v/v HNO3 after collection and filtering through 0.45 µm membranes to remove particles.

#### Metals quantification

ICP-OES is appropriate for mg/L-level quantification of metals in aqueous samples after suitable filtration and acid preservation, and EPA Method 200.7 provides method detection limits including Co aqueous MDL of 0.002 mg/L for total recoverable analysis.

ICP-MS is appropriate when sub-µg/L detection is required; EPA Method 200.8 specifies filtration/acidification steps and reports instrument detection limit information for cobalt in its tables (with Co-59 entries).

For very low-level REE detection (e.g., to confirm residuals after high removal), triple-quadrupole ICP-MS methods report sub-ng/L instrumental detection limits; one example reports an Nd instrumental detection limit of 0.0006 µg/L on an iCAP TQe ICP-MS system.

#### Calibration and QC

QC should include spike-recovery tests and internal standards because matrix effects can bias trace REE results; one method added 0.05 µg/L spikes and reported average recoveries between 90% and 112% across samples.

For ICP-MS sample preparation in an acid matrix, one EPA 200.8 application describes preparing calibration standards, internal standards, and samples in 1% HNO3 (with Au(III) chloride included in that method matrix).

### Data analysis

Removal efficiency should be calculated from initial and final concentrations, and uptake capacity should be calculated on a mass-balance basis as q (mg metal per g dry biomass), consistent with standard biosorption reporting and commonly provided formulas in adsorption studies.

Kinetic curves should be fit to pseudo-first-order and pseudo-second-order models because these are widely used in biosorption studies, including those explicitly reporting pseudo-second-order behavior for REE uptake.

Equilibrium data should be fit to Langmuir and Freundlich isotherms to report qmax and affinity constants, consistent with common practice in biosorption modeling and with explicit examples in REE systems.

### Success criteria

Success should be defined using explicit performance thresholds that include metal removal, loading capacity, and regeneration efficiency, and the protocol specifies target criteria of >90% REE removal, >85% Co removal, >50 mg/g loading capacity, and >80% regeneration while meeting discharge constraints such as POTW limits.

Where selectivity is relevant (e.g., separating REEs from non-REE metals), report selectivity factors or separation factors; one biosorption-based separation report observed separation factors of at least 30 for Nd relative to nearly all non-REE metals after a single adsorption/desorption cycle, showing that quantitative selectivity metrics are feasible and informative.

### Safety

Acid handling is integral to this protocol (sample preservation and regeneration), so laboratory safety controls must include appropriate acid-resistant PPE, secondary containment, and procedures for nitric acid use in preservation and in digestion/stripping methods described in analytical protocols.

Because some analytical workflows and sample digestions use strong acids (including concentrated nitric acid for digestion of solids in some protocols), all digestion steps should be executed under appropriate fume extraction and according to the specific method’s safety guidance.

### Timeline and budget

The achievable project cadence is driven primarily by the number of experimental conditions, replicate requirements, and analytical throughput, and modern ICP-MS workflows can reduce total analysis time to under 3 minutes per sample for multi-element panels in some configurations.

For high-throughput screening, ICP-OES and ICP-MS platforms are reported to support very high daily sample volumes (up to ~2500 samples/day for ICP-OES and up to ~1200 samples/day for ICP-MS in one vendor comparison), which can be used to plan sample batching and laboratory capacity needs for the three-phase program.

Budget should be built bottom-up from the number of samples (conditions × timepoints × replicates) and the chosen analytical method(s), because ICP-MS often requires dilution and careful matrix management for high-TDS coolants, while ICP-OES is more robust for high-TDS and suspended-solids matrices.

## Chapter 20

## TAM/SAM/SOM Market Sizing

### Executive summary

The addressable market for wastewater treatment and metal recovery in defense-grade rare-earth magnet machining is best treated as a *subset* of the broader industrial wastewater treatment and on-site treatment markets, because public sources rarely quantify magnet-machining wastewater volumes directly and frequently state that additional analysis is required for military-specific consumption and waste details.

On the “demand pull” side, rare-earth permanent magnets are described as the widest use of rare earth elements (REEs) and are critical to defense systems (e.g., precision-guided munitions). On the “waste supply” side, multiple sources emphasize that swarf is generated when sintered blocks are shaped to customer specifications and that machining is the largest source of material losses, with cutting and grinding scrap rates reported as high as 30–40% for brittle sintered blocks in some contexts.

Because defense programs represent a small share of total magnet demand and because DOD purchases are described as less than 0.1% of global demand, the standalone defense-only wastewater treatment market is structurally niche, but concentrated into a small set of high-value machining and finishing operations that are currently being expanded or onshored via DOD investments (>$439M since 2020).

### TAM

A practical top-down TAM framing starts from the industrial/on-site wastewater treatment markets, which provide an upper bound on spend for equipment and systems that could plausibly include magnet-machining wastewater treatment modules. The industrial wastewater treatment equipment market is valued at USD 12.23B in 2025 and is projected to reach USD 18.70B by 2030 (8.86% CAGR). In parallel, the global on-site wastewater treatment systems market is cited at USD 23.4B in 2025, forecast to USD 59.4B by 2034 (10.9% CAGR).

A second, magnet-specific TAM lens anchors the scale of magnet production and machining waste that could generate metal-bearing coolants, rinse waters, and sludge. Global rare-earth magnet market value is repeatedly cited around USD 19.5B in 2024 in multiple industry summaries, while one industry report states global rare earth magnet demand “approaches 385,000 tonnes annually in 2025,” valued at approximately USD 19B (CAGR 7.8%).

However, global volume estimates vary by source and definition. For example, one recycling/economics paper estimates global NdFeB production at 130,000–150,000 metric tons/year, while other commentary reports China alone producing ~300,000 tonnes of NdFeB magnets in 2024. The key implication for wastewater opportunity is not a single definitive tonnage, but the consistent conclusion that NdFeB dominates rare-earth magnet demand and that downstream machining is a major loss point that produces swarf and related waste streams that are candidates for on-site treatment and recovery.

In public DOE work, the swarf recycling stream in 2020 is estimated as ~9% of NdFeB magnets produced globally, and the same report notes that the largest material losses occur in the machining step (depending on final size/shape). In a separate market report context, cutting and grinding scrap rates for sintered blocks are cited as high as 30–40%, which provides a plausible “high-loss” bound for machining-driven waste generation where tight tolerances, small parts, and fragile geometries dominate.

### SAM

The SAM here is defined as magnet-machining wastewater treatment opportunities in U.S. and allied defense supply chains (U.S./EU/Japan) that are producing or finishing high-reliability SmCo and NdFeB magnets for defense platforms. Public sources emphasize that defense demand is small relative to global demand, including the explicit statement that DOD purchases represent less than 0.1% of global demand for these materials, which limits purely defense-driven volume but does not eliminate the need for dedicated, compliant, traceable manufacturing and waste handling.

Despite the small global share, U.S. defense magnet requirements are described in the thousands of tons per year in multiple sources, providing an order-of-magnitude anchor for potential machining throughput. One source states that “the US military requires about 3,000 tons of permanent magnets a year,” and another states the Pentagon requires 3,000–4,000 tons of highly specialized magnets annually with projected demand up to 10,000 tons by 2030.

In parallel, the U.S. and allied SAM is shaped by “mine-to-magnet” investments aimed at onshoring production and qualifying non-China supply chains. DOD reports that since 2020 it has awarded more than USD 439M to establish domestic rare earth element supply chains and that these awards are intended to develop the domestic market for magnet production to serve defense and commercial markets.

### SOM

The SOM is defined as the realistically serviceable market reachable by deploying at a small number of U.S. defense-relevant magnet production and finishing sites (3–5 facilities) over a five-year period, consistent with the near-term concentration of domestic capacity expansion into a limited set of named projects and established specialty suppliers.

Public sources identify a short list of U.S.-based facilities or facility-scale initiatives that plausibly concentrate swarf generation and coolant-handling needs:

- Noveon Magnetics is stated to have established a rare earth magnet manufacturing facility in San Marcos, Texas (supported by a $28.8M award).
- MP Materials is described as commissioning a magnetics facility in Texas (“Independence”) that is poised to produce ~1,000 metric tons of finished NdFeB magnets per year, with a production ramp beginning in late 2025, and MP Materials also projects its total U.S. magnet manufacturing capacity reaching ~10,000 metric tons after a new facility begins commissioning in 2028.
- E‑VAC Magnetics is described as establishing a commercial-scale rare earth magnet manufacturing capability by 2025 with a $94.1M award, and this effort is repeatedly associated with Sumter, South Carolina in trade reporting, suggesting a key future locus of U.S. magnet production and associated machining waste streams.

Additional defense-relevant machining capacity may sit within established specialty magnet suppliers and integrators. For example, Arnold Magnetic Technologies is described as headquartered in Rochester, New York with manufacturing facilities in multiple countries including U.S. sites, and Electron Energy Corporation (EEC) is described as operating from two facilities in Landisville totaling over 84,800 square feet, both of which are plausible locations for magnet machining and associated coolant/wastewater handling (even if public excerpts do not quantify their wastewater volumes).

### Per-platform consumption

Per-platform metrics are useful for explaining why defense magnets are strategically important even if they are a small fraction of total global demand. Multiple U.S. government and policy/industry sources repeat platform-level rare-earth content examples: an F‑35 requires more than ~900 pounds of rare earth elements, an Arleigh Burke DDG‑51 destroyer requires ~5,200 pounds, and a Virginia-class submarine requires ~9,200 pounds.

Some sources add a more magnet-specific anchor by stating that an F‑35 contains about 50 pounds of samarium-cobalt magnets, which is relevant because SmCo machining waste streams may have different chemistry and value drivers than NdFeB (e.g., cobalt-bearing waste).

These per-platform numbers are not a direct proxy for magnet-machining wastewater volume, but they are a defensible proxy for the strategic importance of ensuring domestic, traceable production and of managing and recovering critical-material-bearing wastes within secure, compliant supply chains.

### US defense facility map

A facility map for the U.S. defense-relevant rare-earth magnet and upstream processing ecosystem can be described as a small number of current producers, plus a larger number of near-term capacity additions supported by DOD funding. As of a 2023 Federal Register summary, there was “currently only one firm in the United States, Noveon (formerly Urban Mining Company), that produces sintered NdFeB magnets, albeit in small quantities,” and multiple additional firms (MP Materials, Quadrant Magnetics, USA Rare Earth, and Vacuumschmelze) planned to establish U.S. NdFeB magnet manufacturing facilities by 2026.

In DOD’s public “mine-to-magnet” disclosures, the department highlights U.S.-based participants and funded efforts including MP Materials, Lynas USA (oxide production capability by 2026), Noveon Magnetics (San Marcos, TX), TDA Magnetics (capability demonstration), and E‑VAC Magnetics (commercial-scale magnet manufacturing by 2025).

Separately, upstream mining/separation constraints shape where compliant magnet-making and recycling can locate. GAO cites USGS estimates that China mined ~240,000 metric tons of rare earths in 2023 versus ~43,000 metric tons for the U.S., and also notes that the U.S. imported more than 95% of the rare earths it consumed, reinforcing the rationale for domestic processing and for recovering metals from domestic waste streams where possible.

### Growth drivers

The strongest macro drivers of rare-earth magnet demand (and therefore machining throughput and waste generation) are consistently identified as electrification and renewables. DOE reports that EV sales increased from 716,000 vehicles in 2015 to 10.6 million in 2022, and offshore wind annual installations grew from 6.2 GW in 2019 to 21.1 GW in 2021, both of which are magnet-intensive technology transitions that support growth in magnet production, finishing, and recycling industries.

On the magnet-market side, DOE’s NdFeB supply-chain report projects rapid total global demand growth for magnets in net-zero scenarios (12.5% average annual growth through 2030 and 6.3% through 2050) and also projects U.S. demand growth (8.7% average annual growth through 2030 and 5.0% through 2050).

Defense is repeatedly treated as a smaller-but-sticky segment: the Federal Register notes defense demand is only a small percentage of total demand while still stating that NdFeB magnets are required for national defense systems, and other market commentary lists “defense and aerospace” among demand drivers alongside electrification, renewables, and automation.

Finally, policy and supply-chain security are explicit growth drivers for domestic magnet production and, indirectly, for domestic waste management and wastewater treatment tied to magnet machining. DOD frames its investments as building capability across multiple stages of the rare earth supply chain and as sending a “clear signal to private capital” to build resiliency, and multiple public summaries emphasize the strategic goal of a mine-to-magnet supply chain capable of supporting U.S. defense requirements by 2027.

### Revenue scenarios

Publicly available sources do not provide a direct, defensible dollar figure for “defense magnet machining wastewater treatment” as a standalone market, and several sources explicitly indicate that additional analysis is necessary for military-specific consumption or for machining waste quantification at the level needed to monetize wastewater solutions.

Therefore, the most supportable way to present conservative/base/aggressive revenue scenarios is as *deployment pathways* rather than as single-number forecasts. These scenarios are grounded in (i) a small number of identifiable near-term U.S. magnet production and finishing sites, (ii) evidence that machining generates swarf and can have high scrap rates, and (iii) strong growth in broader industrial wastewater and on-site treatment markets.

Conservative scenario: focus on one or two early-adopter sites in the U.S. mine-to-magnet ecosystem (e.g., a magnet manufacturer or finisher associated with DOD awards) where swarf generation from shaping blocks to specification is known to occur and where machining losses are described as a dominant source of material loss.

Base scenario: expand to multiple U.S. facilities (3–5) as domestic capacity ramps, including sites that have explicit magnet production scale targets (e.g., ~1,000 t/yr NdFeB at MP Materials’ Independence facility) and sites funded to establish commercial-scale magnet manufacturing capability (e.g., E‑VAC Magnetics).

Aggressive scenario: scale alongside the projected expansion of U.S. rare-earth magnet manufacturing capacity (e.g., MP Materials’ projected ~10,000 metric tons capacity after commissioning of a new facility in 2028) and integrate wastewater treatment with resource recovery and recycling approaches, aligned with broader trends that resource-recovery and reuse solutions are advancing in the wastewater equipment market.

### Adjacent markets

Even if defense demand is structurally small relative to global totals, adjacent commercial markets that share similar machining and coolant/wastewater challenges (and that may share suppliers, machine shops, and recyclers) are large and fast-growing. These include EV traction motors and wind turbines, both of which are repeatedly cited as major drivers of NdFeB demand and large magnet consumers per platform.

For example, DOE reports that each electric vehicle motor typically requires 1–2 kg of permanent magnet material, and wind turbine permanent magnet mass is cited as 2.7–3.2 tonnes per MW of wind turbine capacity, which implies substantial machining throughput in the broader economy as these deployments scale.

These adjacent markets matter for wastewater treatment TAM because they (a) grow the installed base of magnet machining capacity and (b) strengthen the economic case for modular, on-site treatment systems, a category that is described as growing and increasingly important in industrial contexts, including the defense manufacturing segment.

### Assumptions

This market sizing synthesis relies on several explicit assumptions and constraints imposed by the public-source evidence base. First, public sources frequently quantify platform-level rare earth usage but do not provide platform-level magnet masses for most systems, and at least one DOD-adjacent summary explicitly notes that further detailed data may be required for quantities of rare earth magnets per platform.

Second, the literature and policy documents clearly identify swarf generation during shaping and emphasize that machining is a major loss point, but several public-facing supply-chain reports do not provide a single, definitive machining waste fraction for magnet manufacturing, requiring use of ranges from other sources (e.g., scrap rates up to 30–40% in cutting/grinding, swarf streams estimated at 9% of global NdFeB production, and research reporting swarf generation ranging from 6% to 73% depending on magnet shape).

Third, defense share of the overall permanent magnet market is generally described qualitatively as small, and the most concrete public quantification is the statement that DOD purchases represent less than 0.1% of global demand, which constrains how much of the broader wastewater treatment TAM can be credibly attributed to defense magnets alone without site-level procurement and wastewater-flow data.

Finally, while multiple wastewater market reports provide market sizes and growth rates for industrial wastewater treatment and on-site systems, they do not break out magnet machining as a distinct end-use. As a result, any attempt to translate these top-down market sizes into a magnet-machining-specific dollar TAM requires additional bottom-up facility-level data collection (e.g., coolant volumes, discharge limits, hauling costs, and existing treatment configurations), which is not present in the provided public excerpts.