## Plant Capacities

This section details announced and operational NdFeB magnet manufacturing capacity, focusing on new buildouts in the US, EU, and Vietnam.

| Metric | Value | Unit | Year | Geography | Waste Stream | Conditions | Evidence Level |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Production Capacity (MP Materials) | 1,000 | tonnes/year | 2025 | Fort Worth, TX, USA | Not specified | Initial capacity at "Independence" facility; gradual ramp beginning late 2025 | Commercial |
| Production Capacity (MP Materials, 10X Facility) | 10,000 | metric tons/year | 2028 (expected) | Mountain Pass, CA, USA | Not specified | Planned second domestic magnet facility, "10X Facility" | Policy/market estimate |
| Production Capacity (E-VAC Magnetics) | ~1,500 | tonnes/year | 2025 (late fall) | Sumter, SC, USA | Not specified | Sintered NdFeB magnet plant under construction | Commercial |
| Production Capacity (USA Rare Earth) | 1,200 (initial) | tonnes/year | 2025-2026 | Stillwater, OK, USA | Not specified | Ramping to 4,800 tonnes/year by ~2026 | Commercial |
| Production Capacity (Noveon Magnetics) | >2,000 | tons/year | 2020 (ramped up) | San Marcos, TX, USA | Not specified | Operational facility; expanding capacity beyond 2,000 tons/year | Commercial |
| Production Capacity (Neo Performance Materials) | 2,000 | tonnes/year | 2025 | Narva, Estonia | Not specified | Phase 1 capacity; plans to expand to 5,000 tonnes/year | Commercial |
| Production Capacity (HyProMag USA) | 1,552 (total) | metric tons NdFeB/year | 2027-2029 | Dallas-Fort Worth, TX, USA | Not specified | Texas Hub; comprises 941 tpa recycled magnets and 611 tpa co-products | Policy/market estimate |
| Production Capacity (POSCO International JV) | 3,000 (initial) | tons/year | Not specified | USA (TX, TN, or AZ) | Not specified | Planned factory with future scale-up to 5,000 tons/year | Policy/market estimate |
| Production Capacity (SGI Vina) | 4,000-5,000 | tons/year | 2025 | Quang Nam Province, Vietnam | Not specified | Stage I (1,600 tpa) in Q1-24; Stage II (4,000 tpa) by mid-2025 | Commercial |
| Production Capacity (Shin-Etsu Vietnam) | 2,200 | tons/year | 2018 | Hai Phong Province, Vietnam | Not specified | Expansion completed in mid-2018, doubling capacity from 1,100 tpa | Commercial |
| Production Capacity (Daido Steel) | 1,800 | tons/year | 2030 | Gifu Prefecture, Japan | Not specified | Expansion to 150 tons/month (1,800 tpa) planned for fiscal 2030 | Policy/market estimate |

## Waste Chemistry

This section outlines the chemical composition of various NdFeB manufacturing waste streams as reported in academic and technical literature.

| Metric | Value | Unit | Year | Geography | Waste Stream | Conditions | Evidence Level |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Total REE Concentration | up to 30 | wt% | 2024 | Not specified | Machining waste (cutting/grinding) | Sintered Nd-Pr-Fe-B magnet residue | Commercial |
| Nd Concentration | 27.19 | wt% | 2021 | USA | Magnet Swarf | Baseline composition assumed for TEA/LCA model | Lab-scale |
| Pr Concentration | 3.33 | wt% | 2021 | USA | Magnet Swarf | Baseline composition assumed for TEA/LCA model | Lab-scale |
| Dy Concentration | 0.48 | wt% | 2021 | USA | Magnet Swarf | Baseline composition assumed for TEA/LCA model | Lab-scale |
| Fe Concentration | 66.4 | wt% | 2023 | Not specified | Grinding residue | Industrial machine grinding of N38 magnet | Commercial |
| Fe Concentration | 52.4 | wt% | 2023 | Not specified | Diamond cut residue | Laboratory cutting bench sample | Lab-scale |
| Fe Concentration | 67.3135 | wt% | 2016 | China | NdFeB Sludge | Pretreatment powder from industrial sludge (XRF analysis) | Commercial |
| Fe Concentration | 60.3 | wt% | 2018 | Not specified | Sintering waste | Ground manufacturing waste from sintering stage | Commercial |
| Co Concentration | 1.1343 | wt% | 2016 | China | NdFeB Sludge | Pretreatment powder from industrial sludge (XRF analysis) | Commercial |
| Organics Content | 52.5 | wt% | 2014 | Not specified | Swarf Slurry (AMSS) | Stated as "cooling reagent" in the slurry composition | Commercial |
| Nd Concentration (leachate) | 9.01 | g/L | 2019 | Not specified | NdFeB Scrap Leachate | Scrap dissolved in nitric acid before hydrothermal treatment | Lab-scale |
| Fe Concentration (leachate) | 10.53 | g/L | 2019 | Not specified | NdFeB Scrap Leachate | Scrap dissolved in nitric acid before hydrothermal treatment | Lab-scale |
| Nd Concentration (leachate) | 1.63 | g/L | 2020 | India | EoL HDD Magnet Leachate | Leachate from HCl extraction of ground HDD magnet powder | Lab-scale |
| Fe Concentration (leachate) | ~4.5 | g/L | 2020 | India | EoL HDD Magnet Leachate | Calculated from reported 45.76 wt% Fe in solids at 10 g/L pulp density | Lab-scale |

## Compliance Economics

This section provides cost data for industrial wastewater treatment and hazardous waste disposal, which serve as a baseline for the compliance-driven business case.

| Metric | Value | Unit | Year | Geography | Waste stream | Conditions | Evidence Level |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Hazardous Sludge Disposal | 15,000-25,000 | INR/tonne | 2025 | India | ETP Sludge | Disposal at CPCB-approved TSDFs | Commercial |
| Non-Hazardous Sludge Disposal | 3,000-8,000 | INR/tonne | 2025 | India | ETP Sludge | Not specified | Commercial |
| Hazardous Waste Disposal | 310 | USD/ton | 2021 | California, USA | Restricted hazardous waste | Average disposal fee | Commercial |
| Hazardous Waste Incineration | 529 | USD/ton | 1998 | USA | General hazardous waste | Average cost from USACE survey of RCRA-permitted facilities | Commercial |
| Wastewater Treatment OPEX | 3.09 | USD/m³ | 2020 | Turkey | Metalworking wastewater | Bench-scale continuous electrocoagulation process | Lab-scale |
| Hazardous Liquid Disposal | 1.84-3.11 | USD/gallon | 1998 | USA | Hazardous wastewater | Cost for discharge to POTW/TSD, adjusted to 2000 USD | Policy/market estimate |
| Permit Acquisition Cost | 250,000-500,000 | USD | 1996 | USA | Hazardous waste incinerator | Estimated cost to obtain a RCRA or BIF permit | Policy/market estimate |
| Compliance Testing Cost | ~300,000 | USD | 1996 | USA | Hazardous waste incinerator | Cost per trial burn required for permit, based on vendor quotes | Commercial |
| Annual Compliance Overhead | 50,000-100,000 | INR/year | 2025 | India | ETP Sludge | Documentation & compliance for medium-sized industrial units | Commercial |

## REE Pricing

This section summarizes recent and historical price data for key rare earth elements, highlighting volatility. This value is considered secondary upside.

| Metric | Value | Unit | Year | Geography | Waste Stream | Conditions | Evidence Level |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Nd Oxide Price | 49,763 | USD/metric ton | 2020 | Global | Not specified | Historical average price | Policy/market estimate |
| Nd Oxide Price (Forecast) | 77,500 | USD/metric ton | 2025 | Global | Not specified | Statista forecast | Policy/market estimate |
| NdPr Oxide Price (Forecast) | 90,000 | USD/tonne | 2026 | Global | Not specified | BMI average yearly price forecast | Policy/market estimate |
| Dy Price | 930.70 | USD/kg | 2026 | Not specified | Not specified | Retail price for private investors, last updated Feb 06, 2026 | Market Estimate |
| Dy Oxide Price | 263.34 | USD/kg | 2026 | Northeast Asia | Not specified | Market price for January 2026 | Market Estimate |
| Dy Price Volatility | +105.05 | % | 2026 | Not specified | Not specified | Year-to-date price change as of Feb 06, 2026 | Market Estimate |
| Nd Price | 194.40 | USD/kg | 2026 | Not specified | Not specified | Retail price for private investors, last updated Feb 06, 2026 | Market Estimate |
| Nd Price Volatility | +199.08 | % | 2020-2026 | Not specified | Not specified | Price change since January 2020 | Market Estimate |
| NdPr Price (Historical) | 360,500 | CNY/Tonne | 2020 | China | Not specified | Historical spot price on 2020-01-02 | Market Estimate |
| NdPr Price (Historical Peak) | 1,520,000 | CNY/Tonne | 2022 | China | Not specified | All-time high price in February 2022 | Market Estimate |