# PART II: ECONOMICS, COMPLIANCE TIMELINE, AND BUSINESS CASE

## Chapter 4

### Recoverable Rare Earth Value from Defense Grade Magnet Machining Swarf

### 1. Input assumptions

This economic model uses two reference machining-swarf compositions as representative feedstocks for high-performance permanent magnets (NdFeB and SmCo) and treats them as “typical” rather than universal, because actual chemistry varies by grade and program requirements.

The reference compositions applied throughout the calculations are:

- NdFeB swarf: ~25 wt% Nd, ~5 wt% Pr, ~3 wt% Dy, ~65 wt% Fe, ~1 wt% B.

- SmCo (Sm2Co17) swarf: ~23 wt% Sm, ~50 wt% Co, ~15 wt% Fe, ~7 wt% Cu, ~2 wt% Zr.

Because the value model is intended to be relevant to “high-temperature”/high-coercivity magnet grades, the dysprosium content is treated as a key sensitivity variable. One source notes that NdFeB magnets for high-temperature applications can include up to 11 wt% Dy (as an alloying addition) to maintain coercivity at elevated temperature, implying that Dy content can materially exceed the 3 wt% reference value in some high-temperature magnets.

### 2. Commodity price inputs

The table below summarizes price points used as inputs, emphasizing 2024–2025 where available and retaining some well-cited early-2026 spot assessments where they provide missing coverage for specific oxides (e.g., terbium) or regional benchmarks. NdPr mixed-oxide prices are included because NdPr oxide is a common traded/intermediate form and can be a useful proxy when individual oxide splitting is not modeled.

| Material | Price point used (USD/kg) | Date and basis | Source |
| --- | --- | --- | --- |
| Neodymium oxide (Nd oxide, 99.5–99.9% FOB) | 55.27–56.27 | May 2024 average, FOB | Argus sample report |
| Neodymium oxide (Nd oxide, 99.5–99.9% FOB) | 55.77 | Argus index table value, FOB China | Argus sample report |
| Praseodymium oxide (Pr oxide, 99.5–99.9% FOB) | 55.36–56.36 | May 2024 average, FOB | Argus sample report |
| Praseodymium oxide (Pr oxide, 99.5–99.9% FOB) | 54.58 | Argus index table value, FOB China | Argus sample report |
| NdPr oxide (99% NdPr oxide, ex-works) | 50.94–51.35 | June 4, 2024, ex-works (yuan/kg converted in-report) | Argus sample report |
| Dysprosium oxide (Dy oxide, 99.5% FOB export) | 265–270 | June 2024 export assessment, FOB China | Argus sample report |
| Terbium oxide (Tb oxide, 99.99% FOB export) | 815–835 | June 2024 export assessment, FOB | Argus sample report |
| Samarium oxide (Sm oxide, 99.5% FOB) | 2.01 | Argus index table value, FOB China | Argus sample report |
| Samarium metal (Sm metal, min 99% FOB) | 12.25 | Argus index table value, FOB China | Argus sample report |
| Cobalt metal (U.S. spot) | 33.83 | January 2025 U.S. spot price | USGS monthly commodity summary |
| Cobalt metal (LME average) | 23.63 | January 2025 LME average price | USGS monthly commodity summary |

### 3. NdFeB swarf value model

#### 3.1 Approach

Two complementary bottom-up valuation approaches are presented:

- Element-to-oxide price approach that values recovered Nd oxide, Pr oxide, and Dy oxide using 2024 oxide price points and the reference swarf metal composition (Nd/Pr/Dy wt%).

- Process-yield approach that uses a published techno-economic model of swarf-to-mixed-REO production capacity (100 t swarf/year → 32 t REO/year), coupled with reported REO composition ranges and mixed-REO selling-price assumptions. This pathway avoids the need to model explicit metal-to-oxide stoichiometric conversions in the prose model and stays closer to documented industrial mass-balance assumptions.

Both approaches are aligned with literature indicating that high REE recovery is achievable. For example, an acid-free dissolution route reported ~97% REE recovery and >99.5% purity products, with scenario leached efficiencies of 96–99%.

#### 3.2 Element-to-oxide price approach

Using the reference NdFeB swarf composition (per 1 tonne of swarf: 250 kg Nd, 50 kg Pr, 30 kg Dy) and treating the recoverable value as dominated by Nd oxide, Pr oxide, and Dy oxide, the gross value is computed using a representative recovery yield of ~97% (consistent with published REE recovery levels).

- Nd value (oxide-priced): 250 kg Nd × 97% recovery × $55.27–56.27/kg Nd oxide ≈ $13,400–$13,650 per tonne swarf (oxide price range applied).

- Pr value (oxide-priced): 50 kg Pr × 97% recovery × $55.36–56.36/kg Pr oxide ≈ $2,685–$2,733 per tonne swarf.

- Dy value (oxide-priced): 30 kg Dy × 97% recovery × $265–270/kg Dy oxide ≈ $7,711–$7,857 per tonne swarf.

Summing only these oxide-priced REE components yields an indicative gross recoverable value of roughly $23,800–$24,200 per tonne of NdFeB swarf under the reference 3 wt% Dy composition and 2024 oxide price points.

This result is directionally consistent with published observations that dysprosium can be a major revenue driver in high-Dy feedstocks, because Dy oxide is priced at a materially higher level than Nd oxide in the oxide market data shown above.

#### 3.3 Process-yield approach using mixed REO output

A published TEA-scale mass balance modeled an industrial operation processing 100 metric tons/year of NdFeB magnet swarf to produce 32 metric tons/year of REOs, implying a 0.32 tonne REO output per tonne of swarf input (i.e., 320 kg mixed REO per tonne swarf).

That same analysis reported that the mixed REO output composition contains 73–89% Nd2O3, 9–11% Pr2O3, and 0–18% Dy2O3 (mixed together).

Applying those fractions to 320 kg mixed REO per tonne swarf gives oxide-mass ranges per tonne swarf:

- Nd oxide in mixed REO: 320 kg × 73–89% = 234–285 kg Nd oxide per tonne swarf.

- Pr oxide in mixed REO: 320 kg × 9–11% = 29–35 kg Pr oxide per tonne swarf.

- Dy oxide in mixed REO: 320 kg × 0–18% = 0–58 kg Dy oxide per tonne swarf.

Valuing these oxides using 2024 Argus price points yields a gross-oxide-value envelope (still excluding processing costs):

- Nd oxide value: (234–285 kg) × $55.27–56.27/kg ≈ $12,900–$16,000 per tonne swarf.

- Pr oxide value: (29–35 kg) × $55.36–56.36/kg ≈ $1,606–$1,973 per tonne swarf.

- Dy oxide value: (0–58 kg) × $265–270/kg ≈ $0–$15,660 per tonne swarf (Dy-rich case).

Under this process-yield framing, the Dy fraction dominates the upside, which matches the TEA statement that in a best-case scenario (6% Dy in swarf), dysprosium oxide can represent the largest share of total economic value (52%).

### 4. SmCo swarf value model

#### 4.1 Approach

For SmCo swarf, the dataset contains (i) a reference swarf composition and (ii) at least one Sm recovery-rate datapoint from SmCo5 scrap. The model therefore focuses on cobalt metal value (dominant by mass and price) and a conservative valuation of samarium using a directly cited samarium metal price point to avoid requiring explicit Sm-to-Sm2O3 stoichiometric conversion in the prose model.

#### 4.2 Gross recoverable value per tonne

Per 1 tonne SmCo swarf (reference): 230 kg Sm metal and 500 kg Co metal are present by mass fraction.

For samarium, one SmCo5 scrap study reported obtaining samarium sulfate hydrate (96.5% purity) with a samarium recovery rate of 87.1%; this is used as an indicative recovery yield for Sm-bearing scrap, recognizing that Sm2Co17 may behave differently in practice.

For cobalt, the dataset does not provide a SmCo-specific cobalt recovery yield, so a conservative “high-yield” assumption consistent with published magnet-scrap processing yield ranges is used. One industry/market report describes hydrogen processing of magnet scrap (HPMS) as enabling alloy recovery with 90–98% yield; the lower bound (90%) is applied as a conservative proxy recovery for cobalt-bearing fractions in a high-yield recycling route.

Applying these yields and 2025 cobalt prices gives:

- Sm value (metal-priced): 230 kg Sm × 87.1% × $12.25/kg Sm metal ≈ $2,456 per tonne swarf.

- Co value (metal-priced): 500 kg Co × 90% × $23.63–33.83/kg Co ≈ $10,634–$15,224 per tonne swarf (LME avg vs U.S. spot, Jan 2025).

Under these inputs, the gross recoverable value for SmCo swarf (Sm+Co only, excluding Fe/Cu/Zr credits) is approximately $13,100–$17,700 per tonne depending primarily on cobalt price basis and achieved recovery yields.

### 5. Processing cost estimates

Processing cost estimates in the dataset span a wide range because they reflect different system boundaries (swarf-to-oxide, whole-magnet recycling, and different process chemistries). A TEA of an acid-free dissolution route reported total cost of ~$11 per kg of swarf processed (and ~$29–$36 per kg of mixed REO produced) at a modeled scale, which implies **~$11,000 per tonne swarf** processed as a central cost benchmark for swarf-focused hydrometallurgical processing.

A separate industry article gives a much higher, less specific estimate that recycling 1 ton of NdFeB magnets costs ~$50–$70 per kilogram, which would imply $50,000–$70,000 per tonne; the same article also reports hydrometallurgy can achieve recovery rates up to ~95% and that pyrometallurgy can yield ~70–85% recovery, illustrating that cost and yield vary materially by route and process design.

From a wastewater-generation standpoint (relevant for downstream handling cost and treatment requirements), the same article states that hydrometallurgy can produce 10–15 tons of wastewater per ton of rare earth recovered.

### 6. Net recoverable value

Net value is presented as gross recoverable value minus processing cost, using the TEA-derived $11,000/tonne swarf processing-cost benchmark as the primary reference case.

#### 6.1 NdFeB swarf net value

Using the element-to-oxide price approach gross value of ~$23,800–$24,200 per tonne (reference 3 wt% Dy case), subtracting ~$11,000/tonne processing cost yields a net value of approximately $12,800–$13,200 per tonne swarf in the TEA-like cost regime.

Under the mixed-REO process-yield envelope, gross oxide value spans widely depending on Dy content (because Dy oxide fraction can range from 0% to 18% of the mixed REO output). This implies net value can range from strongly positive to marginal depending on whether the feedstock is Dy-lean or Dy-rich, consistent with TEA statements that Dy content materially shifts the economic value distribution and can dominate value in best-case scenarios.

#### 6.2 SmCo swarf net value

For SmCo swarf, using the gross Sm+Co value range of ~$13,100–$17,700 per tonne and applying the same $11,000/tonne swarf processing-cost benchmark as a proxy yields an indicative net of approximately $2,100–$6,700 per tonne (highly sensitive to cobalt price basis and achieved recovery).

#### 6.3 Comparison to primary ore concentration

Recycling economics are frequently framed around the much higher “grade” of secondary feedstocks compared with primary ores. One article states that mining one tonne of rare earth ore typically yields only 3–5 kilograms of usable material, highlighting why high-grade scrap can be attractive even when processing costs are substantial.

In contrast, rare-earth magnet scrap streams can contain on the order of tens of percent rare earth content, such as the reference NdFeB swarf composition above with substantial Nd/Pr/Dy fractions by mass, and multiple sources report NdFeB magnets can contain “up to ~30%” critical rare earth elements in the magnet alloy.

### 7. Volume context

The potential annual swarf feedstock volume depends on magnet production volumes and the fraction lost as machining and process scrap. One industry-facing source states that Nd-Fe-B magnet production generates 20–30% scrap, while Sm-Co production generates 15–30% waste as sludge and scrap, implying a structurally significant secondary stream relative to finished magnet output.

A TEA-scale analysis estimated 130,000–150,000 metric tons of NdFeB magnets are produced per year globally. Applying the 20–30% production scrap fraction as an indicative range suggests that the global NdFeB manufacturing system could generate on the order of tens of thousands of tonnes of scrap/swarf annually (with exact mass depending on whether the scrap fraction is referenced to input alloy or output magnets).

### 8. Strategic value premium

Beyond commodity value, defense-relevant magnet scrap can carry an additional “strategic” premium when it supports domestic, traceable supply chains and reduces dependence on concentrated upstream refining. One market source states that China controls ~70% of global rare earth oxide refining capacity, which increases geopolitical and supply-chain risk premiums for non-Chinese supply pathways, including recycling.

Downstream demand-side premiums for recycled-content magnets can also elevate realized value. A market report notes that OEMs may pay 10–20% premiums for magnets verified as recycled content, creating a potential margin uplift for compliant recyclers relative to commodity-only valuation approaches.

Policy-linked price support mechanisms can further raise the effective value of domestically produced rare-earth material streams. One recycling-focused article claims the U.S. Department of Defense has guaranteed minimum prices of $110/kg to incentivize domestic production, indicating that certain strategic programs may value domestic supply above spot-market economics for some periods and materials.

## Chapter 5

### DFARS Rare Earth Magnet Compliance Timeline Roadmap for Defense Primes 2024 2028

### Executive summary

January 1, 2027 is the inflection point when DFARS restrictions for samarium-cobalt (SmCo) and neodymium-iron-boron (NdFeB) magnets expand upstream to cover the full supply chain (mined, refined, separated, melted, or produced) in covered countries, not just melting/production stages. Because qualification, re-sourcing, and documentation updates typically take multiple contracting cycles, multiple industry sources urge primes to secure DFARS-compliant magnet supply chains in advance of the statutory deadline, with some suppliers publicly targeting “mid-2026” readiness as a practical cutover point.

The May 30, 2024 final rule is already effective and establishes the staged requirements and the governing contract clause (DFARS 252.225-7052). Between now and the end of 2026, the restriction is narrower (focused on material melted/produced in covered countries), but it still requires primes to ensure traceability through alloying/melting and subsequent magnet manufacturing steps (powder formation, pressing, sintering/bonding, magnetization) for SmCo and NdFeB magnets.

Domestic and allied capacity is expanding, but the “mine-to-magnet” buildout is still in progress and includes schedule and permitting risks; DoD itself has stated that establishing new domestic rare earth sources can take “a decade or longer.” The roadmap below therefore emphasizes (1) near-term supply chain auditing and provenance tracking, (2) accelerated second-source qualification, (3) use of the recycled NdFeB exception where applicable (U.S. milling + U.S. sintering), and (4) early preparation of nonavailability-determination packages when compliant supply cannot be ensured by late 2026.

### Timeline

The timeline below consolidates binding regulatory milestones, expected provenance disclosure implementation windows, and major domestic capacity events relevant to magnet sourcing decisions for defense primes.

#### 2024

On May 30, 2024, DoD issued and made effective a final rule that amended DFARS 225.7018 and the associated clause DFARS 252.225-7052 (Restriction on the Acquisition of Certain Magnets, Tantalum, and Tungsten). The final rule implements a staged restriction: through December 31, 2026, contractors may not deliver covered material “melted or produced” in covered countries; beginning January 1, 2027 the restriction expands to materials “mined, refined, separated, melted, or produced” in covered countries (the expansion date was extended to January 1, 2027 by NDAA FY2024 Section 854).

GAO reports that DoD completed rulemaking for the revised prohibition in May 2024, but that separate provenance/provenance-disclosure rulemaking for permanent magnets was still developing, with a DoD expectation (as of the GAO report) to publish a proposed DFARS rule for public comment in October 2024.

#### 2025

Regulatory implementation expectations begin to tighten in 2025. GAO describes procurement regulation implementation (including magnet source disclosure requirements) as expected in the 2025–2027 window, and notes that the statutory effective date for the new disclosure requirement is “not earlier than June 2025.” GAO further notes that if a contractor cannot make the required disclosure, it will be required to establish a supply chain tracking system to make the disclosure “to the fullest extent possible.”

In parallel, domestic magnet capacity milestones reported in public sources indicate that new U.S. production is starting to come online. MP Materials reported (Jan 22, 2025) that commercial NdPr metal production was already online, trial production of sintered NdFeB magnets was underway, first deliveries were on track for year-end, and the facility targeted ~1,000 metric tons/year of finished magnets with a gradual ramp beginning in late 2025.

For additional U.S. capacity, DoD’s ManTech reporting indicates that (via MCEIP awards) E-VAC Magnetics is expected to establish commercial-scale rare-earth magnet manufacturing capability by 2025 (award value $94.1 million), and that Noveon Magnetics has established a rare earth magnet manufacturing facility in San Marcos, Texas (award value $28.8 million).

#### 2026

2026 is the “execution year” for supplier transitions under the DFARS staged approach, with multiple sources recommending completion of alternative sourcing and qualification actions before the end of 2026 to avoid a delivery cliff in 2027.

Several supplier-side milestones cluster around 2026. Arnold Magnetic Technologies states that DFARS updates take effect January 1, 2027 and that it aims to achieve compliance by mid-2026. Dexter Magnetic Technologies, Magnetic Component Engineering, and Electron Energy Corporation likewise describe a joint effort to achieve DFARS 252.225-7052 compliance for NdFeB and SmCo magnets by mid-2026, ahead of the January 1, 2027 deadline, including work with compliant global suppliers to source rare earth materials.

On the production side, eVAC Magnetics reports that its Sumter, South Carolina facility opened in fall 2025 and will “quickly scale” to 2,000 metric tons/year by the first quarter of 2026, with stated ability to scale up further in later years; the same statement also notes that the Sumter facility sources NdPr feedstock from MP Materials’ Mountain Pass mine through a strategic partnership. Separately, MP Materials’ reporting indicates commissioning of a heavy rare earth separation capability targeted for mid-2026, with a Dy/Tb circuit nameplate capacity of 200 MT/year (within a facility designed to process ~3,000 MT/year of feedstock) intended to support planned production of 10,000 MT/year of high-performance NdFeB magnets.

#### 2027

January 1, 2027 is the binding compliance deadline for the expanded prohibition: contractors “shall not deliver” covered material mined, refined, separated, melted, or produced in any covered country, or end items manufactured in any covered country that contain a covered material. For SmCo magnets, DFARS explicitly states that, effective January 1, 2027, the restriction includes the entire supply chain from mining/production of cobalt and samarium ore or feedstock (including recycled material) through production of finished magnets. For NdFeB magnets, DFARS explicitly states that, effective January 1, 2027, the restriction includes the entire supply chain from mining of neodymium, iron, and boron through production of finished magnets.

The expanded rule also changes how the COTS exception works. Legal commentary notes that, effective January 1, 2027, COTS items will be exempt only if they are less than 50% covered material by weight (not just tungsten), tightening practical compliance requirements for magnet-containing COTS subsystems and assemblies.

Because compliant supply may not exist in sufficient quality/quantity for specific applications, DFARS provides an exception mechanism via nonavailability determinations. DFARS and its implementing materials describe both individual and class nonavailability determinations under DFARS 225.7018-4 when compliant materials are unavailable. For class determinations, USD(A&S) must issue a public notice of intent at least 30 days prior to issuing the determination.

Some domestic production expansions are also described as initiating volumes in 2027. MP Materials’ November 2025 presentation describes an expansion of Independence with “initial volumes starting in 2027.”

#### 2028

A key late-decade capacity event is MP Materials’ planned second domestic magnet manufacturing facility (“10X Facility”), expected to begin commissioning in 2028; MP states that once the new facility is completed and commissioning begins, its total U.S. rare earth magnet manufacturing capacity will reach an estimated 10,000 metric tons. MP further states that, for 10 years following the construction of the 10X Facility, DoD has agreed to ensure that 100% of the magnets produced at the 10X Facility will be purchased by defense and commercial customers, and that DoD has entered into a 10-year price floor commitment of $110/kg for MP’s NdPr products stockpiled or sold.

However, some non-U.S. but non-China supply chain options may have timelines extending beyond 2027. One analysis of Lynas’ Texas refinery project outlines commissioning beginning in Q4 2027 and commercial production in Q2 2028, with a target to produce 5,000 metric tons/year of magnet-grade NdPr, underscoring that some diversified upstream capacity may arrive after the DFARS 2027 compliance deadline.

### Regulatory milestones

The table below summarizes the “what changes when” elements that matter most for magnet procurement and supplier qualification.

| Date or window | What changes | Practical implication for primes |
| --- | --- | --- |
| May 30, 2024 | Final rule issued and effective; DFARS clause 252.225-7052 updated. | Treat as “go-live” for contract language updates and internal compliance program mobilization (policies, supplier questionnaires, traceability expectations). |
| Through Dec 31, 2026 | Prohibition focuses on covered material “melted or produced” in covered countries for deliveries; for SmCo/NdFeB, includes melting and all subsequent phases (powder formation, pressing, sintering/bonding, magnetization). | Immediate focus on traceability through alloying/melting and the magnet-manufacturing chain, even before 2027 upstream expansion. |
| Not earlier than June 2025 to 2027 | Magnet source disclosure requirements expected to start taking effect between 2025 and 2027; if contractor cannot disclose, it must establish a supply chain tracking system to disclose to fullest extent possible. | Build data systems and supplier flowdown now; don’t assume a manual, late-stage affidavit will be sufficient if disclosures cannot be made. |
| Jan 1, 2027 | Expanded prohibition covers “mined, refined, separated, melted, or produced” in covered countries; SmCo and NdFeB explicitly defined as covered materials and covered supply chains (SmCo includes recycled feedstock). | Must have compliant sources (or approved nonavailability determination) for upstream feedstock/provenance, not just for final magnet manufacturing location. |
| Jan 1, 2027 | COTS exemption narrowed; COTS items exempt only if <50% covered material by weight. | Magnet-heavy COTS subsystems may become non-exempt and require the same provenance diligence as non-COTS assemblies. |
| Ongoing | Class nonavailability determinations require USD(A&S) approval and a public notice at least 30 days prior; individual determinations can be made by the head of the contracting activity for one contract. | If you anticipate a waiver need for 2027 deliveries, plan backward from the 30-day notice requirement and the time required to assemble market research and supporting analysis. |

### Domestic capacity coming online

The capacity timeline below is organized by facility and limited to milestones explicitly reported in the provided sources.

#### MP Materials

MP Materials’ magnetics plans provide a staged domestic capability picture. MP’s Jan 22, 2025 release indicates trial production of automotive-grade sintered NdFeB magnets, first deliveries targeted for year-end, and a gradual production ramp beginning in late 2025 toward ~1,000 metric tons/year of finished magnets. MP’s filings also state the company anticipates manufacturing NdFeB permanent magnets by the end of 2025 and that Independence is designed for ~1,000 MT/year of finished magnets at initial design capacity.

For heavy rare earths (relevant to high-coercivity/high-temperature NdFeB grades that rely on Dy/Tb), MP reports commissioning a heavy rare earth separation facility at Mountain Pass in mid-2026 and describes a Dy/Tb circuit nameplate capacity of 200 MT/year within a facility designed to process ~3,000 MT/year of feedstock, intended to support planned production of 10,000 MT/year of high-performance NdFeB magnets.

For larger-scale domestic magnet capacity, MP states it will construct a second U.S. magnet facility (“10X Facility”) expected to begin commissioning in 2028, bringing total U.S. magnet manufacturing capacity to an estimated 10,000 metric tons once completed. MP further states DoD has a 10-year price floor commitment of $110/kg for NdPr products and a 10-year commitment ensuring 100% of magnets produced at the 10X Facility are purchased by defense and commercial customers.

#### VAC and eVAC Magnetics

eVAC reports that its Sumter, South Carolina magnet manufacturing facility opened in fall 2025 and that it will quickly scale to 2,000 metric tons/year by the first quarter of 2026, with an ability to scale to higher levels in subsequent years. The same announcement states that eVAC’s South Carolina facility sources key raw materials via a strategic partnership with MP Materials, which supplies NdPr feedstock from Mountain Pass, California.

#### Noveon Magnetics

DoD ManTech reporting indicates Noveon Magnetics has established a rare earth magnet manufacturing facility in San Marcos, Texas, supported by a $28.8 million MCEIP award, but does not provide a commissioning date or capacity in the cited excerpt.

#### Lynas USA

DoD ManTech reporting indicates Lynas USA received a combined $288 million in MCEIP funding to establish a second domestic, commercial-scale oxide production capability by 2026. Other analyses describe Lynas’ Texas project timelines extending into 2027–2028 for commissioning and commercial production, reflecting that some upstream processing capacity may not align with the 2027 compliance deadline for magnet deliveries.

### Compliance pathway decision matrix

The options below are not mutually exclusive; primes typically pursue multiple pathways in parallel to reduce schedule and certification risk.

| Pathway | When it works best | What must be true by late 2026 | Key actions and evidence packets |
| --- | --- | --- | --- |
| Transition to compliant (non-covered-country) primary supply | Programs with sufficient lead time to qualify new magnet sources before Jan 1, 2027. | Supply chain provenance for SmCo and NdFeB magnets must avoid covered-country mining/refining/separation/melting/production for 2027 deliveries. | Review supply chains and identify alternative sources; suppliers and advisors explicitly recommend supply chain review and alternative sourcing in 2024–2026. |
| Use the recycled NdFeB exception | NdFeB magnets and end items containing NdFeB magnets where recycled feedstock is available and processing can be done domestically. | Milling of recycled material and sintering of the final NdFeB magnet must take place in the United States to qualify for the exception. | Designate recycled feedstock pathways; work with procuring activity on demonstrating compliance to exceptions. |
| Seek nonavailability determination | Critical applications where compliant sources of satisfactory quality/quantity “in the required form” cannot be procured when needed at a reasonable price. | Government decisionmakers must have sufficient market research and analysis to support individual or class determinations; class determinations require public notice at least 30 days prior. | Compile Part A data on alternative sources and detailed market research; template-based packages are explicitly contemplated in PGI 225.7018-4. |

### Action items

The action plan below is organized as a time-phased set of tasks aligned to the DFARS dates and reported implementation windows.

#### 2024 actions

Defense primes should treat the May 30, 2024 final rule as the trigger to mobilize a program-level compliance roadmap, since it establishes the staged restriction and amends DFARS 252.225-7052 in contract language. Legal and policy commentary recommends immediate awareness and preparation steps in 2024, followed by supply chain review in 2025 and supplier transition in 2026.

#### 2025 actions

Primes should begin structured supply chain reviews (down to covered material provenance) and implement tracking systems capable of supporting future source-disclosure requirements, since GAO indicates disclosure implementation is expected between 2025 and 2027 and states that contractors unable to disclose must establish a supply chain tracking system to disclose “to the fullest extent possible.” In the same period, primes should proactively qualify emerging domestic sources as they begin trial production and early shipments, such as MP’s Independence facility ramp beginning in late 2025 and first deliveries targeted by year-end 2025.

#### 2026 actions

By 2026, primes should be executing supplier transitions and qualification campaigns, because the restriction expands upstream on January 1, 2027 and will apply to the entire supply chain for SmCo and NdFeB magnets. Industry advisors explicitly recommend identifying and securing alternative sources in 2026 due to the expanded scope and reduced applicability of the COTS exemption.

Primes should also prepare “waiver-ready” nonavailability determination packages if compliant sources cannot be secured for 2027 deliveries. Multiple sources recommend aggregating data to support nonavailability determinations if noncompliance is anticipated by January 1, 2027, and DFARS/PGI materials specify that class determinations require at least 30 days of public notice and that determination packages require detailed market research and alternative-source analysis.

#### Q4 2026 checkpoint

By the end of 2026, primes should have either completed a transition to compliant sources or be in an advanced stage of a government-approved nonavailability determination path, because the contract-delivery prohibition expands on January 1, 2027 and applies directly to contractor deliveries under covered contracts.

#### 2027 actions

Starting January 1, 2027, primes must ensure magnet-containing deliverables do not contain covered material mined/refined/separated/melted/produced in covered countries (unless an exception applies). For SmCo, the explicit inclusion of “recycled material” within the covered supply chain means primes should not assume that recycled SmCo feedstock is exempt in the way recycled NdFeB can be exempt (under the narrow U.S. milling + sintering condition).

#### 2028 actions

In 2028, primes should reassess their long-term sourcing position in light of new domestic capacity expected to begin commissioning (e.g., MP’s 10X Facility), and consider whether long-term offtake/price-stability arrangements can reduce program risk as demand grows and DFARS compliance becomes a baseline requirement for many magnet-containing systems.

### Risk assessment

The largest compliance risk is a schedule mismatch between (a) the hard DFARS January 1, 2027 expanded prohibition for magnet supply chains and (b) the time required to build and qualify compliant upstream and downstream capacity. DFARS rulemaking itself can take “a year or longer,” and DoD assessed that it can take “a decade or longer” to establish new domestic sources for rare earths, which makes reliance on late-breaking domestic capacity a high-risk strategy for 2027 program deliveries.

A second risk is that supply chain disclosure/provenance requirements may tighten during 2025–2027, and GAO signals that contractors unable to disclose sources will need supply chain tracking systems to comply “to the fullest extent possible.” This creates an operational risk where programs may be blocked not only by material provenance, but also by incomplete documentation systems and weak subcontract flowdown practices.

A third risk is the reduced applicability of COTS-based compliance shortcuts. Effective January 1, 2027, legal commentary notes the COTS exemption is narrowed to items containing less than 50% covered material by weight, which can pull previously “commercial” subassemblies into the scope of origin/provenance diligence if magnets are a substantial fraction of mass (e.g., dense motor components).

Finally, primes should plan for the contingency that nonavailability determinations may be necessary for specific applications. The DFARS framework allows such determinations when compliant covered materials of satisfactory quality/quantity in the required form cannot be procured at a reasonable price, but class determinations require public notice at least 30 days prior and require extensive supporting documentation and market research.

## Chapter 6

### Wastewater Treatment Chemistry for Rare Earth Contaminated Machining Coolant Streams

### Summary

Rare-earth-element-bearing wastewaters are commonly treated with a combination of physicochemical separation, pH-driven precipitation, and (where needed) membrane and sorption polishing to reduce toxicity and enable reuse or compliant discharge. Treatment performance is highly sensitive to solution chemistry, especially pH, oxidation state (e.g., Fe(II) vs Fe(III)), co-ions, ionic strength, initial target-element concentration, and the presence of organic substances that can interfere with precipitation and separations. For machining coolant and metalworking-fluid streams specifically, the presence of oils/organics and surfactants creates fouling and process-integration challenges that typically require dedicated pre-treatment before membranes or adsorption/ion-exchange recovery steps are viable at scale.

### Stream characterization

Machining coolants (especially water-mix cutting fluids) are commonly prepared as oil-in-water emulsions, with concentrates diluted to form emulsions on the order of 2–10% (v/v) oil in water, which strongly shapes separation chemistry and downstream membrane fouling behavior. The literature explicitly treats machining coolant as a high-volume wastewater category where precipitation is attractive because it can be cost-effective for large volumes, including high-salinity streams such as machining coolant, even when more selective methods would be too costly for bulk treatment. Work focused on rare-earth recovery and removal also notes that real industrial waste streams (including machining slurries/coolants) are more complex than synthetic solutions because cutting fluids and organic contaminants introduce matrix effects that complicate REE recovery and can require pre-treatment or pre-concentration.

Where the objective is coolant reuse rather than discharge, process performance is often evaluated on suspended solids, organics, surfactants, and phosphorus metrics that correlate with coolant stability and downstream equipment reliability (e.g., total suspended solids removal, COD/TOC reduction). One study of grinding-fluid recycling used 1000 L baths filled with aqueous grinding fluids (e.g., 2–5% KFL plus 1–2% FC 430 for Zn–Al parts, or 2–5% ZF 322 for steel parts) and noted that such fluids are periodically cleaned using coagulant addition and centrifugation in current practice, illustrating the combination of oil/surfactant and fine-solid contamination typical of industrial coolant loops.

### Chemical precipitation

#### Hydroxide precipitation windows

Across multiple sources, REE hydroxide precipitation is consistently described as a high-pH process in which REEs precipitate as hydroxides when pH is raised into the near-neutral to alkaline range (often cited roughly pH 8–10, depending on matrix and concentration). Experimental pH-onset ranges for hydroxide precipitation differ by element; one reported set of tested REEs begins to precipitate as hydroxides at high pH values including La (pH 7.82–8.10), Ce (pH 7.60–7.80), Pr (pH 7.35–7.89), Eu (pH 6.82–7.30), Tb (~7.20), and Y (pH 6.75–6.83). Consistent with this, Pourbaix-based reasoning in precipitation-focused work also notes that REEs tend to form rare-earth hydroxides in slightly alkaline solutions (e.g., pH > 7.5).

#### Staged pH control for Fe and Al removal

Because iron and aluminum often precipitate at lower pH than REEs, staged neutralization is a common selectivity strategy in REE-bearing aqueous systems, where Fe is removed first at mildly acidic pH before raising pH for REE hydroxides. Practical guidance and review-level summaries describe Fe precipitation occurring first (often around pH ~3–4) prior to REE precipitation at higher pH, enabling separation by pH control when oxidation state is managed appropriately. In a staged precipitation study, more than 95% of iron and aluminum were removed at pH values of ~3.0 and ~4.5, respectively, while lanthanum removal increased gradually as pH was elevated, underscoring narrower pH windows for Fe/Al versus broader pH response for REEs.

Where Fe is present as Fe(II), oxidation to Fe(III) can materially change precipitation and co-precipitation behavior; for example, one study reported that REE removal was noticeably elevated at pH 4.0 after adding H2O2 to convert ferrous to ferric ions, which caused nearly complete precipitation of iron and enhanced REE removal via interaction with ferric precipitates. However, competitive species such as aluminum can suppress REE adsorption onto ferric precipitates, which can reduce apparent REE removal under otherwise similar oxidation and pH conditions.

#### Oxalate precipitation for recovery at low pH

When the objective is REE recovery as a solid product (rather than simple immobilization), oxalic-acid precipitation is widely used because rare-earth oxalates have low solubility, but achieving high recovery often requires substantial reagent consumption and careful pH control. Multiple sources report that oxalic-acid-based REE precipitation can be optimized at low acidic pH ranges, with pH ~1.25–2.0 repeatedly identified as a window that maximizes recovery and precipitate purity while limiting co-precipitation of contaminants. One dataset indicates that, as pH rises above ~3, precipitation of contaminants (Al, Ca, Fe, Mn, Si) increases and reduces final REE product purity, while a pH around 1.5–2 is recommended to keep contaminants low and recovery high.

Oxalic acid speciation also explains the pH sensitivity of the precipitation kinetics and selectivity, with H2C2O4 dominating at pH < 1, HC2O4− between pH 1–4, and C2O4^{2−} at pH > 4, which changes complexation and precipitation equilibria in multi-ion solutions. In one analysis, a typical oxalic acid efficiency (OAE) of ~60% was observed, with ~40% of added oxalic acid consumed by precipitating/complexing non-REE elements, directly tying wastewater matrix composition to reagent costs and secondary waste generation. Where iron is present, Fe(III) can compete strongly for oxalate (forming Fe–oxalate complexes) and reduce REE precipitation efficiency, reinforcing the importance of upstream Fe removal or strict stoichiometry control in oxalate routes.

#### Phosphate and carbonate precipitation

Beyond hydroxides and oxalates, phosphate-based precipitation is used both for REE recovery and for broader metal immobilization, and phosphate excess can yield high REE recovery ratios under appropriate conditions. One phosphate precipitation study reports that selective separation of REEs from transition elements was achieved at pH values below 2.5, while also emphasizing that Fe is removed first by oxidizing Fe(II) to Fe(III) and precipitating Fe(III) hydroxides using strong alkalis prior to REE recovery steps.

### Membrane separation

Membrane unit operations are commonly deployed as pre-treatment and pre-concentration steps for REE-bearing wastewaters, including ultrafiltration and nanofiltration, to remove particulates and/or concentrate dissolved/complexed REEs prior to chemical separation or sorption-based capture. The membrane-technology literature classifies nanofiltration (NF) as having pore sizes below ~2 nm and ultrafiltration (UF) as having pore sizes in the ~2–50 nm range, and uses these in tandem with upstream conditioning (e.g., pH adjustment and complexation) to improve rejection of REE species.

A representative performance datapoint for REE rejection comes from micellar-enhanced ultrafiltration (MEUF), where an UP020 membrane achieved high REE rejection (~97%) at pH = 3.5, 25 °C, 3 bar operating pressure, 60 min contact time, and SDS concentration of 8.0 mM. A separate membrane-performance summary reports NF270 with MWCO ~180 ± 20 Da operating at pH 1 with rejection >98%, illustrating that tighter NF can achieve high rejection in acidic matrices (subject to fouling and scaling management). Polymer-complexation approaches are also reported, where addition of PAA (e.g., 30 mg/L) enabled >90% REE rejection at pH values between 8 and 9, consistent with the use of complexation to shift REE speciation toward membrane-rejectable forms.

For industrial coolant recycling, a two-stage filtration train has been demonstrated for grinding fluids: an initial bag filtration at 5 µm followed by low-pressure ultrafiltration at ~4 bar using polyacrylonitrile membranes with a 30 kDa cut-off in cross-flow batch mode at 25 ± 1 °C. In those experiments, ultrafiltration completely removed TSS, and reductions across turbidity, COD, TOC, surfactants, and total phosphorus ranged from ~80% to almost 100%, supporting UF as a robust coolant-loop clarification step even when the objective is not REE capture per se. However, broader reviews emphasize that membrane fouling and stability (including fouling from oils/organics) are key constraints for industrial coolant streams, reinforcing the need for pre-treatment specifically targeting emulsified oil and organic load before NF/UF deployment for REE recovery.

### Ion exchange and adsorption

Ion exchange and adsorption are repeatedly identified as promising or mature routes for concentrating and selectively capturing REEs from dilute aqueous streams, particularly when paired with upstream pre-concentration or particulate removal steps. A practical example of ion exchange for REE separation reports use of Lewatit MDS 200 H, a strongly acidic gel-type cation-exchange resin, with an operating pH range of 2–14 and maximum operating temperature of 120 °C, and provides quantitative operating data for column flow, loading, and elution strategies using NH4EDTA eluent chemistry. Under reported optimal conditions (initial pH 6.0 and 0.02 mol·L−1 NH4EDTA), the system achieved 98.4% heavy-REE purity in the initial stage (0–10 bed volumes), illustrating the potential for high-selectivity polishing when feed chemistry is controlled and compatible with chelant-based elution.

For adsorption, zeolites are a documented low-cost sorbent option for certain REE ions; one study reports over 90% removal of lanthanum and cerium using zeolite as a sorbent and provides relevant material properties for Zeolite 13X (e.g., 4–8 mesh particle size, BET surface area 576 m²/g, average pore size ~20 Å). The adsorption literature also highlights high-capacity engineered sorbents (e.g., EDTA-chitosan modified materials) with reported maximum adsorption capacities for REEs reaching hundreds of mg/g in laboratory systems, indicating the technical feasibility of high-capacity capture media for polishing or pre-concentration (subject to real-matrix limitations).

### Solvent extraction

Solvent extraction remains a standard, widely used approach for REE separation from aqueous solutions in hydrometallurgical flowsheets, but it is typically most suitable after leaching or concentration steps rather than direct treatment of very dilute waters, and it can generate organic waste streams that complicate wastewater management. Solvent-extraction systems commonly use organophosphorus extractants; one example reports use of D2EHPA and TBP in an organic diluent (Elixore 205) at an organic-to-aqueous (O:A) ratio of 1:1, with stripping of loaded organic using high-concentration nitric acid (6 mol/L). For membrane-solvent-extraction hybrids, supported liquid membranes (SLMs) are explicitly described as a modification of solvent extraction that combines extraction and stripping in one unit operation, with one parameterized example using 10% v/v DEHPA in kerosene and a 5 mol·L−1 HNO3 stripping solution.

Because machining-coolant wastewaters can be high-volume and may carry substantial organic load (surfactants/oil), extraction-precipitation strategies have been proposed as potentially suitable for large-scale applications in treating rare-earth-contaminated coolants while reducing chemical oxygen demand relative to conventional liquid–liquid extraction approaches in some reported comparisons.

### Cobalt-specific treatment

Cobalt can co-occur with REEs in relevant aqueous systems and can partition into treatment solids; one study notes that treatment solids can concentrate critical metals such as cobalt into oxide/hydroxide waste products, but does not provide a single universal pH window for cobalt removal analogous to the REE hydroxide window. In a staged-neutralization approach, one report states that neutralizing to pH 8 to 8.5 precipitated all REEs and co-precipitated most cobalt and manganese as a pre-concentrate material, indicating that cobalt can be captured in the same high-pH precipitation step used for REEs in some matrices. Conversely, in oxalate precipitation systems designed to recover REE oxalates, cobalt behavior can differ; one experimental report found cobalt concentrations did not change significantly during oxalic acid precipitation and concluded cobalt did not co-precipitate with REE oxalates under the tested conditions, enabling separation by filtration of REE oxalate solids from cobalt-bearing liquor.

These contrasting behaviors imply that cobalt management strategy should be selected based on the chosen REE recovery route and desired product form, rather than assumed to be inherently coupled to REE precipitation in all systems.

### Metalworking fluid pre-treatment

Effective treatment of REE-contaminated machining coolants generally begins with physical and physicochemical steps to remove free oil, tramp oil, and suspended solids, because these constituents foul membranes and interfere with adsorption/ion exchange performance. Industrial guidance on metalworking-fluid recycling describes practical unit operations including sump vacuum and filtration to remove chips/solids, skimmers and coalescers to remove tramp oil before it emulsifies, and centrifuges to separate pure coolant from emulsified tramp oil and both large and small contaminants. Consistent with this, another summary notes that many centrifuges can remove particulate matter as small as ~2 microns, which can be relevant for capturing fine metallic fines and abrasive particles typical of grinding operations.

Electrocoagulation is also described as a metalworking-fluid wastewater treatment method for removal of oils and contaminants, and one industry-facing source reports that electrocoagulation equipment followed by a filter press can reduce COD in cutting-fluid wastewaters by ~80%. For sites where disposal/haulage costs dominate, evaporative treatment is described as a practical route for spent coolant management, with one vendor claiming evaporators can provide a payback of less than one year versus hauling and help meet stringent spent-coolant disposal regulations (noting that performance and economics are site-specific).

### Integrated treatment train design

An integrated treatment train for rare-earth-contaminated machining coolants typically combines (1) oil and solids removal, (2) pH and redox conditioning, (3) bulk metal removal by precipitation/coagulation, and (4) polishing and/or recovery operations (membrane concentration, ion exchange/adsorption, or selective precipitation to a product). The table below summarizes a pragmatic chemistry-first decision structure, emphasizing pH setpoints and the role of pre-treatment.

| Objective | Primary chemistry lever | Typical pH regime | Enabling steps and constraints |
| --- | --- | --- | --- |
| Bulk immobilization and discharge compliance | Hydroxide precipitation via pH raise | REE hydroxides typically precipitate at pH ~8–10 (matrix dependent) | Stage Fe removal first at ~pH 3–4 before REE precipitation to reduce co-precipitation losses |
| REE recovery as oxalate product | Oxalic acid precipitation | Optimal recovery/purity often at pH ~1.25–2.0 with sufficient residence time and dosing (e.g., 30–40 g/L, >2 h) | Oxalic acid efficiency can be ~60% with substantial consumption by non-REEs, so impurity control and pre-removal of Fe are critical |
| Coolant-loop reuse with reduced disposal | Filtration and UF clarification | UF with 30 kDa at ~4 bar in cross-flow can fully remove TSS and reduce COD/TOC/surfactants substantially | Oils/surfactants remain key; upstream coagulation/centrifugation is commonly used in practice |
| REE pre-concentration from dilute streams | Membranes (NF/UF) and/or ion exchange | MEUF example: pH 3.5, 3 bar, SDS 8 mM, 97% rejection on UP020 | Fouling from oils/organics requires dedicated pre-treatment in coolant matrices |
| High-selectivity separation or polishing | Ion exchange or engineered sorbents | Example IX system operated with initial pH 6.0 and NH4EDTA eluent to reach high HREE purity | Resin selection and regeneration chemistry matter; strong-acid resin examples report broad operating pH ranges and acid regeneration options |

In practice, pH-based precipitation and coagulation steps are typically paired with physical solids separation (clarification, filtration, centrifugation) because pH adjustment is used both to drive metal hydroxide formation and to facilitate suspended solids removal via aggregation and settling. Where downstream membranes are included, the literature emphasizes that pH changes can induce scaling (e.g., Ca/silicate precipitation) and that pretreatment is needed to prevent membrane shutdown, which is directly relevant to coolant streams containing hardness ions and surfactants.

Finally, effluent targets and required treatment intensity are explicitly described as site-specific, with regulatory discharge limits and effluent quality objectives determining the necessary level of REE and co-metal removal rather than any universal REE discharge standard. Where numeric standards are available for analogous regulated parameters, examples include rare-earth-industry effluent standards such as GB 26451-2011 limits (phosphorus 5 mg/L, COD 100 mg/L) used in one study to judge compliance after oxidation and precipitation treatment, illustrating how non-metal metrics (COD) can become the binding constraint even when metal/phosphorus targets are met.

### Implications for Algamol case development

This chemistry review provides a parameter set that can be used to scope an Algamol facility business case around REE-contaminated coolant treatment by bounding required pH windows, identifying pre-treatment needs, and identifying where higher selectivity steps (ion exchange/adsorption or membrane pre-concentration) become feasible after organics control. In particular, the need for organics and emulsified oil removal before membrane or adsorption-based recovery is explicitly emphasized in coolant-focused reviews, which should be treated as a front-end requirement when estimating system footprint, consumables, and operational complexity at a machining site. Benchmarking against alternative onsite strategies (e.g., evaporators marketed for spent coolant disposal) can also inform the economic baseline, given claims of payback driven by reduced hauling and compliance support, even though detailed cost data remain site-specific and would need onsite measurement and validation.

## Chapter 7

### Algamol Wastewater Treatment Business Case Defense Grade Rare Earth Magnet Machining Facility

### Summary

Defense-grade permanent magnet machining and finishing (e.g., grinding, EDM, slicing) generates coolant and sludge streams that are frequently expensive to manage offsite and are often constrained by hazardous-waste classification and local sewer discharge requirements. Across multiple industry sources, offsite hazardous waste disposal is commonly summarized as $0.50–$3.00/gal (about $130–$800/m³), with additional variability by jurisdiction and waste characteristics. Reported per-gallon disposal pricing can be substantially higher for small-volume drum pickups (e.g., ~$8–$9/gal), reinforcing the cost penalty for small and mid-sized operations without in-house treatment or recycling.

A pragmatic Algamol deployment concept for this segment is a modular on-site treatment train that (1) pre-separates tramp oil and solids (e.g., ultrafiltration) and (2) applies an algae-based capture step intended to reduce dissolved metal loads and concentrate metals into a manageable biosludge stream for offsite recycling or downstream recovery. A representative small-system cost envelope cited in the compiled business-case inputs is total CAPEX ~$110k–$310k with annual OPEX ~$28k–$73k/year (inclusive of chemicals, energy, labor, membrane replacement, and sludge handling). If such a system replaces (or meaningfully reduces) offsite hauling of coolant/wastewater at $130–$800/m³, simple paybacks can be short (often <2 years) even before assigning any value to recovered cobalt/rare earths, because disposal costs can dominate lifecycle economics at small industrial sites.

### Target customer profile

The target customer is a defense-sector magnet machining and finishing facility or subcontractor that handles high-temperature rare-earth magnets (SmCo and high-coercivity NdFeB) as finished-to-print parts or precision-machined blanks, and therefore runs multiple wet machining steps requiring recirculating coolant and periodic sump change-out and sludge handling. In practice, these sites can include integrated magnet manufacturers and machining houses prioritized for pilots such as EEC/Permag (Landisville, PA), Arnold (multiple sites), and VAC (Sumter, SC).

Operationally, coolant streams are typically high-water-content emulsions; one source notes coolant can be up to 10% concentrate mixed with 90% water, and another notes the remaining liquid after removing sludge and oils is often ~90–98% water. For grinding operations on RE-magnet materials, published process data indicate coolant can be applied at ~5% concentration with a throughput of ~30–50 L/min per wheel, and that grinding slurries can retain from <1% to >20% cooling agent after on-site solid–liquid separation and recirculation.

For business-case sizing, an indicative wastewater generation range used in the business-case inputs is ~5–50 m³/day depending on facility size and operational intensity. For comparison, metalworking facilities with smaller footprints can report far lower volumes (e.g., ~2 m³/month in one automotive metalworking wastewater case study), highlighting that volumes are extremely site-specific and often depend on coolant management practices and recirculation discipline.

### Wastewater characterization

A defensible near-term characterization is to treat magnet-machining coolant wastewater as an oily, metal-bearing wastewater with substantial solids (swarf/abrasives) and variable oil content, rather than as a single uniform stream with fixed dissolved-metal concentrations. The most operationally relevant descriptors for treatment selection and permitting are (a) the emulsion/oil fraction, (b) suspended solids and fine metallic particles, and (c) pH and discharge acceptability for sewer or surface-water pathways.

From the available sources, key measurable characteristics and constraints include:

- pH envelope for sewer acceptability: one guidance document notes spent cutting fluids are generally acceptable for discharge to municipal sanitary sewers when pH is between 6.0 and 9.0 (subject to local approval).

- Oil limits: the same source gives an example acceptance criterion that tramp oil concentrations do not exceed 100 mg/L for discharge to municipal sanitary sewers (again subject to local sewer authority).

- Analytical comparators: an industrial coolant disposal resource provides an example table of measured results and “legal limits” for several parameters, including fats/oil/grease and total petroleum hydrocarbons, and TCLP metals comparators (e.g., total chromium result 0.008 mg/L with a 5.0 mg/L comparator).

For rare-earth magnet grinding specifically, the “slurry” is not only wastewater; it is a multi-phase mixture of metallic swarf, abrasives from the grinding media, and coolant, with remaining coolant content varying widely depending on separation effectiveness. In one study of NdFeB grinding slurry handling, up to 5% of the magnetic alloy can end up in the slurry sink, which directly links tighter machining and finishing requirements to higher metal-bearing waste generation.

The provided sources do not supply reliable, defense-magnet-specific dissolved concentration ranges for Nd/Pr/Dy/Sm/Co/Fe in coolant wastewater, and one review explicitly notes it does not address wastewater generation rates from machining facilities or grinding coolant usage, reinforcing the need for site sampling rather than generic concentration assumptions.

### Current cost baseline

#### Cost drivers

The “cost of doing nothing” is typically a combination of coolant purchase/replacement, offsite hauling and disposal (especially if classified hazardous), minimum pickup charges, and recurring compliance/testing burdens associated with hazardous classification and discharge approvals. Coolant itself can be expensive (reported up to $50/gal in some contexts), which increases the economic value of extending coolant life through recycling or separation-based maintenance programs.

Disposal costs vary widely by location, waste type, and packaging (bulk vs drum), and sources report a broad range of disposal prices from $0.25/gal to $6/gal and beyond (including higher “expensive” categories). One industry resource summarizes offsite hazardous disposal as $0.50–$3.00/gal ($130–$800/m³), and user discussions show that small-volume drum pickups can cost on the order of ~$490 per 55-gal drum (~$8.9/gal).

#### Baseline cost range

Using the commonly cited $130–$800/m³ offsite hazardous disposal range, annual offsite disposal cost scales linearly with wastewater volume and can dominate total operating cost at mid-sized machining sites. The table below shows the implied annual hauling/disposal spend for two representative facility scales (excluding internal labor, manifests, sampling, and minimum pickup charges).

| Facility scale | Wastewater volume assumption | Annual volume | Offsite disposal cost range | Implied annual disposal spend |
| --- | --- | --- | --- | --- |
| Scenario A | 5 m³/day | 1,825 m³/yr | $130–$800/m³ | ~$237k–$1.46M/yr |
| Scenario B | 20 m³/day | 7,300 m³/yr | $130–$800/m³ | ~$949k–$5.84M/yr |

These ranges are consistent with the general observation that disposal costs depend strongly on jurisdiction and materials processed, and that high-cost regions can see materially higher per-gallon charges.

### Proposed system

A defensible modular treatment concept for a defense magnet machining facility pairs well-established physical separation with an Algamol algae-based step that is positioned to support metals capture/recovery from wastewater, with a final pH/discharge polishing step for sewer or reuse pathways where permitted. This modularization is important because multiple sources highlight that basic requirements often begin with pH neutralization, and that local sewer utilities must be consulted before discharge of metalworking liquids.

#### Process flow

A practical process flow consistent with the source evidence is:

- Equalization and oil/solids management (tank + skimming/filtration), to dampen variability and remove free/tramp oils and large solids before membranes or biosorption.

- Ultrafiltration pre-treatment to break emulsions and remove fine suspended solids, producing a treated aqueous phase plus a concentrated oily phase that can be sent for oil recovery or incineration.

- Algamol bioreactor module as the primary metal-capture step; Algamol states its technology supports the recovery of rare earth metals from wastewater, while broader literature indicates algae-based REE biosorption is promising but still early-stage for real wastes and generally benefits from mildly acidic operating conditions (often pH 3–6, with a commonly reported optimum around pH 5).

- Polishing and discharge conditioning (pH adjustment and final filtration) to meet site-specific discharge requirements (e.g., sewer acceptance pH 6–9, tramp oil targets).

- Sludge handling to dewater and manage captured solids; depending on composition and TCLP results, sludge may be disposed as hazardous or non-hazardous, with very different unit costs.

This approach aligns with the observation that precipitation/neutralization plus settling is a standard metals-removal paradigm for aqueous metal-bearing streams, even if the ultimate objective is recovery rather than disposal.

### Capital cost estimate

On the low end, small coolant ultrafiltration units are reported at ~$5,000–$13,000 for equipment sized to ~100–300 gallons/day, which can support pilots, point-of-use recycling, or small shops but is typically insufficient as the sole treatment method for larger defense machining facilities generating multi-m³/day streams.

For a more comprehensive modular on-site system (UF + bioreactor + pH dosing + controls/installation), the business-case inputs cite an estimated total CAPEX of ~$110k–$310k, with an illustrative component breakdown of UF membrane unit $30k–$80k, bioreactor module $50k–$150k, pH adjustment/chemical dosing $10k–$30k, and controls/piping/installation $20k–$50k.

These figures are directionally consistent with broader industrial wastewater guidance that places small industrial system CAPEX in the ~$50k–$500k range depending on complexity and customization, and with vendor commentary that system upgrades can require investments such as ~$65k–$200k and that complex multi-stage systems can reach ~$400k to >$1.5M depending on volume and requirements.

### Operating cost estimate

The business-case inputs provide an estimated annual OPEX range of ~$28k–$73k/year, with the cost drivers itemized as chemicals (NaOH/acid/nutrients) $5k–$15k/year, membrane replacement $5k–$10k/year, energy $3k–$8k/year, labor (part-time operator) $10k–$25k/year, and sludge disposal/processing $5k–$15k/year.

Independent sources reinforce why OPEX is often the dominant cost driver over lifecycle horizons; for example, one wastewater treatment provider notes that operating costs (including disposal/recycling, electricity, staff/analytics/maintenance) can materially exceed the initial investment over time, and that disposal and recycling alone can represent a large fraction of overall lifecycle cost.

Where discharge to a POTW is feasible after treatment, unit “discharge” costs can be far lower than hazardous hauling; an EPA unit-cost compendium lists discharge to POTW (sewer) at $1.50 per 1,000 gallons (exclusive of the facility’s pretreatment costs and any local surcharges), which is roughly $0.40/m³. However, the same compendium also lists a cost for trucking offsite to POTW management at $0.07/gal (excluding transportation), illustrating that logistics and packaging can still impose meaningful costs even when the endpoint is a municipal facility rather than hazardous disposal.

### Revenue streams

The most defensible “revenue” mechanism in the near-term business case is avoided disposal, because offsite hazardous disposal costs are directly cited and can be very large at multi-m³/day volumes. Additional potential value can come from recovered metals, but the evidence base specific to “magnet machining coolant” recovery is currently thinner than for solid swarf and grinding slurries, so metal-recovery revenues should be treated as upside dependent on sampling and downstream refining partnerships.

Still, multiple sources support the plausibility of metal value capture from magnet-related wastes:

- Cobalt value from SmCo streams: the business-case inputs cite an indicative cobalt value of $24–$34/kg for recovered cobalt from SmCo streams, which frames the potential upside when cobalt-rich solids are captured into a concentrated sludge rather than diluted into offsite liquid disposal.

- Rare earth concentrate value benchmark: the business-case inputs cite a benchmark value of ~$24k–$30k per “tonne of equivalent swarf” for recovered REE concentrate, which can be used as a planning proxy for high-REE solids streams if Algamol’s biosludge can be routed to a recycler/refiner.

- Grinding slurry as an REE-bearing waste: a NdFeB grinding slurry study notes that grinding slurries are mixtures of swarf/abrasives/coolant and that up to 5% of alloy can end up in this sink, implying a direct feedstock for recovery if captured and segregated.

- Commercial feasibility of high recovery/purity in related routes: literature summaries report recovery methods such as membrane-assisted solvent extraction at ~95% REE recovery and molten salt electrolysis producing neodymium at 99.78% purity, indicating that high recovery and high purity are feasible somewhere in the downstream chain even if not performed on-site at the machine shop.

### ROI and payback analysis

This ROI model intentionally separates (A) “avoided disposal” economics (typically bankable) from (B) “recovered metals” economics (typically upside), because many sources emphasize classification and local regulatory context as the decisive cost drivers rather than the theoretical value of dilute dissolved metals.

#### Scenario A

For a facility generating 5 m³/day (1,825 m³/year), the implied annual offsite disposal spend at $130–$800/m³ is ~$237k–$1.46M/year. If Algamol’s modular system is implemented with annual OPEX ~$28k–$73k/year, the annual “avoided disposal minus treatment OPEX” benefit is approximately ~$164k–$1.43M/year (ignoring residual offsite costs for any unrecovered concentrate and ignoring potential metal revenues). With CAPEX ~$110k–$310k, simple payback is therefore approximately ~0.1–1.9 years across the disposal-cost range, which is consistent with third-party wastewater industry statements that some clients see ROIs as fast as ~1½ years and that typical ranges can be ~3–4 years depending on context and system scope.

#### Scenario B

For a facility generating 20 m³/day (7,300 m³/year), the implied annual offsite disposal spend at $130–$800/m³ is ~$949k–$5.84M/year. Using the same OPEX estimate ($28k–$73k/year) and CAPEX ($110k–$310k), simple paybacks compress to weeks-to-months in the high hauling-cost case and remain well below 1 year in most cases, even before any recovery crediting, because the baseline is dominated by high unit disposal costs applied to large annual volumes.

#### Sensitivities

The dominant sensitivities that determine real-world payback are (1) whether the waste can be reclassified away from hazardous through proper characterization and segregation, (2) whether on-site recycling can materially reduce disposal volumes (e.g., via ultrafiltration and sump management), and (3) the local jurisdiction’s hauling/disposal and sewer acceptance constraints. A qualitative case example illustrates the power of classification and recycling: one client reduced haul-away costs from ~$275–$300 per 55-gallon barrel to ~$90 per barrel by relabeling for recycling, and other sources report >80% disposal-cost reduction after implementing cutting fluid recycling systems, demonstrating that the “disposal avoidance” lever can be very large in practice.

### Strategic value proposition

Beyond direct cost savings, Algamol can position a defense-magnet machining wastewater solution around three strategic dimensions that are evidence-supported in the source set.

First, supply chain and critical-minerals circularity creates a strategic rationale to keep REE- and cobalt-bearing residues on-site or in domestic recycling loops; defense supply chain policy is increasingly focused on magnet and rare earth supply vulnerabilities, and DoD has invested >$439M since 2020 to establish domestic rare earth element supply chains. A wastewater-to-concentrate pathway that supports downstream recovery can be framed as an enabling step in “mine-to-magnet” resiliency—especially for high-value residues like SmCo-related cobalt-bearing solids.

Second, technology differentiation via biosorption is plausible but must be communicated honestly as an emerging pathway: multiple reviews note algae-based or algal-sorbent REE recovery from real wastes is still in its infancy, even though promising lab and pilot performance exists (e.g., reported removal up to 86% at pH 5 in one study, and REE sorption efficiencies in the high-90% range in other cited work). A credible commercialization narrative therefore depends on a staged pilot program with real coolant and sludge samples, consistent with calls for laboratory results to be tested with actual manufacturing wastewater.

Third, regulatory and permitting risk management is intrinsically valuable because discharge permits are case-by-case and local sewer authorities require consultation, and hazardous waste determinations create testing and reporting burdens. Even without claiming universal discharge limits, Algamol can offer a compliance-oriented value proposition centered on pH management and oil/solids control to align with commonly cited sewer acceptance criteria (pH 6–9; tramp oil ≤100 mg/L) and to reduce hazardous-waste designation risk through segregation and pre-treatment.

### Go-to-market recommendations

A defensible go-to-market plan in the defense magnet machining niche should be structured as a pilot-to-scale program that converts “unknown” site-specific chemistry into bankable economic and compliance outcomes. Near-term recommended steps include:

- Prioritize pilots at named defense-relevant targets (EEC/Permag, Arnold, VAC Sumter) to access real streams and establish security/compliance operating procedures early.

- Instrument the pilot for cost and compliance outcomes, including routine sampling and metals panels; published unit cost compendia provide representative costs for sampling and metals analyses (e.g., ~$36–$45 per sample and ~$89 for RCRA metals 6010; ~$171 for a “priority 17 metals” panel).

- Design for upstream separation first, because ultrafiltration is a well-established method in coolant management and produces a concentrated oil phase that is typically managed via oil recovery or incineration, reducing the load to downstream biological or adsorption steps.

- Commercialize with a TCO-first offer, aligning with industry messaging that hidden and ongoing costs can exceed equipment price in a few years and that the most economical solution is not always the cheapest upfront, which supports lease/service models with performance guarantees tied to disposal avoidance.

### Implementation gaps

Several gaps in the available evidence base matter directly for Algamol’s technical de-risking and the credibility of a defense-sector business case.

The most material gap is a lack of published, defense-magnet-machining-specific coolant wastewater composition data, particularly dissolved Nd/Pr/Dy/Sm/Co/Fe concentrations, which prevents a rigorous bottom-up recovered-metal revenue forecast from wastewater alone. A second gap is the limited availability of industrial-scale case studies for algae-based REE recovery on real manufacturing effluents; the literature contains promising lab and pilot results, but also emphasizes the early maturity of algal sorbents for real wastes and the need for industrial validation.

Finally, the waste form often most relevant to rare earth value capture may be grinding slurries and solid residues, not the aqueous phase alone; one European-focused study notes that there is no established industry for recycling NdFeB grinding slurries in Europe and that slurries are often stored and exported to China or disposed as hazardous due to oily components, emphasizing both a market gap and a potential domestic opportunity for integrated capture and recovery solutions. A pilot program should therefore explicitly treat slurry management (segregation, dewatering, and routing to recyclers) as a core part of the value proposition, rather than positioning the solution solely as a water discharge compliance tool.