# PART III Regulatory landscape facility models and market strategy

## Chapter 8 Consolidated executive briefing

### Strategic Briefing High Temperature Rare Earth Permanent Magnets in the Defense Sector Key Findings and Algamol Market Opportunity

#### 1. The Defense Magnet Market

Rare-earth permanent magnets are enabling components across major defense platforms and subsystems, with DoD explicitly citing their importance across aircraft, submarines, and UAVs. The scale of rare-earth usage at the platform level is substantial; DoD cites that an F-35 requires more than 900 lb of rare earth elements (REEs), an Arleigh Burke DDG-51 destroyer about 5,200 lb, and a Virginia-class submarine about 9,200 lb, which underscores why magnet availability and provenance have become procurement-critical.

For high-temperature, high-reliability defense environments (roughly 150–550°C), the source base points to two primary material families. Samarium-cobalt (SmCo5 and Sm2Co17) magnets are repeatedly described as offering strong hard-magnetic properties, thermal stability, and corrosion resistance, and as preferred for high-temperature applications (e.g., 200–550°C). High-temperature NdFeB is positioned as viable mainly by coercivity-engineering: heavy rare earth (HREE) additions (Dy/Tb) to prevent demagnetization around 150–200°C, plus microstructure engineering such as grain-boundary diffusion (GBD) to raise coercivity without bulk HREE loading.

Quantitatively, SmCo magnets are reported to span roughly 16–33 MGOe in (BH)max across grades and processing conditions. A specific Sm2Co17-grade example (EEC 2:17-27) is reported at (BH)max 26–28 MGOe with intrinsic coercivity 34.1–36.1 kOe and Br 10.5–10.8 kGs, which is directionally consistent with the “high-temperature” use case emphasized for SmCo in defense subsystems. Commercial SmCo grade data (e.g., VACOMAX) also explicitly cites (BH)max in the ~20–33 MGOe range and maximum operating temperature up to ~350°C for some alloys, providing a practical “catalog anchor” for high-temperature procurement and qualification decisions.

On the supply side, the public record emphasizes structural concentration risk in upstream REE supply chains. GAO summarizes rare-earth supply chains as global and multi-tiered, with China dominating mining and processing stages, and with many supply-chain nodes between ore and delivered defense components. Related policy/legal commentary also quantifies China’s position as ~60% of rare earth mining and ~90% of rare earth processing (2023), which is a key driver of Western “re-shoring” and recycling push strategies.

A small set of defense-relevant suppliers recur across the sources for “defense-grade” magnets and assemblies. Examples include Electron Energy Corporation (EEC), which states it purchases pure raw materials (Sm, Co, Fe, Cu, Zr) for in-house production of SmCo alloys and is ITAR/DDTC registered with AS9100D/ISO 9001 certification, and Arnold, which cites ITAR-registered supply chain for its SmCo RECOMA products and lists AS9100/ISO 9001 certifications, signaling a defense-ready compliance posture. VACUUMSCHMELZE is positioned in the source set as a leading global manufacturer of advanced magnetic materials and as the source of VACOMAX SmCo material and downstream processing capabilities including wire-EDM, which matters because machining and tight tolerances are a central cost/yield driver for defense magnets.

#### 2. The Waste Problem

The machining stage is consistently described as the dominant driver of material loss and waste generation for sintered rare-earth magnets, because sintered magnets cannot be produced directly to final, precise shapes and are therefore made as bulk blocks that must undergo extensive machining and grinding to meet final geometry and tolerances. One public U.S. report states machining recovery rates of 60–90% depending on size/complexity, implying 10–40% of material becomes machining waste in many cases, and explicitly identifies machining as the step with the greatest material loss.

Across the broader magnet manufacturing literature, reported waste fractions can be even higher for certain shapes: multiple sources cite 6–73% of raw material becoming swarf depending on final magnet geometry, while “typical” NdFeB cutting and grinding scrap is frequently cited at ~20–30 wt% of starting alloy in manufacturing practice. Waste is not a minor side stream: one analysis estimates the swarf recycling stream in 2020 was ~9% of global NdFeB magnet production, indicating swarf-handling and recycling logistics are already a non-trivial industrial system.

Waste form also matters. A review focused on magnet waste states that more than 70% of manufacturing waste consists of sludge that is heavily oxidized and contaminated with carbon from lubrication, which directly complicates recovery and creates a wastewater and sludge-handling problem rather than a clean “metal scrap” problem. Additionally, magnet cuttings/swarf can contain machine cutting fluids and magnetic powder, and cutting is performed before magnetization, which has implications for handling (magnetic vs non-magnetic), contamination control, and downstream separations.

SmCo machining has an additional yield penalty because Sm-Co magnets are brittle. Sources attribute production loss up to 20–30% in volume to brittleness, and separately note Sm-Co machining failure rates up to ~20%. This brittleness-driven fracture risk is observable at the part level; one study reported 1 of 7 reference SmCo magnets (~14.3%) fractured during wire-EDM preparation, illustrating why high-reliability, tight-tolerance programs can see compounding scrap and rework costs in finishing operations.

The “value leaking” through this waste stream is amplified in high-temperature/high-coercivity NdFeB because Dy/Tb are expensive, and their use scales with temperature requirements. Sources note Dy/Tb additions are used to prevent demagnetization in the 150–200°C regime, and non-defense examples report NdFeB magnets can contain substantial HREE additions (e.g., “about 10 wt% Dy/Tb” in certain high-temperature motor contexts) and “up to 11 wt% dysprosium” cited in another high-temperature application context, indicating the magnitude of the value-at-risk can rise substantially above baseline compositions in the highest-temperature NdFeB grades.

#### 3. The Regulatory Tailwind

The procurement environment is hardening around magnet provenance, with a single date acting as a forcing function for defense primes. GAO reports DoD completed DFARS rulemaking for the revised prohibition and that the expanded prohibition will go into effect January 1, 2027, and also flags that DoD will have to require contractors to disclose magnet sources in DoD systems. The DFARS final rule published May 30, 2024 is effective and sets the staged restriction framework in the DFARS clause; sources explicitly cite the rule’s May 30, 2024 effective date and the January 1, 2027 expansion that prohibits covered materials mined/refined/separated/melted/produced in covered countries.

A key nuance for strategy is that DFARS covers both SmCo and NdFeB as “covered materials,” and that the 2027 expansion explicitly extends to the full upstream supply chain for both magnet types, not only final magnet manufacturing steps. DFARS also contemplates exceptions via nonavailability determinations, and the Federal Register text states DoD can issue individual and class nonavailability determinations if compliant materials are unavailable (DFARS 225.7018-4).

Recycling becomes strategically important because DFARS provides a specific compliance pathway for NdFeB magnets manufactured from recycled material if (and only if) the milling of recycled material and the sintering of the final magnet occur in the United States. This “recycled NdFeB” pathway is a direct demand driver for U.S.-based recycling and refining capacity and increases the strategic value of domestically captured and traceably processed manufacturing scrap and end-of-life magnet feedstock.

The DoD response to upstream concentration risk is not only regulatory. GAO reports that since 2020, DoD has awarded approximately $439 million to vendors to reestablish domestic rare earth supply chains, and DoD messaging emphasizes the importance of keeping critical supply-chain nodes in the United States to secure magnet supply.

In Europe, the policy direction is similar in objectives but different in mechanism. The EU’s Critical Raw Materials Act (CRMA) sets 2030 benchmarks to reduce dependence on third countries, including targets of at least 10% extraction, 40% processing, and 25% recycling capacity (of EU annual consumption) for strategic raw materials, and a diversification target to avoid >65% reliance on any single third country at any processing stage. For permanent magnets, CRMA includes labeling and recycled-content disclosure mechanisms for certain products, including requirements to indicate magnet type (NdFeB, SmCo, etc.) and disclosure of the share of key elements recovered from post-consumer waste by May 24, 2027, alongside a pathway for minimum recycled content rules by end-2031; however, the CRMA also explicitly exempts products primarily designed for defense or space applications from these Article requirements, so defense procurement effects may be more indirect (industrial capacity and recycling ecosystem) than direct contract flowdown in many cases.

EU dual-use export controls can further shape availability and cross-border logistics for high-performance magnets. Regulation (EU) 2021/821 establishes an export-control regime for dual-use items, requires export authorization for items listed in Annex I, and requires exporters to keep detailed records for at least five years, which increases compliance friction and can tighten the practical availability of certain high-performance magnet products for international programs.

#### 4. The Wastewater Opportunity

The “machining waste” problem is also a wastewater and coolant-management problem, because grinding/cutting slurries are mixtures of metallic swarf, grinding-media abrasion, and a relevant amount of cooling agent, and residual coolant can remain from <1% to >20% by weight even after on-site solid-liquid separation and coolant recirculation. In addition, swarf streams may contain cutting fluids and magnetic powder, which creates a combined solids-and-organics treatment challenge rather than a simple “settle out metal” problem.

From a treatment-chemistry standpoint, the literature indicates that pH-driven precipitation is a workhorse method for metal removal in industrial wastewaters. Sources describe hydroxide precipitation via pH adjustment as widely used for metal-ion removal, and emphasize that pH adjustment is commonly applied to precipitate metal hydroxides and facilitate removal of suspended solids. For REEs specifically, multiple sources converge that REE hydroxide precipitation typically occurs in the alkaline range (roughly pH ~8.6–10, matrix dependent).

Selectivity is typically achieved with staged pH control, especially when iron is abundant (as in NdFeB-related waste streams). Review-level guidance and EPA-oriented sources describe iron removal prior to REE precipitation by adjusting pH to about 3–4 (often cited ~3.5) to precipitate Fe(III) hydroxides/oxides while keeping REEs in solution, followed by raising pH for REE precipitation to avoid co-precipitation losses. This staged approach is directionally consistent with the chemistry reality that magnet-waste-derived solutions and slurries often contain large iron fractions (e.g., NdFeB magnets with ~69% iron by weight in one supply-chain report framing), which otherwise drives high sludge volumes and poor selectivity if treated in a single pH step.

A key operational constraint for machining coolants is organic load (oils, surfactants). A coolant-treatment source notes that water-mix cutting fluids are diluted to create oil-in-water emulsions (2–10% by volume oil in water), and a coolant-focused REE recovery review notes membrane-based pre-concentration is promising but fouling from oils/organics complicates direct membrane use and typically requires pretreatment to remove organics and emulsified oils before membrane/adsorption-based REE recovery is viable.

Within this chemistry and operations envelope, Algamol’s stated positioning is that its technology supports recovery of rare earth metals from wastewater, which aligns with the market need for treatment-plus-recovery rather than disposal-only solutions in a DFARS- and CRMA-driven environment. At the same time, the research base cautions that algal sorbent research for REE recovery from real wastes is still in its infancy, and biosorption is commonly preferred under mildly acidic conditions (pH 3–6, with a commonly reported optimum around pH ~5), which matters for how Algamol is integrated with staged precipitation and oil-removal steps (i.e., it is unlikely to be a “single-box” solution).

Economically, the strongest initial ROI lever is disposal avoidance. Offsite hazardous waste disposal is commonly cited at $0.50–$3.00/gal (about $130–$800/m³), with additional minimum pickup charges and substantial variability by site and classification. On the cost side of the solution, the compiled Algamol deployment inputs cite a total installed CAPEX estimate of $110k–$310k and annual OPEX components (chemicals, membrane replacement, energy, labor, sludge handling) that sum to about $28k–$73k per year.

The table below shows a simple disposal-avoidance payback framing using only cited disposal ranges and cited CAPEX/OPEX ranges (excluding potential recovered-metal revenues and excluding residual offsite disposal for concentrated residues).

| Scenario | Wastewater volume | Disposal cost range | Annual disposal baseline | Annual treatment OPEX | CAPEX range | Simple payback estimate |
| --- | --- | --- | --- | --- | --- | --- |
| Small facility | 5 m³/day | $130–$800/m³ | ~$237k–$1.46M/yr (calc.) | ~$28k–$73k/yr | $110k–$310k | ~0.1–1.9 years (calc.) |
| Medium facility | 20 m³/day | $130–$800/m³ | ~$949k–$5.84M/yr (calc.) | ~$28k–$73k/yr | $110k–$310k | <1 year in most cases (calc.) |

This framing is consistent with broader industry narratives that wastewater projects can achieve rapid ROI when hauling/disposal is the baseline, including claims of sub-year payback for certain evaporation systems versus hauling and typical ROIs in the ~1.5–4 year range for broader industrial wastewater investments (noting that these are general benchmarks and site-specific).

#### 5. Go to Market

The near-term commercial opportunity is to target magnet suppliers and machining houses where (a) high-value rare earth and cobalt material is being converted into contaminated sludge, and (b) DFARS-driven provenance and domestic-processing incentives increase the strategic value of keeping these byproducts controlled, characterized, and routed to compliant downstream recovery.

A practical first wave of targets named in the compiled business-case inputs includes EEC/Permag (Landisville, PA), Arnold (multiple sites), and VAC (Sumter, SC). These targets align with the supplier landscape in which EEC is ITAR/DDTC registered and sources/purchases raw materials for in-house SmCo alloy production, and Arnold states ITAR-registered supply chain locations for its SmCo RECOMA products and defense-focused quality system certifications.

The recommended commercialization path is a pilot-to-scale program that uses on-site sampling and controlled demonstration to convert uncertain wastewater composition into bankable operating parameters. This is supported by both (i) the general industrial wastewater guidance that field tests and sometimes pilot tests are important and can be coordinated with a reputable water-treatment firm, and (ii) the biosorption literature’s explicit caution that lab-scale results need to be advanced and tested with actual wastewater samples from manufacturing sources.

From a system-architecture perspective, the best-supported “minimal viable product” is an integrated train that begins with pretreatment for oils/solids (to protect downstream concentration/capture steps) and then applies staged chemistry and/or biosorption where it has a plausible operating window. This approach is consistent with (a) coolant pretreatment requirements before membrane/adsorption-based REE recovery and (b) the practical staged precipitation concept (Fe at ~pH 3–4; REE hydroxides at ~pH 8.6–10) for selectivity and sludge reduction.

#### 6. Key Risks and Unknowns

The largest technical and commercial unknown is that published data are still thin on site-specific coolant wastewater chemistry for defense magnet machining, particularly for dissolved Nd/Pr/Dy/Sm/Co concentration ranges in real coolant loops and the partitioning between aqueous phase, fines, and oily sludge under different coolant chemistries and separation practices. This matters because the revenue upside (metals recovery from wastewater) depends on actual loading and partitioning, while the cost downside (treatability, sludge disposal) depends on emulsion stability, organics, and contaminants, which are known to complicate REE recovery from real machining slurries.

A second risk is technology maturity for algae-based capture in real industrial matrices. The literature states research into algal sorbents for REE recovery from real wastes is still in its infancy, and this must be reflected in commercial claims and risk allocation (e.g., performance guarantees should initially be based on disposal reduction and compliance metrics rather than recovered-metal credits).

A third risk is regulatory and procurement interpretation drift as DFARS source-disclosure requirements are implemented. GAO indicates DoD is moving toward requiring contractors to disclose magnet sources, and DFARS-related implementation for provenance disclosure is expected between 2025 and 2027, which means supplier questionnaires and traceability expectations may tighten during the same period that primes are requalifying suppliers for the 2027 expanded prohibition.

Finally, the supply-chain transition remains schedule-critical. GAO notes supply chains are multi-tier and China-dominated, and DoD has assessed it can take a decade or longer to establish new domestic sources for rare earths, which implies a non-trivial risk that capacity and qualification lag the January 1, 2027 prohibition expansion for certain niche high-temperature grades unless recycling and allied sourcing pathways scale quickly.

Near-term diligence priorities therefore center on actions that directly de-risk both technical performance and investor-underwritable economics:

- A defensible sampling plan for coolant, sludge, and swarf streams (including oil content, pH, solids, and metals) to bound treatability and confirm what fraction of rare earth value is in the solid phase versus aqueous phase.

- A pilot design that explicitly includes oil/solids pretreatment before any membrane/adsorption/biosorption steps, consistent with coolant fouling constraints noted in the literature.

- A commercialization narrative tied to DFARS 2027 and the “recycled NdFeB milled and sintered in the U.S.” exception, because those rules create a premium for domestic, traceable processing of magnet manufacturing byproducts into compliant feedstock pathways.

## Chapter 9 EU defense procurement rules

### EU Defense Procurement Rules for Rare Earth Permanent Magnets REACH Dual Use Controls and Critical Raw Materials Act

### Executive Summary

The European Union's regulatory framework for rare earth permanent magnets in the defense sector is a multi-layered system designed to enhance strategic autonomy and de-risk supply chains heavily dependent on China. Unlike the direct procurement prohibitions found in the US Defense Federal Acquisition Regulation Supplement (DFARS), the EU's approach is centered on industrial policy, capacity building, and market-shaping incentives. Key regulations include the European Critical Raw Materials Act (CRMA), which sets ambitious 2030 benchmarks for domestic extraction, processing, and recycling; the REACH regulation, which governs the use of chemical substances like cobalt; and the Dual-Use Export Control Regulation (2021/821), which manages the export of sensitive technologies including high-performance magnets.

This regulatory suite is complemented by the European Defence Industrial Strategy (EDIS) and funding instruments like the European Defence Fund (EDF) and the European Defence Industry Programme (EDIP), which aim to bolster the European Defence Technological and Industrial Base (EDTIB) and encourage intra-EU sourcing. While the EU's legislative framework is less defense-specific than that of the US, its overarching goal is to mitigate geopolitical supply risks and foster a resilient domestic value chain for materials critical to defense, aerospace, and the green transition. For technology providers in the waste treatment and material recovery sector, these regulations—particularly the CRMA's aggressive recycling targets—create a significant market opportunity by establishing both a regulatory mandate and a strategic premium for recycled rare earth materials sourced within the Union.

### European Critical Raw Materials Act

Adopted by the Council in March 2024 and entering into force in May 2024, the European Critical Raw Materials Act (CRMA), designated as Regulation (EU) 2024/1252, is the cornerstone of the EU's strategy to secure a sustainable supply of materials essential for its strategic sectors, including defense and aerospace. The Act explicitly identifies "rare earth elements for permanent magnets (Nd, Pr, Tb, Dy, Gd, Sm, and Ce)" as "strategic raw materials," placing them at the center of EU industrial policy.

The CRMA establishes clear quantitative benchmarks to be achieved by 2030 to reduce the Union's external dependencies:

- EU Extraction: At least 10% of the EU's annual consumption from domestic extraction.

- EU Processing: At least 40% of the EU's annual consumption from domestic processing.

- EU Recycling: At least 25% of the EU's annual consumption from domestic recycling.

- Diversification: No more than 65% of the Union's annual consumption of each strategic raw material at any processing stage should come from a single third country.

To support these goals, the CRMA introduces "Strategic Projects" that can benefit from streamlined and predictable permitting timelines (e.g., 15 months for processing and recycling projects) and support in accessing finance.

For permanent magnets specifically, the CRMA introduces circularity measures. It mandates that certain products (e.g., MRI devices, motor vehicles, industrial robots) bear a label indicating the presence and type of permanent magnets (NdFeB, SmCo, etc.). Furthermore, it requires a "data carrier" (akin to a product passport) with detailed information on the magnet's composition to aid recyclers. By May 2027, companies must publicly disclose the share of recycled content (Nd, Dy, Pr, Tb, B, Sm, Ni, Co) in their magnets, and the Commission is empowered to set minimum recycled content shares by 31 December 2031. Crucially, products primarily designed for defense or space applications are explicitly exempt from these labeling and recycled content disclosure requirements.

### REACH Regulation and Rare Earth Substances

The EU's regulation on the Registration, Evaluation, Authorisation and Restriction of Chemicals (REACH), Regulation (EC) No 1907/2006, governs the use of chemical substances and their potential impacts on human health and the environment. This framework intersects with the high-temperature magnet supply chain, particularly concerning cobalt, a key constituent of SmCo magnets.

Cobalt and its compounds have been under scrutiny within REACH, with some being placed on the Candidate List of substances of very high concern (SVHC). This can trigger obligations for suppliers and may lead to future restrictions. The CRMA acknowledges this intersection, stating that the Commission will consider the "specific operational realities of the extractive, recycling and processing sectors" in any revision of REACH to ensure policy coherence. The regulatory landscape also includes restrictions on other processing agents like boric acid and rules on workplace exposure, which affect magnet manufacturing processes. For defense applications, compliance with REACH is mandatory, and any future restrictions on cobalt or rare earth compounds could significantly impact the availability and cost of qualified magnetic materials.

### Dual Use Export Controls

The export of high-performance permanent magnets from the EU is governed by Regulation (EU) 2021/821, which establishes a Union-wide regime for the control of "dual-use items"—goods, software, and technology that can be used for both civil and military purposes. An export authorisation is required for any items listed in Annex I of the regulation.

This regulation is directly relevant to high-performance samarium-cobalt (SmCo) and high-coercivity neodymium-iron-boron (NdFeB) magnets, which have critical applications in military systems like guided munitions, drones, and aerospace actuators. The control lists can be based on technical parameters, such as a magnet's maximum energy product (BHmax), which determines its strength. The Commission is empowered to update these control lists via delegated acts to reflect changes in international export control regimes like the Wassenaar Arrangement. Exporters of controlled magnets must navigate a complex licensing process, which may require individual, per-order authorizations and detailed end-use statements, and maintain detailed records for at least five years.

### EU Defence Procurement Directive

Directive 2009/81/EC sets out specific EU-wide rules for the procurement of arms, munitions, and sensitive security equipment. A key objective of the directive is to balance the principles of the internal market with the security interests of Member States, including the security of supply. The directive allows contracting authorities to impose requirements related to security of supply in tender documents, which can be used to favor more resilient and local supply chains.

While the directive aims to create a more open and competitive European defense market, Member States frequently invoke Article 346 of the Treaty on the Functioning of the European Union (TFEU) to exempt certain defense procurements from EU rules, citing the protection of essential national security interests. The European Defence Industrial Strategy (EDIS) and related instruments like EDIRPA aim to encourage more coordinated procurement and strengthen the application of EU rules to bolster the European Defence Technological and Industrial Base (EDTIB). As part of the RESourceEU action plan, the Commission is exploring revisions to the Defence Procurement Directive to include incentives for diversifying critical raw material supplies.

### EU Defence Industrial Strategy

The EU's industrial strategy for defense is centered on achieving "strategic autonomy" and de-risking from geopolitical dependencies, particularly on China for critical raw materials and components. This strategy is being implemented through a combination of policy initiatives and funding instruments.

The European Defence Industrial Strategy (EDIS) and the European Defence Industry Programme (EDIP) aim to enhance the readiness and resilience of the EDTIB. A key proposal invites Member States to procure at least 50% of their defense investments within the EU by 2030, rising to 60% by 2035. Instruments like the European Defence Industry Reinforcement through common Procurement Act (EDIRPA), with a budget of €300 million until 2025, introduce strict sourcing requirements. For a project to receive support, contractors must be established in the EU and not controlled by a third country, and the cost of components originating in the Union must be no lower than 65% of the end product's value.

The European Defence Fund (EDF), with a budget of over €1 billion per year (2021-2027), funds collaborative research and development to foster innovation within the EDTIB. These programs are designed to work in synergy with the CRMA, leveraging defense spending to create a stable demand for domestically produced critical materials like rare earth magnets.

### European Rare Earth Magnet Production Capacity

The EU's domestic production capacity for high-performance rare earth magnets is limited, representing a significant vulnerability. Current European production is estimated at around 1,000 tonnes per year, compared to approximately 16,000 tonnes of magnets imported from China annually. This means the EU is over 90% dependent on China for these critical components.

The key established producer in Europe is Vacuumschmelze (VAC) in Germany, which has been making SmCo magnets since 1973 and NdFeB magnets since 1985. Other notable players include Less Common Metals (LCM) in the UK and a Shin-Etsu facility in Germany. However, the landscape is evolving. The European Raw Materials Alliance (ERMA) has identified 14 "mine-to-magnet" projects with a potential investment volume of €1.7 billion, aiming to recapture a significant share of global production. New entrants are also emerging, such as Neo Performance Materials' commercial magnet plant in Estonia and recycling initiatives by companies like Solvay and Carester in France. These recyclers could contribute an estimated 3,800 tonnes of magnet production in the coming years if sufficient feedstock is available.

### Comparison EU vs US Regulatory Approaches

The EU and the US share the strategic goal of securing rare earth magnet supply chains but employ different primary regulatory mechanisms. The US approach is more direct and defense-focused, while the EU's is broader, more integrated into industrial policy, and relies more on market incentives and benchmarks.

The table below outlines the key differences:

| Feature | EU Approach | US Approach |
| --- | --- | --- |
| Primary Instrument | Critical Raw Materials Act (CRMA): Sets EU-wide benchmarks for domestic extraction (10%), processing (40%), and recycling (25%) by 2030. | DFARS 252.225-7052: Direct procurement prohibition on SmCo and NdFeB magnets and alloys from "covered countries" (China, Russia, etc.). |
| Core Mechanism | Industrial Policy and Market Shaping: Uses targets, streamlined permitting for "Strategic Projects," and future recycled-content mandates to build domestic capacity. | Procurement Prohibition: Directly bans the acquisition of non-compliant materials for Department of Defense contracts, creating a protected domestic market. |
| Timeline | 2030 Benchmarks: Long-term capacity-building goals. Specific mandates like minimum recycled content are phased in by 2031. | January 1, 2027 Deadline: Hard deadline for the expanded prohibition to cover the entire supply chain from mine to magnet. |
| Recycling | Central Pillar: 25% recycling capacity target by 2030; upcoming restrictions on scrap exports to retain feedstock within the EU. | Specific Exemption: Allows the use of NdFeB magnets made from recycled material if the milling and sintering steps occur in the US. |
| Funding Approach | Coordination of Funds: Relies on leveraging existing EU funds (EDF, InvestEU) and national programs via a CRM financing hub. No new central fund was created by CRMA. | Direct Investment: The DoD has made direct investments (e.g., over $439 million) and equity stakes (e.g., MP Materials) via the Defense Production Act (DPA). |
| Defense Specific Rules | Exemptions: Defense and space products are exempt from CRMA's magnet labeling and recycled content disclosure rules. Sourcing rules are in separate defense programs (e.g., EDIRPA). | Core Focus: The DFARS rule is exclusively for defense procurement and imposes the strictest sourcing requirements on this sector. |

### Implications for Algamol and Waste Treatment

The EU's comprehensive regulatory framework, particularly the CRMA, creates a fertile market for advanced material recovery and wastewater treatment technologies like Algamol. The key opportunities are driven by several interconnected factors:

- Mandated Recycling Capacity: The CRMA's benchmark for the EU to have the capacity to recycle 25% of its annual strategic raw material consumption by 2030 is a powerful market signal. With current REE recycling rates below 1%, achieving this target requires massive investment in new technologies and facilities, creating demand for efficient and environmentally sound recovery processes.

- Restrictions on Scrap Exports: The Commission's plan to propose restrictions on the export of permanent magnet scrap and waste by Q2 2026 will trap valuable feedstock within the EU. This ensures that domestic recyclers and technology providers will have access to the raw materials needed to build a circular economy, turning a waste stream into a strategic asset.

- Future Minimum Recycled Content: The CRMA empowers the Commission to set minimum recycled content shares for elements like neodymium, dysprosium, and samarium in new permanent magnets by 2031. This will create direct, legally binding demand for high-purity recycled materials that technologies like Algamol could recover from machining sludge and other waste streams.

- Strategic Value Premium: Beyond commodity value, domestically recycled and recovered rare earths will carry a "strategic premium." For defense contractors and other critical industries, using materials recovered within the EU contributes directly to the strategic autonomy and supply chain resilience goals of the CRMA and EDIS. This can be a significant advantage in public procurement, where resilience and security of supply are increasingly important criteria.

- REACH Compliance: As regulations like REACH place increasing scrutiny on substances like cobalt, recovering these materials from waste streams for reuse becomes more economically and regulatorily attractive than disposal or sourcing primary materials from regions with less stringent environmental oversight.

Together, these drivers create a compelling business case for deploying wastewater and material recovery solutions at magnet manufacturing and finishing facilities across the EU. The CRMA's targets transform waste treatment from a cost center into a strategic enabler for the entire European defense and high-tech industrial base.

## Chapter 10 EEC Permag facility deployment model

### Algamol Deployment Model EEC Permag SmCo Magnet Facility Landisville PA

### Summary

Electron Energy Corporation (EEC) is a rare-earth permanent magnet manufacturer headquartered in Landisville, Pennsylvania, with a second listed Lancaster, Pennsylvania address at 300 Running Pump Road, and it has operated since 1970. EEC’s product focus includes samarium-cobalt (SmCo) magnets and assemblies that serve high-reliability aerospace and defense applications, including publicly referenced programs such as JSF, F-18, Patriot, Predator, and Navy nuclear programs. EEC describes itself as vertically integrated and documents an in-house SmCo manufacturing chain that includes induction melting of elemental inputs, powder milling to ~3 µm, pressing (axial/transverse/isostatic), sinter/solution/aging heat treatments, and brittle-magnet machining steps such as grinding, slicing, and wire EDM.

From a wastewater and pretreatment perspective, the relevant local context in Lancaster County includes industrial-user permitting and “local limits” under a POTW pretreatment program (e.g., Lancaster Area Sewer Authority’s program), which is designed to prevent pollutants that could interfere with the POTW or contaminate biosolids. However, the provided EEC-facing sources do not disclose site-specific wastewater volumes, coolant chemistries, or quantitative scrap/waste generation metrics, which constrains a fully parameterized ROI model and necessitates a measurement-first pilot plan for any on-site treatment and metals recovery concept.

### Facility profile

EEC was founded in 1970 and is headquartered in Landisville, PA, with a primary listed address of 924 Links Avenue, Landisville, PA 17538, and an additional Lancaster, PA address at 300 Running Pump Road, Lancaster, PA 17603. The company describes itself as a leading producer of rare earth magnets specializing in SmCo magnet manufacturing and also lists NdFeB and magnet assemblies among its specialties. EEC’s defense relevance is supported both by its industry positioning (defense and aerospace markets) and by explicit examples of programs and platforms served in public-facing descriptions (e.g., JSF, F-18, Patriot, Predator, Navy nuclear programs).

EEC facility footprint can be triangulated from multiple company materials that describe a 40,000 sq. ft. facility and the addition of a new ~45,000 sq. ft. facility to house magnet finishing and assembly operations, yielding a combined manufacturing footprint on the order of ~85,000 sq. ft. EEC’s current workforce size is presented in public sources in the ~130–150 employee range, indicating a mid-sized specialty manufacturer in a highly regulated supply chain.

#### Ownership and corporate relationships

EEC joined Magnetic Holdings, LLC in 2024, and Magnetic Holdings was subsequently rebranded as Permag, which also includes Dexter Magnetic Technologies and Magnetic Component Engineering (MCE). Separately, an investor profile notes that Tinicum acquired a controlling interest in Permag’s parent company (Robert Family Holdings, LLC) in January 2021 and that the parent was renamed TSL Engineered Products, LLC.

### SmCo manufacturing operations

EEC explicitly documents an in-house SmCo alloy and magnet production pathway beginning with induction melting, in which it purchases pure raw materials (Sm, Co, Fe, Cu, and Zr) for in-house production of SmCo 1:5 and 2:17 alloys and alloys these metals in an inert gas using an induction melting furnace. EEC also documents that it processes crushed alloy powder via ball or jet milling to reduce average particle size to approximately 3 microns, which is a key input specification for subsequent pressing and sintering steps. Green compacts are formed using axial, transverse, or isostatic presses, aligning with standard powder-metallurgy approaches used for high-performance rare-earth permanent magnets.

EEC describes densification and magnetic-property development via sinter, solution, and aging processes conducted under vacuum or inert gas, which corresponds to the canonical Sm2Co17 (“2:17”) processing route requiring solution/aging to develop high coercivity and thermal stability. Downstream, EEC notes that sintered rare earth magnets are brittle and are machined to final dimensions using grinding, slicing, or wire EDM, and it also lists facility machining capabilities including WEDM, Ram EDM, CNC machining, grinders, and slicers. EEC further states that magnetization, stabilization, and testing are performed to satisfy customer requirements, implying that magnetization and verification are integrated into its finishing and quality flow for defense-grade deliverables.

#### Finished parts and assemblies

EEC positions itself as a supplier not only of magnets, but also of assemblies (e.g., rotor assemblies and traveling wave tube assemblies), which is consistent with delivering finished-to-print components and subassemblies into defense/aerospace system BOMs rather than only shipping sintered blocks.

### Products and grades

EEC’s public materials emphasize multiple SmCo offerings, including a “34 Grade” SmCo and an “Ultra-High Temperature SmCo 2:17” family with operating temperatures up to 550°C. EEC also describes ultra-high temperature SmCo stability in the 400°C to 550°C range on its SmCo product materials, reinforcing the high-temperature defense niche for Sm2Co17-based magnet solutions.

The table below compiles the numeric magnetic-property anchors available in EEC’s 34 Grade sell sheet.

| Parameter | EEC 2:17-34 (Sintered Sm2Co17) | Source |
| --- | --- | --- |
| (BH)max | 34 MGOe | EEC 34 Grade sell sheet |
| Br | 11.9 kG | EEC 34 Grade sell sheet |
| Hc | 11.1 kOe | EEC 34 Grade sell sheet |
| iHc (min) | 18 kOe | EEC 34 Grade sell sheet |
| Max operating temperature | 300°C | EEC 34 Grade sell sheet |
| DFARS status | “DFARS compliant” | EEC 34 Grade sell sheet |

EEC’s own stated application set for SmCo includes high-speed motors, generators and actuators, microwave signal amplification, and aircraft/aerospace assemblies, which map directly onto defense-relevant subsystems such as precision actuation, high-temperature motors/generators, and RF/microwave assemblies used in radar/EW architectures.

### Machining operations and waste streams

EEC identifies multiple machining methods used for rare-earth magnets and associated hardware, including grinding, slicing, wire EDM (WEDM), and other precision machining equipment, and it explicitly notes that sintered rare earth magnets are brittle and therefore machined via these techniques to reach final dimensions. EEC also advertises in-house capabilities for wire EDM and precision machining when describing magnet assembly offerings, supporting the view that EEC performs significant downstream finishing internally rather than outsourcing all final machining.

The available sources do not provide quantitative waste fractions (kerf loss, grinding sludge mass, scrap rates) or coolant chemistry details for EEC’s operations, so a deployment model must treat waste characterization as an early-stage data-gathering activity tied to the site’s documented machining and finishing process steps.

#### Waste stream mapping for pilot scoping

A practical way to scope wastewater-relevant streams without assuming unreported values is to map them to EEC’s stated finishing operations (machining, magnetizing, testing, and shipping) and machining modalities (grinding, slicing, WEDM). For example, a pilot can explicitly segment sample points associated with (1) grinding operations, (2) EDM operations, and (3) post-machining magnetization/testing areas, because these are explicitly present in EEC’s documented capabilities and finishing operations.

### Wastewater characterization

EEC’s sources establish that the company performs brittle-magnet machining (grinding, slicing, and wire EDM) and that it conducts magnetization, stabilization, and testing as part of its manufacturing flow, which are operations that typically drive industrial wastewater and residual streams that fall under local pretreatment programs when discharged to a POTW. For Lancaster County, LASA describes an approved Pretreatment Program, states that it issues Industrial User Permits for significant industrial users, and explains that “Local Limits” apply to industrial and other non-domestic facilities discharging to the treatment facility.

At a permitting-interface level, publicly available Pennsylvania wastewater permitting resources emphasize the role of NPDES permits and associated documentation for identifying treatment processes and discharge limits for local facilities, and they direct users to the linked permit documents for specific POTW discharge limits. As an example of the kind of constraints that appear in local wastewater permits in Lancaster County, an NPDES permit for a Lancaster County WWTP shows a pH effluent range of 6.0 to 9.0 and provides design-capacity context, illustrating the type of effluent constraints that industrial pretreatment programs are designed to protect.

### Local regulatory context

Because EEC’s manufacturing footprint is located in Lancaster County (Landisville and Lancaster addresses), an on-site wastewater treatment and recovery deployment needs to be designed to align with a POTW pretreatment framework and any applicable industrial-user permitting requirements. LASA’s pretreatment program description emphasizes preventing the introduction of pollutants that could interfere with the POTW or contaminate resulting biosolids, which is directly relevant to any process that could generate metal-bearing residuals or sludges that would otherwise be discharged or hauled offsite.

Pennsylvania DEP materials emphasize that permitted-facility reports and individual NPDES permit documents are the mechanism for obtaining permit details, and DEP guidance highlights that permit conditions and discharge limits for industrial wastewater discharges in Landisville are influenced by state procedures and by the local POTW.

### System concept

Any algae-based or bio-based treatment concept for EEC should be framed around EEC’s documented process reality, namely that the facility performs (a) alloying and sintered magnet production steps and (b) downstream machining, magnetization, stabilization, and testing in-house, all within a regulated defense supply chain with ITAR and quality-management certifications. EEC states it is ITAR compliant and DDTC registered and certified to AS9100D and ISO 9001:2015, which implies that any deployed wastewater system would need to be compatible with controlled-access production areas and auditable quality and compliance systems.

Within those constraints, a deployment concept can be structured as a modular add-on to the existing finishing operations rather than a modification to EEC’s alloy/powder/sinter process flow, because EEC’s publicly described finishing operations already include machining and magnetization/testing, which are the most plausible touchpoints for wastewater-relevant streams at a magnet manufacturer.

### Financial model

A rigorous facility-specific financial model cannot be completed from the provided sources because EEC’s public materials do not disclose machining-waste volumes, coolant consumption, wastewater flow rates, or current disposal and treatment costs, even though they do document the presence of machining operations and finishing activities that would create those costs. A defensible approach is therefore to define a parametric model that uses measurable site inputs—such as hours of grinding/WEDM operation, mass of collected solids/residuals, and any applicable industrial-user permit fees or surcharges—rather than attempting to infer costs from unreported values.

To remain aligned with local regulatory reality, the model should also include a compliance cost/benefit component based on industrial-user permitting and local limits under the applicable pretreatment program, because LASA explicitly regulates significant industrial users and sets local limits for industrial dischargers to the POTW.

### Strategic value

EEC’s strategic value to defense supply chains is rooted in its position as a U.S. SmCo producer with ITAR and DFARS compliance claims and in its vertically integrated capability for SmCo alloy and magnet production, including in-house production of SmCo 1:5 and 2:17 alloys from elemental feedstocks. Public materials also emphasize Permag/EEC’s investment in expanding alloying, pressing, and fabrication equipment and “more than doubling” production capacity, which indicates a growth trajectory that can increase the volume and operational importance of downstream finishing activities over time.

For an on-site wastewater treatment and recovery system provider, the strategic engagement thesis is that a defense-critical, vertically integrated SmCo producer with in-house machining and magnetization/testing presents an opportunity to (1) reduce compliance risk under industrial pretreatment/local limits and (2) create a traceable pathway for managing metal-bearing residuals within a controlled, auditable environment consistent with ITAR and AS9100-quality expectations.

### Pilot proposal

A pilot program at EEC should be scoped first around (a) EEC’s documented finishing operations (machining, magnetization/stabilization/testing) and (b) the local pretreatment program and industrial-user permitting framework, because these are the best-supported operational and regulatory anchors in the provided sources. The pilot should also explicitly include a sampling and analysis plan that links any measured wastewaters or residuals to the specific machining modalities EEC lists (e.g., grinders and WEDM), because these modalities are directly stated in EEC’s manufacturing capability descriptions.

To ensure the pilot is implementable, the engagement should incorporate early coordination with the relevant local pretreatment authority, since LASA’s program issues Industrial User Permits for significant industrial users and enforces local limits on industrial discharges to its POTW.

## Chapter 11 Competitive technology analysis

### Conventional Wastewater Treatment Technologies for Rare Earth and Cobalt Recovery in Defense Magnet Machining

### Overview

Defense magnet machining and finishing operations can generate wastewater and metal-bearing process liquids that contain rare earth elements (REEs) and, in some cases, cobalt (Co), creating both a compliance burden and a potential recovery opportunity for critical materials. Conventional treatment and recovery approaches for these aqueous streams commonly include chemical precipitation, membrane filtration (typically as pre-treatment and/or concentration), ion exchange, solvent extraction (usually after concentration), and electrochemical approaches for selective removal or direct recovery in certain cases.

Across the conventional methods, the same engineering trade-off appears repeatedly: the most operationally straightforward approaches (notably precipitation) can be cost-effective at large volumes but tend to be non-selective and produce significant sludge requiring downstream handling, whereas the more selective approaches (ion exchange, some membranes, solvent extraction) can be more complex and/or cost-intensive and are often more compatible with lower-flow, cleaner, or more concentrated streams rather than oily emulsions or mixed-metal slurries.

### Chemical precipitation

Chemical precipitation is widely described as simple, fast, and low cost, and is frequently cited for high removal performance when conditions are well controlled, making it an attractive first-line treatment for metal-bearing waters and leachates.

#### Mechanisms and pH windows

Precipitation of REEs occurs when sparingly soluble compounds (e.g., hydroxides, fluorides, phosphates, oxalates, carbonates) form and exceed solubility, causing solids that can be separated by settling/filtration.

A common staged-precipitation concept is to drive precipitation in distinct pH windows where different metals remove preferentially, with one summary describing Fe removal at pH ~3–4, REE precipitation at pH ~8–9, and Co precipitation at pH ~9–11.

For oxalate precipitation specifically, multiple sources emphasize that oxalate is among the most effective precipitants for REEs and can work at low pH, with one thermodynamic study explicitly describing oxalate as the “most effective precipitant” with “outstanding precipitation behavior” even at low pH.

Detailed oxalate operating windows and dosing requirements are reported in experimental work: an “optimal pH of 1.25 to 2” is described as tending to maximize RE recovery and precipitate purity, and a pH of 1.5–2 with reaction time >2 h and oxalic acid dosage of 30–40 g/L is described as optimizing REE recovery at ~95% to nearly 100% for individual REEs in one study.

The same oxalate-focused work provides quantitative dose–response behavior at a moderate REE feed concentration (440 mg/L), noting that 10 g/L oxalic acid is a minimum capable of recovery while higher doses (≥20 g/L) significantly increase recovery, and that at pH 1.5 and 120 min, recovery of 95% was achieved at 20 g/L and approached 100% at 30 g/L.

In addition to oxalate, precipitation can be carried out via carbonate and double-sulfate routes depending on upstream chemistry; in one comparative precipitation study (for Nd-Fe-B leachate), oxalic-acid precipitation achieved total REE precipitation efficiency of 99.94% with total recovery efficiency of 99.44%, while sodium carbonate precipitation achieved 99.93% precipitation efficiency and 99.12% overall recovery after transformation and washing.

For double-sulfate precipitation, the same study reports optimum conditions including a sodium sulfate amount ratio of 1.3:1.0, a reaction temperature of 80 °C, and reaction time of 1 h, with individual precipitation efficiencies (Ce, Pr, Nd, Gd) around 98.7–98.8% and total REE precipitation efficiency of 98.75%.

#### Recovery performance and impurity co-precipitation

High recovery via oxalate precipitation is repeatedly reported, including a hydrometallurgical study where precipitation of Nd, Pr, Dy, Gd and Ho reached ~99% using only 1.8× the stoichiometric amount of oxalic acid (following HCl leaching).

However, precipitation-based recovery is also frequently described as vulnerable to impurity effects and non-selective co-precipitation, with one study reporting that while >99% REE precipitation could be achieved with oxalate, product purity was “significantly affected by impurities like Fe and Co,” and providing an example where Fe and Co content increased to 1.28% and 0.24% in the product, reducing final purity.

The same work notes a mechanistic cause: oxalate ions can complex strongly with dissolved iron, leading to inefficient precipitant utilization and low-purity REE products.

Some precipitation approaches are explicitly positioned to mitigate impurity impacts by changing the chemistry: for REE-enriched sulfate solutions, double-sulfate precipitation is described as achieving effective separation with recovery less affected by iron and cobalt impurities, although high Na2SO4 levels (8× stoichiometric) may be required to achieve >98% REE precipitation, particularly for heavy REEs.

Sequential pH adjustment has been used to exploit “well defined separation of removal pH ranges,” with one study adjusting sequentially between pH 3.0 and 8.0 (including steps at pH 4.0, 5.0, and 8.0) and reporting efficient recovery due to these separations; the same study notes that flocculant can increase overall recovery but decrease the REE weight percent in the precipitate (i.e., a grade–recovery trade-off).

#### Sludge and downstream processing

Multiple reviews describe precipitation as potentially producing impure precipitates and additional waste, with “large amounts of slurry” or “large sludge volumes” highlighted as recurring disadvantages that require dewatering, disposal, or further processing of mixed precipitates.

A further limitation is that precipitation can create secondary sludge even when used as part of otherwise high-efficiency hydrometallurgical recovery flowsheets, emphasizing that precipitation can shift the problem from dissolved metals to solid waste management if solids are not valorized into recoverable products.

### Membrane filtration

Membrane filtration is often framed as a selective separation tool that can operate continuously and avoid chemical addition, but is also widely described as subject to fouling, maintenance demands, and potentially higher costs than precipitation for comparable bulk removal—especially when streams contain organic/inorganic foulants.

#### Roles in a treatment train

Ultrafiltration (UF) is commonly positioned as pre-treatment for emulsion breaking and solids removal, which is particularly relevant to industrial streams that contain fine particulates or emulsified organics.

Nanofiltration (NF) is commonly presented as a route for partial rejection and concentration of REEs (or other multivalent metals), producing a retentate that may require additional recovery steps.

#### Example performance metrics

Quantitative membrane performance is highly configuration- and chemistry-dependent, but some sources provide numeric examples. One review reports an MEUF/UF configuration (UP020 membrane) achieving 97% REE rejection under conditions including pH 3.5, 60 min contact time at 25 °C, 3 bar pressure, and SDS concentration of 8.0 mM.

The same review reports NF270 achieving >98% rejection at pH 1 in a tabulated summary, illustrating the high rejection possible in strongly acidic conditions for certain membranes and conditions.

#### Limitations

Membrane filtration is repeatedly described as prone to fouling and scaling that necessitate pre-treatment and periodic cleaning, with high operating costs attributed to fouling and maintenance in multiple sources.

In contexts involving radiation (relevant to some defense/nuclear-adjacent waste streams), polymeric membrane damage is explicitly cited as another potential operating risk and cost driver.

### Ion exchange

Ion exchange is frequently positioned as well suited to solutions containing traces of metal ions and as a route to higher selectivity when appropriate resin chemistry is chosen, including the potential for selective separation of individual REEs with selective REE resins.

#### Selectivity and operating considerations

Selectivity in ion exchange is described as dependent on resin choice and operating conditions, with one source noting that resins generally have better selectivity for ions with higher valence/charge and that selective separation of rare earths requires careful selection of resin characteristics (exchange groups, cross-linking, exchange capacity) and operating conditions.

While specific pH windows depend on resin chemistry and feed, one source notes cation exchangers “work at low pH (3–5),” indicating potential compatibility with moderately acidic streams when resin stability permits.

#### Constraints

Despite its selectivity advantages, ion exchange is repeatedly described as limited by resin costs and the need for regeneration, and as susceptible to fouling by organics, especially for complex industrial wastewaters.

### Solvent extraction

Solvent extraction is widely used in hydrometallurgy and is often described as a highly selective route for producing high-purity REE products, but is also commonly described as ill-suited for dilute wastewater because it relies on organic solvents and typically becomes more practical after upstream concentration or clarification.

#### Extractants and example operating configurations

Organophosphorus extractants such as D2EHPA, Cyanex grades, and TBP are frequently cited as extractants used for rare earth separations from chloride and nitrate media.

One study explicitly describes a solvent-extraction system used upstream of precipitation, using Elixore 205 diluent with D2EHPA and TBP as extractants at an organic-to-aqueous ratio of 1:1, and stripping the loaded organic with 6 mol/L nitric acid, illustrating the acid-intensive nature of stripping for some systems and the production of concentrated acidic strip solutions that require downstream recovery steps.

A separate review describes solvent extraction as already used for other acidic REE-bearing leachates and as applicable in a wide concentration range compared to lower-concentration methods such as membrane separation or ion exchange, consistent with its role as a workhorse for concentrated hydrometallurgical streams rather than dilute wastewater polishing.

#### Performance examples and dependencies

Solvent extraction performance can depend strongly on aqueous chemistry (e.g., nitrate concentration), with one review reporting that under an “optimum condition” including 0.1 mol/L D2EHPA and 4 mol/L HNO3 (and feed pH 4.6), Nd3+ extraction was near 92.9%, and that increasing NO3− ions from 2 mol/L to 3 mol/L enabled 99% Nd3+ transport in a membrane-transport context using extractant chemistry (highlighting the importance of nitrate-driven speciation and transport).

#### Constraints and waste streams

Solvent extraction is explicitly associated with organic solvent handling risks and potential VOC emissions and fire hazards, and is described as not suitable for dilute wastewater in one comparative summary, reinforcing its typical placement after pre-concentration rather than as a front-end wastewater treatment method.

### Electrochemical recovery

Electrochemical approaches (including electrowinning/electrodeposition and related electro-driven separations) are often described as capable of direct metal recovery without chemical addition, but also as challenged by energy demands and operational limitations for dilute streams and mixed matrices.

#### Examples and constraints for dilute feeds

A review of REE recovery methods reports that capacitive deionization achieved 36.5% Co removal efficiency at an initial cobalt concentration of 5 mg/L, illustrating both applicability to dilute metals and the potential limitation in single-pass removal depending on configuration and kinetics.

For electrochemical methods more broadly, electrode passivation and relatively high energy consumption are noted drawbacks for electrocoagulation, while scale-up challenges and energy consumption barriers are noted for industrial commercialization of some electrical-based methods.

#### Sludge considerations

Some electrochemical modes can reduce or avoid sludge formation relative to chemical precipitation in certain configurations, with electrodeposition described as depositing targeted species on a cathode surface and noting that sludge is not formed in that treatment mode, although other electrical treatments (e.g., electrocoagulation) can increase sludge production.

### Comparison

The table below summarizes the principal, source-supported “shape” of each conventional technology family as it is typically applied to REE/Co-bearing aqueous streams, emphasizing what each method produces (sludge, concentrate, eluate, or metal deposit) and the main operational constraints reported in the literature.

| Technology | Primary output | Strengths | Common constraints |
| --- | --- | --- | --- |
| Chemical precipitation | Metal-bearing sludge/slurry requiring solid–liquid separation | Simple and low CAPEX with high removal often cited (>95%) | Non-selective, mixed precipitates, large sludge volumes requiring further processing/disposal |
| UF and NF membranes | Clarified permeate and metal-rich retentate/concentrate | UF supports emulsion/solids removal; NF can concentrate REEs without chemical addition | Fouling and maintenance costs; may yield concentrate rather than final product; high CAPEX described in some summaries |
| Ion exchange | Loaded resin and a regenerant eluate (concentrate) | Can be selective and regenerable; suitable for trace ions and can separate individual REEs with selective resins | Expensive resins and regeneration; fouling by organics; limited to low concentrations in some summaries |
| Solvent extraction | Separated aqueous/organic phases and concentrated strip solutions | Highly selective and established for concentrated hydrometallurgy with high purity | Requires organic solvents (fire/VOC risks) and is described as not suitable for dilute wastewater |
| Electrochemical recovery | Metal deposits and/or treated water, depending on mode | Direct metal recovery without chemical additions in some approaches | High energy for dilute streams and fouling/operational limits noted; some modes face passivation/energy barriers |

### Practical implications

From the available evidence, precipitation remains the most “default” conventional approach for bulk removal and recovery of REEs because it is repeatedly described as simple and cost-effective and can achieve high recovery when optimized (including near-quantitative precipitation in some oxalate/carbonate/double-sulfate systems).

The major engineering drawback of precipitation for a recovery-oriented process is the consistent emphasis on non-selectivity and sludge generation, which can create a second-stage separation problem (purifying a mixed precipitate) and a disposal problem if the solids are not further processed into saleable products or refined concentrates.

Membrane filtration, particularly UF, is best supported as an enabling pre-treatment for solids and emulsion removal, while NF can serve as a concentration step with high reported rejection in some conditions (e.g., >98% in acidic conditions in one summary).

Ion exchange is best supported as a selective approach for lower-concentration solutions and for separations requiring higher purity, but resin cost and organic fouling are widely cited constraints that may be critical for complex industrial streams unless upstream clarification and organic removal are effective.

Solvent extraction is strongly supported as a high-selectivity approach for concentrated hydrometallurgical streams using organophosphorus extractants, but multiple sources explicitly flag poor fit for dilute wastewater due to solvent requirements and associated hazards, indicating that it is most appropriately evaluated as a downstream polishing/purification step after concentration (e.g., NF, evaporation, or resin eluate concentration).

Electrochemical recovery shows promise for direct metal recovery in some configurations, but the cited examples and summaries emphasize performance and energy challenges for dilute feeds and operational issues such as passivation, suggesting it is most plausibly deployed either on concentrated streams or for targeted ions where electrochemical selectivity is strong and deposits are readily recovered.

### Competitive Technology Analysis for Rare Earth and Cobalt Recovery from Magnet Machining Streams Part 2

### Biosorption and bioaccumulation

Biosorption and bioaccumulation rely on the migration of dissolved REEs from water to algal biomass surfaces and/or into cells, followed by a second step that transfers the concentrated REEs back into a recoverable solution (e.g., via desorption or extraction). In practice, this means performance is governed by surface functional groups (cell-wall polysaccharides/proteins, extracellular polymeric substances (EPS), and engineered/modified sites), aqueous speciation, and competing ions that occupy binding sites or alter charge balance.

Reported biosorption performance varies widely by organism, metal, and test conditions, but the literature provides quantitative anchors that are relevant to defense-magnet wastewater scenarios spanning trace (µg/L) through moderate (mg/L to hundreds mg/L) metal concentrations. For neodymium uptake by the microalga Chlorella vulgaris at pH 5, batch tests with an initial Nd concentration of 250 mg/L reported maximum Nd uptakes of 126.13 mg/g at 21°C and 157.40 mg/g at 35°C, with equilibrium reached within 30 minutes, and a Langmuir maximum capacity (qmax) of 188.68 mg/g. In another biological sorbent example, Physcomitrella patens demonstrated Nd sorption rates/capacities reported as ~0.74–0.75 mmol/g and showed strong pH dependence with maximum Nd biosorption around pH ~3 and near-constant performance up to pH 6. For cyanobacterial/EPS-derived sorbents, maximum adsorption capacities in the range of 89.12–138.2 mg/g were reported across strains, and EPS-extracted biomass retained substantial fractions of the untreated adsorption capacity (e.g., ~59% to ~84% depending on strain).

Biosorption frequently exhibits an “optimal window” in mildly acidic conditions for many REE systems, consistent with both adsorption site protonation behavior and REE speciation. A review focused on microorganisms and dilute REE feeds notes biosorption is commonly preferred at pH 3–6, with a frequently reported optimum around pH 5. Experimental work with the acidophilic red microalga Galdieria sulphuraria showed removal efficiency from solution increasing from below 9% at pH 2.5 to as high as 86% at pH 5.0 in mixed-metal (ppm-level) tests, and also cites prior work reporting >90% REE removal from a 5 ppm solution under highly acidic conditions (pH 1.5–2.5), indicating that both organism choice and nutrient/medium conditions can shift the usable pH window. In seaweed-based biosorption at trace concentrations, living Ulva sp. achieved removals up to 88% in 24 hours under optimized conditions (10 µg/L, salinity 15, dosage 5.0 g/L), and ≥90% removal at 96 hours with reduced biomass dosage (4.2 g/L).

Selectivity is a key uncertainty for algae-based processes when applied to mixed-metal machining coolants. For example, Galdieria sulphuraria biosorption data showed no particular selectivity towards REEs over Cu in mixed-metal tests, which is directly relevant to real-world coolants and rinse waters that often contain transition metals alongside REEs. Conversely, some literature argues for the potential of EPS and related biological materials as effective adsorbents for separation and recovery from wastewater, but without consistently reporting separation factors in realistic multicomponent streams. Where biosorption is used primarily as a concentration step, downstream recovery is commonly framed as desorption and/or conversion of the biomass to a denser, metal-rich intermediate. One seaweed study verified that REEs removed by Ulva sp. were mostly surface-bound and were easily desorbed with a 15-minute EDTA wash; it also notes that diluted acids and chelating agents can be efficient for regeneration and REE recovery from seaweed biomass. More generally, adsorption–desorption cycling and reversibility are described as key factors for environmentally friendly and cost-effective implementation, and EPS/biomass can be dried into a metal-enriched powder for further processing.

From an energy-and-waste perspective, biosorption is repeatedly positioned as a “green” alternative to conventional routes because it can avoid large volumes of chemical precipitation sludge and can operate at dilute concentrations where solvent extraction is inefficient. Biosorption has been described as efficient and cost-effective and as avoiding generation of chemical sludge in comparison with conventional methods. Multiple reviews contrast biological approaches with conventional hydrometallurgical/pyro/electrometallurgical recovery methods that can be energy-intensive and can generate secondary pollutants such as spent extractants/electrolytes.

### Comparison matrix

The table below provides a side-by-side comparison of six technology families as they relate to dilute-to-moderate REE and cobalt streams typical of machining coolants, rinses, and associated wastewater. Where the reviewed sources did not provide quantitative CAPEX/OPEX ranges, the table records that limitation and cites qualitative cost statements.

| Technology family | Removal efficiency evidence | Selectivity evidence | CAPEX and OPEX signal | TRL signal | Feed concentration fit | Key advantage | Key disadvantage |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chemical precipitation | Reported as high removal (e.g., >95%) in general descriptions, but commonly produces mixed precipitates. | Generally non-selective; overlapping precipitation windows can drive co-precipitation losses (e.g., REE with Al). | “Relatively low costs” and low energy use stated qualitatively; sludge can cause secondary pollution. | Commercially common for metals wastewater treatment and bulk removal. | Best for moderate-to-high metals where sludge handling is acceptable; not ideal for trace recovery value capture. | Simple, robust, high removal. | Large sludge volumes, low selectivity, downstream refining required. |
| Membranes (UF/NF/RO and membrane distillation) | UF/NF described as pre-treatment and concentration; DCMD example reports flux 7.0±0.5 L/m²·h and modeled yield of 193.2 g of 99% EuCl₃ per 1000 m³ AMD when integrated with adsorption/concentration. | NF can reject multivalent ions and has promise for REE separation; performance sensitive to fouling in practice. | High installation/maintenance costs and frequent replacement noted qualitatively; no universal $ ranges provided. | UF/NF used commercially for pre-treatment; advanced integrated schemes often bench/pilot in the cited studies. | Useful as front-end solids/emulsion removal and to concentrate dilute streams. | Produces a concentrate stream without immediate chemical sludge generation. | Fouling and maintenance burden; often still requires a downstream recovery step to make product. |
| Ion exchange and chelating resins | High retention can be achieved under optimized pH; examples include 97.78% retention at pH 2 on a cation resin (AG 50W-X4) and >99% retention at pH 7 on an anion resin (reported in batch tests). | Selectivity depends on resin functional groups; cation resins can show higher affinity for heavy REEs; chelation and EDTA elution can yield high HREE purity (e.g., 98.4% HREE in eluate under specific column conditions). | No direct CAPEX/OPEX; positioned as suitable for dilute feeds and “high selectivity” with manageable maintenance in qualitative summaries. | Resins are commercial products; specific REE fractionation flowsheets may be lab-to-pilot in literature demonstrations. | Strong fit for dilute-to-moderate feeds where long contact time and regeneration are feasible. | High selectivity potential and regenerability; can produce concentrated eluates for downstream precipitation/refining. | Slow processing at low concentrations can lead to prolonged batch operations (reported up to a month in one discussion). |
| Biosorption and bioaccumulation | Quantitative examples include: C. vulgaris Nd uptake up to 157.40 mg/g at pH 5 with equilibrium within 30 min; Ulva sp. removals up to 88% in 24 h and ≥90% at 96 h in trace solutions; EPS adsorption capacities 89.12–138.2 mg/g in single-metal tests. | Selectivity is variable; some systems show limited selectivity against competing ions (e.g., no particular selectivity of REEs over Cu for G. sulphuraria), while EPS is positioned as promising for separation without quantified separation factors in the cited excerpt. | Often framed as low-energy/low-cost and sustainable; avoids chemical sludge, but no system-level CAPEX/OPEX numbers provided in the reviewed excerpts. | Repeatedly described as needing further development toward practical deployment; positioned as lab-to-pilot with commercial potential. | Strong conceptual fit for dilute streams where solvent extraction is inefficient (<100 ppm). | Potentially lower energy and lower secondary chemical waste; yields metal-enriched biomass for downstream recovery. | Early TRL; sensitive to pH/speciation and competing ions; longer contact times in some systems and biomass handling required. |
| Solvent extraction | High extraction is achievable at high concentrations (e.g., ~99% extraction of Nd/Tb/Dy using 0.8 M D2EHPA from a 1500 mg/L-each synthetic feed; D2EHPA equilibrium reached after ~10 min in one study). | Single-stage separation factors can be low for adjacent REEs, requiring many stages (e.g., 40–50 stages for separation factor ~2 noted in one discussion). | Characterized as having high capital and operating costs and operational challenges (emulsification, flooding, loading limits); quantitative $ not provided in cited text. | Commercially used and often the preferred industrial process for REE production and separations. | Best fit for concentrated leachates; limited applicability when REEs are dilute (<100 ppm). | Mature, scalable, continuous, high throughput at concentration. | Poor economics for dilute streams; solvent loss and secondary waste concerns; complex stage count for fine separations. |
| Electrochemical and electro-membrane | Electrodialysis examples include 99.52% Sc3+ removal from 20 ppm at 3 V and extraction/recovery values reported for Nd/Dy/Tb (e.g., Tb 90% extracted, 55% recovered) in a cited dataset; chelation-assisted electrodialysis achieved Dy yield 82% and purity 86% with energy sensitivity to voltage (SEC 48 kWh/mol at 14 V vs 14 kWh/mol at 4 V). | Selectivity can be tuned with chelants (EDTA) and membrane configuration; a separation factor of 56 was reported for Dy vs Nd/Pr under stated conditions. | Energy use and scale-up constraints are repeatedly emphasized; quantitative system cost not provided in the cited excerpts. | Many approaches are lab-to-pilot, with limited large-scale application experience in wastewater contexts. | More viable when a concentrate is available or when architectures overcome mass-transfer limits in dilute feeds. | Potential for selective recovery without chemical sludge; continuous operation possible. | Energy intensity and mass-transfer limits in dilute streams; REE electrodeposition is not feasible in aqueous electrolytes due to very negative standard potentials. |

### Positioning

Algamol’s “algae-first” positioning is best supported when the wastewater is (a) too dilute for solvent extraction to be efficient, and/or (b) burdened by complex organic matrices where precipitation would create large mixed sludges and lose value in low-grade solids. A key technical rationale is that conventional organic-solvent extraction has limited applicability when REE concentrations are below ~100 ppm, which is consistent with many rinse waters and diluted coolant effluents (even if grinding sludge itself is solids-rich). In those dilute regimes, biosorption is repeatedly presented as a greener and cheaper tool to remove REEs from aqueous solutions and as having advantages such as lower sludge volumes and the potential for element recovery.

A practical differentiation thesis for Algamol is to focus less on “replacing” precipitation (which is described as simple, low-cost, and high-removal but non-selective) and more on building a hybrid, compliance-ready recovery train where algae-based capture produces a security-relevant, metal-enriched intermediate while minimizing secondary chemical sludge. In this framing, biosorption acts as a selective or semi-selective capture/concentration step, followed by desorption into a smaller-volume eluate suitable for conventional refining (e.g., oxalate precipitation or other downstream hydromet steps).

A recommended “default” hybrid architecture consistent with the reviewed evidence is:

- UF as an upfront step for emulsion breaking and solids removal in oily machining streams, reducing downstream fouling and biomass poisoning risk.

- Biosorption/bioaccumulation operated in a pH regime compatible with strong REE uptake (many studies use or cite pH ~5 as effective), while explicitly controlling pH because REE complexation changes across pH 4–8 and above pH 8 due to organic and carbonate complexes.

- A downstream desorption/regeneration step (e.g., EDTA wash or diluted acid) to transfer REEs from biomass into a smaller-volume solution, enabling a conventional polishing step if needed.

This positioning also helps manage a known weakness: biosorbents may not be selective for REEs over key transition metals (e.g., Cu) in real mixed wastewaters, which implies either upstream impurity control or downstream fractionation is required. Complementary “precision” capture technologies—such as lanmodulin-based supports that can separate binary Nd/Y or Nd/Dy pairs—exist as a longer-term pathway to true REE fractionation, but scale-up may be constrained by biomolecule production and purification considerations discussed in the literature.

### Recommendations

Technology development priorities should be anchored in the demonstrated sensitivities of biosorption systems and in defensible test protocols that map to real machining wastewater complexity. The first recommendation is to generate performance curves across realistic pH and competing-ion conditions, because pH changes both microbial surface charge and metal speciation, and low pH can electrostatically repel cations while higher pH exposes deprotonated binding sites that enhance adsorption.

These pH studies should explicitly track REE complexation behavior (organic complexes at pH 4–8 and carbonate complexes above pH 8), since speciation shifts can change both uptake and desorption behavior even at constant total metal concentration.

The second recommendation is to prioritize experiments with both NdFeB-relevant REE and SmCo-relevant transition metals (notably cobalt) in the same matrix, and to quantify selectivity tradeoffs rather than relying on single-metal capacity tests. This is supported by evidence that some biosorbents show no selectivity for REEs over Cu in mixed feeds, and by separate demonstrations that algae can remove cobalt efficiently under specific conditions (e.g., 87.8% removal from a 50 mg/L Co feed in 60 minutes at pH 5).

The third recommendation is to design and validate a closed-loop regeneration approach early, because adsorption–desorption cycling and reversibility are flagged as central to cost-effective implementation. EDTA desorption (15 minutes in one Ulva study) and regeneration with diluted acids are concrete starting points for bench-scale recycle-loop demonstrations, and EPS/biomass drying into a metal-enriched powder provides an alternative pathway to a solid intermediate for downstream processing.

Finally, any pilot should be built as an integrated train rather than a “single-box” biology demo, because upstream solids/emulsion management and downstream recovery chemistry are critical determinants of operability. The reviewed literature explicitly supports UF as a pre-treatment for emulsion breaking and solids removal, and it repeatedly emphasizes that further research and optimization are necessary to determine feasibility and efficiency of EPS/biomass adsorption-based recovery in practical deployment contexts.

## Chapter 12 Pitch deck framework

### Algamol Pitch Deck Framework Rare Earth Recovery from Defense Magnet Machining Wastewater

### Deck overview

This framework is designed as a slide-by-slide structure for a 16-slide investor and customer deck, plus appendix slides, focused on (1) defense-driven demand for high-performance rare-earth magnets, (2) value leakage from machining waste and coolant slurries, and (3) Algamol’s positioning as a wastewater treatment and metals-recovery enabler in a tightening provenance/compliance environment.

### Slide 1

Slide title: Algamol

Key message (1 sentence): Algamol enables recovery of rare earth metals from wastewater, turning a defense-critical waste stream into a domestic supply and compliance advantage.

Supporting data points to include:

- “supports the recovery of rare earth metals from wastewater.”

- DoD emphasis on keeping “critical nodes” inside the U.S. for secure magnet supply.

- DFARS expanded prohibition effective January 1, 2027 for covered magnets/materials from covered countries.

Visual suggestion: A simple graphic showing “wastewater → recovered metals concentrate → compliant domestic value chain.”

### Slide 2

Slide title: The problem

Key message (1 sentence): Defense magnet production loses a material fraction as machining swarf/sludge, and the resulting coolant-contaminated waste is expensive to dispose of and hard to recycle cleanly.

Supporting data points to include:

- “the new solid scrap and cutting sludge ‘swarf’… accounts for 20–30 % of the starting alloy.”

- “About 20–30 wt.% scrap is generated during the NdFeB magnets cutting and grinding processes.”

- “More than 70% of the waste generated in manufacturing consists of sludge, which is heavily oxidized and contaminated with carbon from lubrication.”

- Offsite hazardous waste disposal “typically $0.50–$3.00/gallon ($130–$800/m3).”

Visual suggestion: Photo/illustration of grinding sludge and a callout showing 20–30% loss and $130–$800/m³ disposal range.

### Slide 3

Slide title: The market

Key message (1 sentence): Rare earth magnets are required across major U.S. defense platforms at large scale, making magnet waste and provenance a strategic issue—not just an operational one.

Supporting data points to include:

- DoD statement that rare earth permanent magnets are essential across defense capabilities (e.g., F-35, submarines, UAVs).

- Platform REE requirements: F-35 “more than 900 pounds,” DDG-51 “5,200 pounds,” Virginia-class submarine “9,200 pounds.”

Visual suggestion: Three platform images (F-35, DDG-51, submarine) with large numeric REE labels.

### Slide 4

Slide title: The regulatory tailwind

Key message (1 sentence): DFARS expands to upstream “mined/refined/separated/melted/produced” restrictions on January 1, 2027, making traceable domestic recycling pathways a procurement lever for NdFeB in particular.

Supporting data points to include:

- DFARS final rule effective date anchor: “Effective May 30, 2024.”

- The 2027 prohibition: “Effective January 1, 2027, do not acquire any covered material mined, refined, separated, melted, or produced in any covered country…”

- Covered magnets include “(1) Samarium-cobalt magnets; (2) Neodymium-iron-boron magnets.”

- Recycled NdFeB exception: “manufactured from recycled material if the milling… and sintering… takes place in the United States.”

Visual suggestion: A timeline from 2024 → 2027 showing the “scope expansion cliff,” plus a branch highlighting “Recycled NdFeB (US milled + sintered) exception.”

### Slide 5

Slide title: The solution

Key message (1 sentence): Algamol’s modular on-site treatment concept pairs pre-treatment for oily machining coolants with an algae-based capture step positioned to support rare earth recovery from wastewater.

Supporting data points to include:

- Membrane/adsorption recovery from coolants typically needs pretreatment to remove oils/organics.

- Water-mix cutting fluids form oil-in-water emulsions (2–10% oil by volume).

- Algamol positioning: “supports the recovery of rare earth metals from wastewater.”

Visual suggestion: Process block diagram: (1) equalization + oil/solids removal → (2) algae-based capture → (3) polishing and concentrate/sludge handling.

### Slide 6

Slide title: How it works

Key message (1 sentence): A staged treatment train can manage oily metalworking fluids and selectively separate iron vs rare earths using known pH windows, while concentrating REE-bearing residues for downstream recovery.

Supporting data points to include:

- Operational selectivity: Fe can be precipitated first at ~pH 3–4, then REE precipitation occurs at higher pH to avoid co-precipitation.

- REE hydroxide precipitation range: commonly achieved by increasing pH to ~8.6 and typically occurs ~8.6–10.

- Membrane-based pre-concentration is promising, but oil/organic fouling complicates direct use and implies pretreatment requirements.

Visual suggestion: A “pH cascade” diagram with three bands: Fe removal (~3–4), biosorption window (mildly acidic), REE precipitation (~8.6–10).

### Slide 7

Slide title: Competitive advantage

Key message (1 sentence): Algamol is positioned as a “treatment + recovery” module that can complement standard precipitation and protect downstream membrane/recovery steps in oily coolant matrices where fouling is a known constraint.

Supporting data points to include:

- Chemical precipitation via pH adjustment is widely used for metal-ion removal.

- Membrane recovery is promising but fouling from oils/organics complicates direct membrane use; pretreatment is usually required.

- The algae/biomass pathway is still emerging for real wastes (risk and differentiation must be communicated honestly).

Visual suggestion: A 2×2 comparison matrix (CAPEX vs selectivity; or TRL vs waste compatibility) comparing precipitation, membranes, ion exchange (placeholder), and Algamol biosorption.

### Slide 8

Slide title: Unit economics

Key message (1 sentence): Even before recovered-metal upside, avoiding hazardous hauling at $130–$800/m³ can support rapid payback against a $110k–$310k modular system with ~$28k–$73k/year OPEX drivers.

Supporting data points to include:

- Disposal benchmark: $0.50–$3.00/gal ($130–$800/m³).

- CAPEX estimate: $110k–$310k.

- OPEX components: chemicals $5k–$15k/yr; membrane replacement $5k–$10k/yr; energy $3k–$8k/yr; labor $10k–$25k/yr; sludge disposal/processing $5k–$15k/yr.

Visual suggestion: Waterfall chart showing “baseline hauling spend → on-site treatment OPEX → net savings,” with CAPEX shown as an upfront bar.

### Slide 9

Slide title: SmCo opportunity

Key message (1 sentence): High-temperature defense magnets often use SmCo, which is preferred for 200–550°C applications, and its finishing waste is amplified by brittleness-driven loss and fracture risk.

Supporting data points to include:

- SmCo is described as preferred/ultimate for high-temperature applications “200–550°C.”

- Brittle magnet loss “up to 20–30% in volume.”

- Sm-Co machining failure rate “up to 20%.”

Visual suggestion: Side-by-side graphic: “SmCo at high temperature” (200–550°C) vs “brittleness waste,” with a callout to brittleness-driven loss range.

### Slide 10

Slide title: Beachhead customer

Key message (1 sentence): EEC in Landisville, PA is a defense-relevant SmCo producer with a documented in-house process chain (melting → ~3 µm powder → pressing → sinter/solution/aging → grinding/slicing/WEDM) and certifications aligned with defense programs.

Supporting data points to include:

- Location: “Electron Energy Corporation, 924 Links Avenue, Landisville, PA 17538.”

- Alloy inputs: purchases pure raw materials (Sm, Co, Fe, Cu, Zr) for in-house SmCo 1:5 and 2:17 alloys.

- Powder size: average particle size reduced to “≈ 3 microns.”

- Pressing: axial, transverse, or isostatic presses.

- Heat treatment: sinter, solution, and aging under vacuum or inert gas.

- Machining: grinding, slicing, wire EDM for brittle sintered magnets.

- Compliance: “ITAR COMPLIANT AND DDTC REGISTERED” and “certified to AS9100D and ISO 9001:2015.”

Visual suggestion: Map of Landisville/Lancaster County with an “EEC” pin, plus a vertical process-flow graphic ending in machining and shipping.

### Slide 11

Slide title: Go to market

Key message (1 sentence): A pilot-first approach is supported by wastewater best practices and biosorption literature calling for validation with actual industrial samples, with priority pilots aimed at defense-relevant magnet suppliers and machinists.

Supporting data points to include:

- Priority targets named in compiled inputs: “EEC/Permag (Landisville PA), Arnold (multiple sites), VAC (Sumter SC).”

- Pilot importance: “Field tests and sometimes pilot tests are also important items…”

- Scale-up requirement: lab experiments “need to be advanced to industrial levels and tested with actual wastewater samples from manufacturing sources.”

Visual suggestion: Three-phase roadmap: pilot at EEC → multi-site deployments → scale with DFARS timing.

### Slide 12

Slide title: Business model

Key message (1 sentence): Because hauling/disposal costs can dominate, a service or lease model tied to verified disposal reduction can be underwritten even while metals-recovery performance is validated on real waste matrices.

Supporting data points to include:

- Disposal cost anchor for savings-based contracting: $0.50–$3.00/gal ($130–$800/m³).

- Technology maturity caveat: research into algal sorbents for REE recovery from real wastes is “still in its infancy.”

- Pilot testing is a recognized pathway to derisk performance in industrial water treatment.

Visual suggestion: Pricing schematic with three options: (1) lease + monthly fee, (2) performance-based (share of hauling savings), (3) sale + support; include “pilot validation gate.”

### Slide 13

Slide title: Strategic value

Key message (1 sentence): Waste capture and recovery supports defense supply security goals, improves provenance readiness as disclosure requirements phase in, and can align with domestic “mine-to-magnet” investments underway since 2020.

Supporting data points to include:

- Supply-security rationale: “It’s important that all those critical nodes happen inside the U.S.”

- Provenance disclosure direction: DoD “will have to require that contractors disclose the source of certain rare earth permanent magnets…”

- DoD funding scale: “Since 2020, DOD has awarded approximately $439 million…” to reestablish domestic rare earth supply chains.

Visual suggestion: Four-pillar slide (Security, Compliance, Cost, Circularity) with each pillar anchored to one cited statement/data point.

### Slide 14

Slide title: Market sizing

Key message (1 sentence): Magnet swarf and sludge is already a sizable stream relative to global production, and machining scrap fractions in the 20–30% range imply meaningful capture volumes where high-value magnets are finished to tight tolerances.

Supporting data points to include:

- Swarf stream estimate: “The volume of the swarf recycling stream in 2020 is estimated as 9% of the NdFeB magnets produced globally.”

- Typical machining scrap: 20–30% of starting alloy as swarf in manufacturing and 20–30 wt% scrap during NdFeB cutting/grinding.

- Machining is called out as the step with the greatest material loss; machining recovery rate 60–90% depending on complexity (implying 10–40% loss).

Visual suggestion: TAM/SAM/SOM concentric circles with (a) global NdFeB production baseline, (b) swarf stream fraction (~9%), and (c) defense-relevant subset (placeholder).

### Slide 15

Slide title: Risks

Key message (1 sentence): The core technical risk is variability and contamination in real machining slurries and coolants, and the core commercialization risk is proving performance on real streams early enough to match DFARS timing pressure.

Supporting data points to include:

- Real waste streams complicate recovery due to cutting fluids/organics; pretreatment is often required.

- Membrane-based REE recovery from coolants is complicated by fouling from oils/organics and typically needs pretreatment.

- DFARS 2027 expansion date is a fixed milestone that drives procurement urgency.

Visual suggestion: Risk matrix with “technical” (matrix effects, fouling, pH control) and “schedule” (2027 deadline) axes, plus a mitigation column tied to pilot testing.

### Slide 16

Slide title: Team and ask

Key message (1 sentence): Raise $[X]M to run a pilot at a defense-relevant SmCo producer and mature the process to industrial validation using real wastewater samples, consistent with published scale-up needs for biosorption approaches.

Supporting data points to include:

- Validation requirement: lab-scale experiments must be tested with actual wastewater samples from manufacturing sources.

- Maturity caveat and why funding is needed: algal sorbents for real wastes are still “in its infancy.”

- Named pilot targets include EEC/Permag, Arnold, VAC Sumter (beachhead list).

Visual suggestion: “Use of funds” pie chart (pilot build, sampling/analytics, field ops, BD), plus a 6–12 month pilot timeline ending with a validated performance report.

### Appendix A1

Slide title: Detailed financial model

Key message (1 sentence): A transparent model should separate (1) disposal avoidance (bankable) from (2) metals recovery (upside), using cited disposal, CAPEX, and OPEX ranges as the base case.

Supporting data points to include:

- Disposal: $130–$800/m³ benchmark.

- CAPEX: $110k–$310k; OPEX line items as cited ranges.

- If needed, include third-party ROI benchmarks (e.g., ROIs as fast as 1½ years; typical 3–4 years).

Visual suggestion: Spreadsheet screenshot mockup with scenario toggles (flow rate, disposal cost, treatment OPEX) and outputs (payback, NPV).

### Appendix A2

Slide title: SmCo grade anchor

Key message (1 sentence): EEC’s 2:17-34 grade provides a defense-relevant numeric anchor for high-energy SmCo magnets and illustrates why finishing operations (and their waste streams) concentrate significant embedded value.

Supporting data points to include:

- EEC 2:17-34: (BH)max 34 MGOe; Br 11.9 kG; iHc min 18 kOe; max operating temp 300°C; “DFARS compliant.”

- Machining to final dimensions via grinding/slicing/WEDM for brittle sintered magnets (connects performance to finishing waste).

Visual suggestion: Property radar chart (BHmax/Br/Hc/iHc/Tmax) plus a process note “sintered → machined to print.”

### Appendix A3

Slide title: DFARS details

Key message (1 sentence): DFARS covers both SmCo and NdFeB magnets as “covered materials,” and the January 1, 2027 expansion reaches upstream mining/refining/separation, with a specific recycled-material exception only for NdFeB if U.S. milled and sintered.

Supporting data points to include:

- Covered material includes SmCo and NdFeB magnets.

- 2027 expansion language (“mined, refined, separated, melted, or produced”).

- NdFeB recycled exception language (U.S. milling + sintering).

- SmCo rule scope includes “including recycled material… through production of finished magnets” (useful to frame SmCo compliance complexity).

Visual suggestion: One-page “what changes in 2027” schematic with two branches (NdFeB with recycling exception vs SmCo without such an exception in the cited text).

### Appendix A4

Slide title: Treatment chemistry

Key message (1 sentence): The most defensible near-term architecture combines pretreatment for oily coolants with staged pH control for selective metals removal, consistent with documented precipitation and coolant-fouling constraints.

Supporting data points to include:

- Coolant emulsions: 2–10% oil-in-water (water-mix cutting fluids).

- Staged pH: Fe removal first (~3–4), then REE hydroxide precipitation at ~8.6–10.

- Membrane-based pre-concentration is promising but fouling from oils/organics complicates direct use; pretreatment is usually required.

Visual suggestion: A “treatment train” slide with unit operations and pH setpoints labeled, plus an “organics pretreatment” callout.

### Appendix A5

Slide title: Waste form reality

Key message (1 sentence): Magnet-manufacturing waste is frequently sludge-like and coolant-contaminated rather than clean scrap, which affects both downstream recovery and wastewater treatment design constraints.

Supporting data points to include:

- “More than 70% of the waste… consists of sludge” that is oxidized and lubrication-contaminated.

- Grinding slurry description: mixture of metallic swarf, grinding-media abrasion, and cooling agent, with remaining coolant varying from <1% to >20%.

Visual suggestion: Cross-section illustration of slurry (solids + abrasive + coolant) with the <1% to >20% residual coolant band highlighted.

### Appendix A6

Slide title: Pilot site interface

Key message (1 sentence): If the pilot discharges to a local POTW, the relevant interface is a pretreatment program with industrial user permits and local limits designed to prevent pollutant interference and biosolids contamination.

Supporting data points to include:

- LASA: “approved Pretreatment Program” and it “issues Industrial User Permits” for significant industrial users.

- Pretreatment goal: prevent pollutants that interfere with the POTW or contaminate biosolids.

Visual suggestion: A compliance flow diagram: sampling → pretreatment permit → local limits monitoring → discharge approval loop.

## Chapter 13 Arnold facility model

### Algamol Deployment Model Arnold Magnetic Technologies Multi Site Defense Magnet Operations

Arnold Magnetic Technologies is a multi-division, multi-site producer of permanent magnets and assemblies that explicitly targets aerospace and defense markets and emphasizes ITAR/DFARS compliance. Arnold’s footprint spans U.S. manufacturing (including Rochester, NY and Marengo, IL), European operations (including the UK and Switzerland), and Asia (including Shenzhen, China and a newer Thailand site), creating both scale advantages and compliance complexity for defense magnet supply chains.

### Company profile

Arnold is headquartered at 770 Linden Avenue, Rochester, New York 14625. Company materials describe Arnold as having an operating history of over ~130 years and long-standing supply of “defense sensitive materials” to U.S. and allied markets. Arnold states it is a subsidiary of Compass Diversified (NYSE: CODI), and Compass Diversified reports acquiring Arnold in March 2012 for $129 million. Separately, an Arnold UK “History” page states that “as of the latest updates” Arnold operates under the ownership of TMS International.

### Locations and capabilities

Arnold publicly lists a global manufacturing footprint that includes permanent magnet production and/or assemblies in Rochester, NY (U.S.), Sheffield (UK), Lupfig (Switzerland), Amata City (Thailand), and Shenzhen (China). Arnold also lists Woodstock, Illinois as a site for Alnico and Precision Thin Metals, and provides a Woodstock address at 1005 Courtaulds Dr, Woodstock, IL 60098.

Arnold has stated that its Rochester headquarters fabricates and tests SmCo and builds custom assemblies. Arnold also states that it manufactures RECOMA Samarium Cobalt magnets in Rochester, NY and Marengo, IL.

The table below consolidates the most directly evidenced site-level roles from the provided sources.

| Site | Evidence of role relevant to defense magnet operations | Notes for Algamol targeting |
| --- | --- | --- |
| Rochester, NY (770 Linden Ave) | Headquarters and permanent magnets; Rochester address listed. Fabrication/testing of SmCo and building custom assemblies stated. | Likely hub for RECOMA SmCo work and defense assembly work based on stated SmCo fabrication/testing and “custom assemblies.” |
| Marengo, IL (300 West Street) | Listed as a company location (Marengo, IL 60152). RECOMA SmCo manufacturing stated in Rochester and Marengo. | Candidate site for SmCo production steps and/or machining to print associated with RECOMA magnets. |
| Woodstock, IL (1005 Courtaulds Dr) | Woodstock, IL address listed; quality certifications page ties Woodstock to Alnico and PTM. A 130,000 sq ft Woodstock facility is described as opening in January 2025 for Alnico and Precision Thin Metals. | Near-term commissioning/relocation window can enable “design-in” of waste/coolant handling and metal recovery processes for magnet-adjacent operations. |
| Lupfig, Switzerland | Address listed (Hubelacherstrasse 15, CH-5242 Lupfig). Listed as “Permanent Magnets and Assemblies.” | EU/Swiss operations may be relevant for EU defense programs and non-U.S. supply chain work. |
| Sheffield, UK | Listed on the corporate site as “Permanent Magnets & Assemblies.” | UK assembly site can be a second-stage integrator where magnet finishing/assembly waste streams may occur. |
| Shenzhen, China | Shenzhen address listed (Guangming, Shenzhen). Quality certifications page lists “SmCo Group” and “Bonded Magnet Group” in Shenzhen. Shenzhen facility achieved AS9100D certification and is described as the sixth Arnold location to hold that certification. | China location creates DFARS/covered-country complexity for U.S. defense programs, even if it is AS9100D-certified. |
| Chonburi, Thailand | Arnold states it opened a 26,250 sq ft manufacturing site in 2024 producing magnet assemblies, rotors, and motors. | Newer site suggests greenfield opportunities for embedding water reuse and waste capture into assembly/rotor lines. |
| Ganzhou, China (JV mine) | Arnold materials list a “Joint Venture - Mine Ganzhou, CN.” | Upstream JV highlights exposure to China-based raw material nodes that may be incompatible with DFARS pathways for certain contracts. |

Arnold also reports maintaining “twelve manufacturing facilities, plus their joint venture,” and lists a broader set of division sites including Ogallala, NE; Madison, WI; Marietta, OH; Norfolk, NE; and Greenville, OH.

### Product portfolio

Arnold describes itself as a manufacturer of high-performance permanent magnets and explicitly lists RECOMA samarium cobalt, neodymium iron boron (NdFeB), Alnico, and precision magnetic assemblies among its offerings. Arnold markets RECOMA Samarium Cobalt in “standard grades 18–35E” with “custom grades for high temperature,” and it states that RECOMA 35E is “the most power dense samarium cobalt magnet on the market.” Arnold also states that it can adjust material composition “for performance at extreme temperatures up to 550°C.”

Arnold’s RECOMA documentation distinguishes two SmCo families, stating that Sm2Co17 magnets show the highest magnetic performance at elevated temperatures while SmCo5 offers easier magnetizing and strong corrosion resistance. For NdFeB, Arnold publishes a catalog of “Neodymium-Iron-Boron Magnet Grades” and notes these are also referred to as “Neo” or NdFeB magnets. Arnold’s catalog also lists “Grain Boundary Dysprosium Diffused Grades” and “Grain Boundary Diffused Grades,” signaling availability of diffusion-enhanced, high-coercivity NdFeB options relevant to high-temperature and/or demagnetization-resistance design constraints.

### Certifications and programs

Arnold states that all of its facilities are AS9100 certified. Arnold’s aerospace and defense market page lists ISO 9001:2015 and AS 9100 Rev D as certifications. Arnold’s quality certifications materials list NADCAP as a certification, with scope indicated for Precision Thin Metals.

For export-control and defense procurement positioning, Arnold states that “all locations” in its RECOMA supply chain are registered with ITAR and can adhere to DFARS clauses. Arnold further positions itself as “registered with ITAR” and able to adhere to DFARS clauses for defense work.

On the customer/program side, Arnold’s aerospace and defense materials state that its products are used in “commercial and defense aviation, satellites,” and list defense-relevant application areas including rotor assemblies, electric motors, auxiliary power units, flap actuators, magnetic bearings, electromagnetic brakes, waveguides for microwave transmission (TWT), EMI/RFI shielding, fuel systems, and “black chaff.” An Arnold acquisitions/mergers infographic also indicates key customers include defense primes such as Raytheon, Lockheed Martin, and Northrop Grumman.

### Machining and waste

Arnold presents itself as a supplier of “precision magnetic systems to print” and emphasizes vertically integrated processes and contract manufacturing capabilities that span magnets and assemblies. On its precision magnetic assemblies page, Arnold lists machining and finishing capabilities including EDM, ID slicing and grinding, in-house machining of hubs/rotors/cylinders/sleeves, magnetic and thermal stabilization, rotor dynamic balancing, serializing, and magnetic characterization.

Arnold also states it provides “advanced machining, grinding, and finishing capabilities” to achieve tight tolerances suitable for defense-grade applications. The provided sources do not quantify the fraction of magnet material lost as kerf, grinding sludge, or rejected parts during Arnold’s machining operations.

### Wastewater characterization

The provided sources establish that Arnold performs EDM and grinding/slicing operations and manufactures magnetic assemblies, which are the kinds of operations that commonly drive the “waste stream” interface for any on-site capture or treatment initiative. However, none of the provided Arnold sources describe coolant chemistries, wastewater volumes, metal concentrations, or sludge handling methods at any Arnold site.

As a result, an Arnold-specific wastewater characterization for Algamol must be treated as a data-collection task rather than a literature-derived specification, and it must be anchored to Arnold’s explicitly stated machining processes and sites.

### DFARS compliance

Arnold states it can supply NdFeB magnets “fully compliant” with DFARS 225.7018. Arnold also states that RECOMA supply chain locations are ITAR-registered and can adhere to DFARS clauses, and it positions RECOMA samarium cobalt as “DFARS compliant.”

Arnold’s communications also describe active DFARS preparation for 2027, stating it is preparing for “DFARS Compliance updates starting Jan 1st 2027” and plans to establish a fully compliant supply chain for NdFeB and SmCo magnets by mid-2026. In parallel, Arnold states it has entered into strategic agreements to secure samarium raw material supply outside China and to reduce dependency on Chinese sources amid export-control uncertainties.

### Algamol system design

Because Arnold’s evidenced manufacturing and assembly processes include EDM, slicing, and grinding, any Algamol deployment concept for Arnold should be designed around capture points associated with these operations and with assembly-related washing/cleaning where applicable. Arnold explicitly highlights magnet production/fabrication, magnetizing, stabilization/calibration, and testing as part of its permanent magnet capabilities, which can be used to bound where in the process route a waste/water interface is most likely to exist.

At a program level, Arnold’s multi-country footprint implies that any Algamol deployment strategy should be segmented into (a) U.S. defense-program-aligned sites and (b) non-U.S. sites that may serve different customer bases or export-control contexts, with separate data packages for ITAR/DFARS handling.

### Financial model

The sources provided for Arnold do not include site-level wastewater disposal costs, coolant spend, sludge disposal costs, or magnet machining scrap rates, so a bottom-up ROI cannot be computed from the evidence set alone. The evidence does support that Arnold is investing “over $50 million” in facility improvements, manufacturing technology upgrades, and capacity expansion, indicating ongoing capital programs that could potentially be leveraged to integrate waste capture or treatment systems as part of broader manufacturing upgrades. The evidence also supports that Arnold is expanding capacity via a new Thailand permanent magnets manufacturing facility and by adding/modernizing facilities such as Woodstock, which can change both production scale and waste/water load profiles over time.

### Strategic value and pilot proposal

Arnold’s stated position as a DFARS- and ITAR-capable supplier to aerospace and defense markets makes traceable, domestic-aligned handling of rare-earth-bearing process streams strategically relevant, especially as the company explicitly prepares for DFARS updates effective January 1, 2027. Arnold’s publicly listed application set in defense aviation, satellites, motors, and RF/microwave subsystems suggests that customer programs may demand both performance and supply-chain assurance, which can extend to how critical-material-bearing waste is handled inside controlled facilities.

A pragmatic pilot pathway, constrained to what the sources actually substantiate, is to start at a U.S. site with explicitly stated SmCo fabrication/testing and assembly work and documented machining capability (e.g., Rochester, NY and/or the Rochester-linked assemblies capability set), and build a measurement-first program focused on sampling and quantifying process-stream composition before selecting treatment architecture. A second pilot option is to engage during transition windows such as the January 2025 Woodstock facility opening (Alnico and Precision Thin Metals), where new processes and infrastructure could be instrumented from the outset even if the most REE-intensive streams remain concentrated in SmCo/NdFeB operations.

The largest gaps that must be closed to turn this facility model into an executable Algamol project are (1) which specific Arnold sites run the highest-volume rare-earth magnet machining steps (EDM/slicing/grinding) for SmCo and NdFeB, (2) what coolant and cleaning chemistries are used, and (3) what the current waste handling pathway is at each site. These gaps are not addressed in the provided sources and require direct site engagement and sampling under the relevant quality and export-control frameworks that Arnold states it can support.

## Chapter 14 VAC Sumter facility model

### Algamol Deployment Model Vacuumschmelze VAC Sumter SC New US Magnet Production Plant

### Summary

Vacuumschmelze (VAC Group) is a Germany-based rare-earth permanent magnet producer headquartered in Hanau (Grüner Weg 37, D-63450 Hanau) and described as a global producer of advanced magnetic solutions and rare-earth permanent magnets. The company reports significant global scale (nearly 4,000 employees) and multiple production facilities, while also highlighting vertical-integration capability “from base element to end product.”

VAC’s U.S. expansion is executed through eVAC Magnetics LLC, a VAC Group subsidiary that built a new rare-earth permanent magnet manufacturing facility in Sumter County, South Carolina (Pocotaligo Industrial Park). Public reporting places operations coming online in late autumn 2025 / end of 2025 and describes the facility as having begun shipments after opening in November 2025. Public sources also describe a production capacity target on the order of ~1,600–2,000 metric tons per year (with various documents citing “up to 1,600 MT” NdFeB magnet capacity and “just under 2,000 metric tons a year,” and local reporting stating 2,000 metric tons/year by spring 2026).

The Sumter facility is positioned as a dual-use asset supporting both commercial and military demand, with multiple sources explicitly linking the plant to magnets used in defense applications (including F-35 and UAVs) and to domestic supply chain resilience objectives. The plant’s business model and capital stack are strongly shaped by public incentives and national-security-linked funding, including a reported $94.1 million U.S. Department of Defense agreement for manufacturing equipment and production line engineering and a $111.9 million federal tax credit (Section 48c).

### Company

VAC Group is described as a leading global producer of advanced magnetic solutions, rare earth permanent magnets, and inductive components, with corporate operations based in Hanau, Germany. VAC reports a broad manufacturing footprint (facilities in Finland, Slovakia, China, and Malaysia) and maintains a model in which different plants are optimized for different product groups and scales (e.g., small-series precision work vs. flexible, partially automated production).

VAC’s permanent-magnet portfolio is branded around VACOMAX (SmCo) and VACODYM (NdFeB), and the company states it has produced VACOMAX SmCo magnets (powder-metallurgical) since 1973 and VACODYM NdFeB magnets at industrial scale since 1986. VAC also links its aerospace/defense market access to quality management standards, including AS9100/EN 9100 (with EN 9100 referenced as a certification basis for permanent magnets).

Ownership has shifted recently, with multiple sources stating VAC was acquired by Ara Partners in October 2023.

### Facility

The eVAC Magnetics Sumter facility is located in the Pocotaligo Industrial Park in Sumter County, South Carolina, and multiple announcements describe the project as a “more than half a billion-dollar investment,” with reported capital investment of $506 million and planned creation of about 300 jobs. One local report describes the plant footprint as 410,000 square feet on 83 acres, while other public announcements describe the project site as 85 acres.

Public timelines consistently place the facility’s operational start in late 2025 (e.g., “online in late autumn 2025,” “complete in late fall 2025,” and “come online in the second half of 2025”). A later local report states the facility opened in November 2025 and shipped its first NdFeB permanent magnets to customers.

Capacity is reported in several ways. Local reporting states the facility will reach a 2,000-metric-ton annual production capacity by spring 2026 and may scale output substantially thereafter. A separate policy-focused document states the magnet-making facility will be able to produce up to 1,600 metric tons of NdFeB permanent magnets, and notes a potential expansion to 4,000 metric tons within 12–18 months after start-up (conditional on agreements and lead times).

### Products

Public reporting and VAC communications identify the Sumter facility as producing high-performance rare-earth permanent magnets, including NdFeB and (in some reporting) SmCo types, with end-markets spanning EV traction motors and defense applications. Multiple sources also describe the facility as producing NdFeB magnets specifically and link those magnets to automotive and defense use cases.

At the corporate product-family level, VACODYM magnets are NdFeB-based (neodymium-iron-boron alloy basis) and are described as “highest energy density” grades, while VACOMAX magnets are rare-earth–cobalt (SmCo-class) and described as high-coercivity, temperature-stable magnets with corrosion stability benefits.

### Grades

VAC’s NdFeB portfolio includes VACODYM 902 TP, which VAC describes as heavy-rare-earth-free and manufactured entirely in the Western Hemisphere. A VAC press release on the VACODYM 902 TP family reports room-temperature remanence of at least 1.40 T and coercivity of at least 1190 kA/m for VACODYM 902 TP.

VAC’s VACODYM product family page reports (across grades) Br minimum ranges of 1.03–1.42 T and typical (BH)max of 225–415 kJ/m³, and notes that an established grain boundary diffusion process can increase coercivity up to a maximum of 600 kA/m (for applicable magnets).

For SmCo, a trade source describing VAC’s permanent magnet capabilities notes that Vacomax sintered SmCo alloys are used in high-performance rotors and cites Vacomax 262 HRP as enabling application temperatures up to 250 °C in a rotor system context.

The table below summarizes publicly stated grade-level performance anchors that are relevant for high-reliability motor, actuator, and defense/aerospace applications.

| Product family | Magnet type | Example grade | Public performance anchors | Notes for defense procurement relevance |
| --- | --- | --- | --- | --- |
| VACODYM | NdFeB | VACODYM 902 TP | Br ≥ 1.40 T; Hc ≥ 1190 kA/m (reported for 902 TP) | Marketed as heavy-rare-earth-free and made in the Western Hemisphere |
| VACODYM | NdFeB | Family ranges | Br min 1.03–1.42 T; (BH)max typ 225–415 kJ/m³; grain boundary diffusion can raise coercivity up to 600 kA/m | Process options cited that are commonly associated with high-coercivity/high-temp NdFeB needs |
| VACOMAX / Vacomax | SmCo | Vacomax 262 HRP | Application temperatures up to 250 °C in cited rotor context | SmCo is frequently positioned for high-temperature, high-reliability duty cycles in aerospace/defense motors |

### Manufacturing

VAC’s NdFeB manufacturing is described (at the product-family level) as powder-metallurgical and sintering-based, with production routes that depend on form factors and tolerances and can involve larger isostatically pressed blocks or die-pressed parts. The same source describes die-press alignment as using strong magnetic fields oriented parallel (axial) or perpendicular (transverse) to the pressing direction depending on geometry and grade family (AP vs TP).

At the enterprise level, VAC describes itself as vertically integrated, with statements that it can “design and manufacture from base element to end product” and that it is “extensively vertically integrated.” A technical brochure further states VAC can perform processing steps in-house, ranging from vacuum melting of alloys through coating and magnetization, assembly, and final inspection (with those statements made specifically about VAC’s established European operations).

For Sumter specifically, one policy-focused description frames the facility as “onshoring the NdFeB permanent magnet supply chain” and states the magnet plant will produce up to 1,600 MT NdFeB magnets and is supported by DoD Title III Defense Production Act authorities. The same source states that a next-phase, co-located metal- and alloy-making facility is planned to produce rare earth metals and strip cast alloys, with capacity up to 2,000 MT of magnet alloy annually starting in early 2027 (sized to support the 1,600 MT magnet plant).

### Compliance

The Sumter facility is repeatedly positioned as a contribution to a resilient, U.S.-based rare-earth permanent magnet supply chain and a step toward reducing dependence on China for rare-earth magnet production. A policy-focused document states that eVAC’s vertically integrated campus would “eliminate reliance on foreign sources — particularly China” by processing non-Chinese rare earth oxides into finished magnets and explicitly links this to “compliance with … DFARS 225.7018,” with a 2027 deadline for excluding adversarial sourcing for NdFeB-containing defense components.

In parallel, DFARS rules (as published on acquisition.gov) state that effective January 1, 2027 contractors shall not deliver “covered material” mined/refined/separated/melted/produced in “any covered country,” and also include an exception for NdFeB magnets manufactured from recycled material if milling and sintering occur in the United States. DFARS also specifies that for samarium-cobalt magnets, the restriction includes the entire supply chain from mining (including recycled material) through production of finished magnets, except as provided by additional exceptions.

VAC also publicly positions itself as compliant with U.S. and EU regulatory expectations, including DFARS and ITAR (as a general compliance statement) and lists quality certifications such as DIN EN ISO 9001:2015 and EN 9100:2018 in corporate materials.

### Waste

Public documentation indicates the Sumter facility is expected to generate magnet production by-products, explicitly described as “swarf,” and VAC has contracted for recycling of these by-products. A Cyclic Materials announcement states that VAC and Cyclic signed a 10-year exclusive agreement to recycle 100% of the magnet production by-products (swarf) produced at the Sumter facility, which was described as scheduled to begin operations at the end of 2025. The same announcement states VAC has an option to reintegrate recycled mixed rare earth oxides (rMREOs) from Cyclic into its own production processes, connecting manufacturing waste streams to a circular feedstock pathway.

Separate reporting similarly notes a long-duration recycling agreement to process byproducts from the Sumter facility, reinforcing that by-product handling is part of the operating model rather than an afterthought. In addition, local reporting indicates the plant includes quality control areas and automated/robotic inspection operations, consistent with a facility designed for tightly controlled product outputs where scrap and rework streams can be material (even though public sources do not quantify scrap rates).

At the corporate capability level (not necessarily Sumter-specific), VAC describes advanced machining for magnetic materials—including wire EDM and grinding—to meet stringent specifications, and also references EDM-based production approaches in its manufacturing capability descriptions.

### Algamol fit

The public record demonstrates that (1) Sumter is intended to be a high-volume rare-earth magnet manufacturing site, (2) its by-products (swarf) are explicitly anticipated, and (3) there is already a contractual pathway to collect and recycle those by-products in North America. These three points define the minimum integration boundary conditions for any onsite water, coolant, or effluent treatment system intended to capture value from magnet-making waste streams at Sumter.

State and local incentives explicitly included “water and wastewater improvements” as eligible project-support uses, indicating that water and wastewater infrastructure is a recognized part of the site build-out and may be adaptable for future add-on treatment or capture systems (without implying any particular technology choice).

### Financial model

A facility-specific financial model for Sumter must start from the disclosed project economics and public co-investments. Multiple sources describe the project as a more-than-half-billion-dollar investment, with one local report stating $506 million in capital investment for the site. Public incentives and federal support include a $111.9 million Section 48c Qualifying Advanced Energy Project Tax Credit, and a DoD agreement reported as $94.1 million to acquire/install manufacturing equipment, operationalize technical infrastructure, and engineer production lines.

A Reuters-syndicated business report states VAC received about $200 million in U.S. government funding and tax credits to build a $500 million plant and states the Sumter plant’s capacity is just under 2,000 metric tons per year, with up to 90% of output going to General Motors and the remainder supplying the U.S. Department of Defense.

The table below consolidates publicly stated facility-scale financial anchors and support mechanisms.

| Category | Publicly stated figure | Notes |
| --- | --- | --- |
| Capital investment | $506M | Local reporting of total capital investment for Sumter |
| Federal tax credit | $111.9M (Section 48c) | Described as a Qualifying Advanced Energy Project Tax Credit supporting construction |
| DoD agreement support | $94.1M | Described as funding for equipment, infrastructure, and engineering production lines |
| State/local infrastructure support | $13M Rural Infrastructure Fund grant; $2M LocateSC grant | Explicitly includes “water and wastewater improvements” among eligible uses |
| Capacity anchor | Up to 1,600 MT/year NdFeB magnets | Policy-focused description of magnet-plant capacity |
| Capacity anchor | Just under 2,000 metric tons/year | Reuters-syndicated estimate of plant capacity |

### Strategic value

The Sumter facility is consistently framed as strategically significant for the United States because it increases domestic rare-earth permanent magnet production and reduces reliance on foreign (particularly Chinese) supply chains, a theme repeated in local reporting and federal policy discussions. VAC and partners also explicitly link U.S.-produced rare-earth magnets to defense platforms and systems, including the F-35 and UAVs, and DoD communications list a wider set of magnet-dependent platforms including submarines, missiles, radar systems, UAVs, and precision-guided munitions.

From a defense procurement standpoint, the facility’s strategic value is also tied to the January 1, 2027 DFARS sourcing restrictions and the need for non-covered-country production pathways for both NdFeB and SmCo magnets in defense end-items.

### Integration

Sumter’s key integration premise is that waste streams are expected and contractually managed, with a declared intention to recycle 100% of magnet-production by-products (swarf) through a long-term agreement, and an option to reintegrate recycled mixed rare earth oxides into VAC’s own production processes. Any onsite capture or treatment system (including a wastewater-oriented system) would therefore need to map cleanly onto the existing swarf collection and recycling logistics, rather than creating a parallel, conflicting waste disposition pathway.

To translate this into an execution plan without over-claiming beyond public sources, the immediate next step is a due-diligence package of questions that align with the published facility design and compliance goals:

- What are the physical forms of “swarf” generated at the plant (grinding sludge, EDM fines, saw kerf, coating overspray), and what proportion is currently routed to the contracted recycler under the “100%” agreement language?

- Which stages of the NdFeB supply chain are executed on the Sumter campus at start-up vs. after the planned early-2027 alloy/metal facility comes online (rare-earth metals and strip-cast alloy production), and how does that change the waste stream composition and security requirements?

- What quality system and audit artifacts are required for Sumter’s defense-market deliveries under AS9100/EN9100-aligned expectations, and how will waste handling be documented to preserve traceability and compliance narratives?

These questions are directly motivated by the public claims of a China-independent supply chain, DFARS-linked compliance positioning, and contractual commitment to recycle manufacturing by-products, which collectively define the operational constraints for any onsite waste or wastewater intervention at the facility.